CHAPTER 1

LISTING 1.1 Example of Entity Architecture

entity Half_adder is

port(a: in bit; b : in bit; S : out bit;

C: out bit);

end half_adder;

architecture dtfl_half of Half_adder is

begin

S <= a xor b; -- statement 1

C <= a and b; -- statement 2

--Blank lines are allowed

end dtfl_half;

Listing 1.2 Example of a Verilog Module

module Half_adder(a,b,S,C);

input a,b;

output S, C;

// Blank lines are allowed

assign S = a ^ b; // statement 1

assign C= a & b; // statement 2

endmodule

Listing 1.3 Example of Behavioral Description

VHDL1B Description

entity Half_adder is

port(a: in bit; b : in bit; S : out bit;

C: out bit);

end half_adder;

architecture beh_half of Half_adder is

begin

process (a, b)

begin

if (a /= b) then

S <= ‘1’;

else

S <= ‘0’;

--Blank lines are allowed

end if;

end process;

end beh_half;

Verilog Description

module Half_adder(a,b,S,C);

input a,b;

output S, C;

reg S,C;

// Blank lines are allowed

always @ (a,b)

begin

if (a != b)

S = 1’b1;

else

S = 1’b0;

end

endmodule

LISTING 1.4. Example of Structural Description

VHDL Description

entity Half_adder is

port(a: in bit; b : in bit; S : out bit;

C: out bit);

end half_adder;

architecture struct_exple of Half_adder is

--ADDITIONAL BINDING IS NEEDED TO RUN THIS PROGRAM;

--SEE CHAPTER 4

component xor2

--The above statement is a component statement

port(I1, I2 : in std_logic;

O1 : out std_logic);

end component;

component and2

port(I1, I2 : in std_logic;

O1 : out std_logic);

end component;

begin

X1: xor2 port map (a,b, S);

A1: and2 port map (a,b, C);

end struct_exple;

Verilog Description

module Half_adder1(a,b,S,C);

input a, b;

output S,C;

and a1(C,a,b);

//The above statement is AND gate

xor x1(S,a,b);

//The above statement is EXCLUSIVE-OR gate

endmodule

LISTING 1.5 An Example of A Switch-Level Description

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity Inverter is

Port (y : out std_logic; a: in std_logic );

end Inverter;

architecture Invert_switch of Inverter is

--additional binding is needed to run this program;

--see chapter 5

component nmos

--nmos is one of the key words for switch-level.

port (O1: out std_logic; I1, I2 : in std_logic);

end component;

component pmos

--pmos is one of the key words for switch-level.

port (O1: out std_logic ;I1, I2 : in std_logic);

end component;

for all: pmos use entity work. mos (pmos_behavioral);

for all: nmos use entity work. mos (nmos_behavioral);

--The above two statements are referring to a package mos

--See details in Chapter 5

constant vdd: std_logic := ‘1’;

constant gnd : std_logic:= ‘0’;

begin

p1 : pmos port map (y, vdd, a);

n1: nmos port map (y, gnd, a);

end Invert_switch;

Verilog Description

module invert(y,a);

input a;

output y;

supply1 vdd;

supply0 gnd;

pmos p1(y, vdd, a);

nmos n1(y, gnd, a);

/*The above two statement are using the two primitives

pmos and nmos*/

endmodule

LISTING 1.6 Example of Mixed-Language Description

module Full_Adder1 ( x,y, cin, sum, carry);

input x,y,cin;

output sum, carry;

wire c0, c1, s0;

HA H1 (y, cin, s0,c0);

// Description of HA is written in VHDL in the

// entity HA

..................

endmodule

library IEEE;

use ieee.std_logic_1164.all;

entity HA is

--For correct binding between this VHDL code and the above

--Verilog code, the entity has to be named here as HA

port (a, b : in std_logic; s, c: out std_logic);

end HA;

architecture HA_Dtflw of HA is

begin

s <= a xor b;

c <= a and b;

end HA_Dtflw;

CHAPTER 2

Listing 2.1 HDL code of Figure 2.1

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity andor is

port (a,b,c,d: in std_logic; y : out std_logic);

end andor;

architecture andor_dtfl of andor is

signal s1,s2 : std_logic;

begin

s1 <= a and b; --statement 1.

s2 <= c and d; --statement 2.

y <= s1 or s2; --statement 3.

end andor_dtfl;

Verilog description

module andor (a,b,c,d, y );

input a,b,c,d;

output y;

wire s1, s2; /* wire statement here is not necessarily

needed since s1 and s2 are single bit*/

assign s1 = a & b; //statement 1.

assign s2 = c & d; //statement 2.

assign y = s1 | s2; //statement 3.

endmodule

Listing 2.2 HDL code of Figure 2.1 with 10 ns delay

VHDL description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity andor_dly is

port (a,b,c,d: in std_logic; y : out std_logic);

end andor_dly;

architecture andor_dtfl of andor_dly is

constant dly : time := 10 ns;

signal s1,s2 : std_logic;

begin

s1 <= a and b after dly; --statement 1.

s2 <= c and d after dly; --statement 2.

y <= s1 or s2 after dly; --statement 3.

end andor_dtfl;

Verilog description

module and_orDlyVr( a,b,c,d, y );

input a,b,c,d;

output y;

time dly = 10;

wire s1, s2;

/* wire above is not necessarily needed

since s1 and s2 are single bit*/

assign # dly s1 = a & b; //statement 1.

assign # dly s2 = c & d; //statement 2.

assign # dly y = s1 | s2; //statement 3.

Endmodule

Listing 2.3 HDL Code of the Full Adder From Figure 2.5

VHDL description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity fulladder is

Port ( a,b,c : in std_logic;

sum, Carryout : out std_logic);

end fulladder;

architecture flad_dtfl of fulladder is

begin

Sum <= (not a and not b and c) or

(not a and b and not c) or

(a and not b and not c) or

(a and b and c);

Carryout <= (a and b) or (a and c) or (b and c);

end flad_dtfl;

Verilog description

module fulladder(a, b, c);

output Sum, Carryout;

input a, b, c;

assign Sum = (~ a & ~ b & c)|( ~ a & b & ~c)|

( a & ~b & ~c)|( a & b & c) ;

assign Carryout = (a & b) | (a & c) | (b & c);

endmodule

Listing 2.4a HDL Code of a 2x1 Multiplexer: VHDL and Verilog

VHDL description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity mux2x1 is

port (A, B, SEL, Gbar : in std_logic;

Y : out std_logic);

end mux2x1;

architecture MUX_DF of mux2x1 is

signal S1, S2, S3, S4, S5 : std_logic;

constant dly : time := 7ns;

Begin

-- Assume 7 nanoseconds propagation delay

-- for all and, or, and not operation.

st1: Y <= S4 or S5 after dly;

st2: S4 <= A and S2 and S1 after dly;

st3: S5 <= B and S3 and S1 after dly;

st4: S2 <= not SEL after dly;

st5: S3 <= not S2 after dly;

st6: S1 <= not Gbar after dly;

end MUX_DF;

Verilog Description

module mux2x1 (A, B, SEL, Gbar, Y);

input A, B, SEL, Gbar;

output Y;

wire S1, S2, S3, S4, S5;

time dly = 7;

/* Assume 7 time units delay for all and, or, not operations. The

delay here is expressedin simulation screen units. */

assign # dly Y = S4 | S5; //st1

assign #dly S4 = A & S2 & S1; //st2

assign #dly S5 = B & S3 & S1; //st3

assign #dly S2 = ~ SEL; //st4

assign #dly S3 = ~ S2; //st5

assign #dly S1 = ~ Gbar; //st6

endmodule

Listing 2.4 HDL Code of a 2x4 Decoder Without Time Delay

VHDL description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity decoder2x4 is

port ( a, b : in std_logic;

D : out std_logic_vector (3 downto 0));

end decoder2x4;

architecture decder_dtfl of decoder2x4 is

begin

D(0) <= not a and not b;

D(1) <= a and not b;

D(2) <= not a and b;

D(3) <= a and b;

end decder_dtfl;

Verilog description

module decoder2x4( a, b, D);

input a,b;

output [3:0]D;

assign D[0] = ~a & ~ b;

assign D[1] = a & ~ b;

assign D[2] = ~a & b;

assign D[3] = a & b;

endmodule

Listing 2.5 HDL Code for a 2x2 Unsigned Combinational Array Multiplier:

VHDL and Verilog

VHDL Description

--For simplicity, propagation delay times are not

-- considered here.

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity mult_comb is

port ( a,b: in std_logic_vector (1 downto 0);

P : out std_logic_vector ( 3 downto 0));

end mult_comb;

architecture mult_dtfl of mult_comb is

begin

P(0) <= a(0) and b(0);

P(1) <= (not a(1) and a(0) and b(1)) or

(a(0) and not b(0) and b(1)) or

(a(1) and b(0) and not b(1))or

(not a(0) and a(1) and b(0));

P(2) <= (not a(0) and a(1) and b(1)) or

( a(1) and not b(0) and b(1));

P(3) <= a(0) and a(1) and b(0) and b(1);

end mult_dtfl;

Verilog Description

module mult_arry (a, b, P);

input [1:0] a, b;

output [3:0] P;

/*For simplicity, propagation delay times are not

considered in this example.*/

assign P[0] = a[0] & b[0];

assign P[1] = (~a[1] & a[0]& b [1]) |

(a[0] & ~b[0]& b [1])|

(a[1] & b[0]& ~b [1]) |

(~a[0] & a[1]& b [0]);

assign P[2] = (~a[0] & a[1]& b [1]) |

(a[1] & ~b[0]& b [1]);

assign P[3] = (a[0] & a[1]& b[0] & b [1]); endmodule

Listing 2.6 HDL Code for a D-Latch: VHDL and Verilog

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity D_Latch is

port (D, E : in std_logic;

Q, Qbar : buffer std_logic);

--Q and Qbar are declared as buffer because they act

--as both input and output, they appear on the

--right and left hand side of a signal assignment.

--inout or linkage could have been used instead of buffer.

end D_Latch;

architecture DL_DtFl of D_Latch is

constant Delay_EorD : Time := 9 ns;

constant Delay_inv : Time := 1 ns;

begin

--Assume 9-ns propagation delay time between

--E or D and Qbar; and 1 ns between Qbar and Q.

Qbar <= (D and E) nor (not E and Q) after Delay_EorD;

Q <= not Qbar after Delay_inv;

end DL_DtFl;

Verilog Description

module D_latch (D, E, Q, Qbar);

input D, E;

output Q, Qbar;

/* Verilog treats the ports as internal ports,

so Q and Qbar are not considered here as

both input and output. If the port is

connected externally as bidirectional,

then it should be declared as inout. */

time Delay_EorD = 9;

time Delay_inv = 1;

assign #Delay_EorD Qbar = ~((E & D) | (~E & Q));

assign #Delay_inv Q = ~ Qbar;

endmodule

Listing 2.7 HDL Code of a 2x2 Magnitude Comparator

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity COMPR_2 is

port (x, y : in std_logic_vector(1 downto 0);

xgty, xlty : buffer std_logic;

xeqy : out std_logic);

end COMPR_2;

architecture COMPR_DFL of COMPR_2 is

begin

xgty <= (x(1) and not y(1)) or (x(0) and not y(1)

and not y(0)) or

x(0) and x(1) and not y(0));

xlty <= (y(1) and not x(1)) or ( not x(0) and y(0)

and y(1)) or (not x(0) and not x(1) and y(0));

xeqy <= xgty nor xlty;

end COMPR_DFL;

Verilog Description

module compr_2 (x, y, xgty, xlty, xeqy);

input [1:0] x, y;

output xgty, xlty, xeqy;

assign xgty = (x[1] & ~ y[1]) | (x[0] & ~ y[1]

& ~ y[0]) | (x[0] &

x[1] & ~ y[0]);

assign xlty = (y[1] & ~ x[1] ) | (~ x[0] &

y[0] & y[1]) |(~ x[0] &

~ x[1] & y[0]);

assign xeqy = ~ (xgty | xlty);

endmodule

Listing 2.8 Three-Bit Ripple-Carry Adder Case Study

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity adders_RL is

port (x, y : in std_logic_vector (2 downto 0);

cin : in std_logic;

sum : out std_logic_vector (2 downto 0);

cout : out std_logic);

end adders_RL;

--I. RIPPLE-CARRY ADDER

architecture RCarry_DtFl of adders_RL is

--Assume 4.0-ns propagation delay for all gates.

signal c0, c1 : std_logic;

constant delay_gt : time := 4 ns;

begin

sum(0) <= (x(0) xor y(0)) xor cin after 2*delay_gt;

--Treat the above statement as two 2-input XOR.

sum(1) <= (x(1) xor y(1)) xor c0 after 2*delay_gt;

--Treat the above statement as two 2-input XOR.

sum(2) <= (x(2) xor y(2)) xor c1 after 2*delay_gt;

--Treat the above statement as two 2-input XOR.

c0 <= (x(0) and y(0)) or (x(0) and cin) or

(y(0) and cin) after 2*delay_gt;

c1 <= (x(1) and y(1)) or (x(1) and c0) or

(y(1) and c0) after 2*delay_gt;

cout <= (x(2) and y(2)) or (x(2) and c1) or

(y(2) and c1)after 2*delay_gt;

end RCarry_DtFl;

Verilog Description

module adr_rcla (x, y, cin, sum, cout);

input [2:0] x, y;

input cin;

output [2:0] sum;

output cout;

// I. RIPPLE CARRY ADDER

wire c0, c1;

time delay_gt = 4;

//Assume 4.0-ns propagation delay for all gates.

assign #(2*delay_gt) sum[0] = (x[0] ^ y[0]) ^ cin;

//Treat the above statement as two 2-input XOR.

assign #(2*delay_gt) sum[1] = (x[1] ^ y[1]) ^ c0;

//Treat the above statement as two 2-input XOR.

assign #(2*delay_gt) sum[2] = (x[2] ^ y[2]) ^ c1;

//Treat the above statement as two 2-input XOR.

assign #(2*delay_gt) c0 = (x[0] & y[0]) |

(x[0] & cin) | (y[0] & cin);

assign #(2*delay_gt) c1 = (x[1] & y[1]) |

(x[1] & c0) | (y[1] & c0);

assign #(2*delay_gt) cout = (x[2] & y[2]) |

(x[2] & c1) | (y[2] & c1);

endmodule

Listing 2.9 Three-Bit Carry-Lookahead Adder Case Study

VHDL Description

--II. CARRY-LOOKAHEAD ADDER

architecture lkh_DtFl of adders_RL is

--Assume 4.0-ns propagation delay for all gates

--including a 3-input xor.

signal c0, c1 : std_logic;

signal p, g : std_logic_vector (2 downto 0);

constant delay_gt : time := 4 ns;

begin

g(0) <= x(0) and y(0) after delay_gt;

g(1) <= x(1) and y(1) after delay_gt;

g(2) <= x(2) and y(2) after delay_gt;

p(0) <= x(0) or y(0) after delay_gt;

p(1) <= x(1) or y(1) after delay_gt;

p(2) <= x(2) or y(2) after delay_gt;

c0 <= g(0) or (p(0) and cin) after 2*delay_gt;

c1 <= g(1) or (p(1) and g(0)) or (p(1) and p(0)

and cin) after 2*delay_gt;

cout <= g(2) or (p(2) and g(1)) or (p(2) and p(1)

and g(0)) or(p(2) and p(1) and

p(0) and cin) after 2*delay_gt;

sum(0) <= (p(0) xor g(0)) xor cin after delay_gt;

sum(1) <= (p(1) xor g(1)) xor c0 after delay_gt;

sum(2) <= (p(2) xor g(2)) xor c1 after delay_gt;

end lkh_DtFl;

Verilog Description

// II. CARRY-LOOKAHEAD ADDER

module lkahd_adder (x, y, cin, sum, cout);

input [2:0] x, y;

input cin;

output [2:0] sum;

output cout;

/*Assume 4.0-ns propagation delay for all gates

including a 3-input xor.*/

wire c0, c1;

wire [2:0] p, g;

time delay_gt = 4;

assign #delay_gt g[0] = x[0] & y[0];

assign #delay_gt g[1] = x[1] & y[1];

assign #delay_gt g[2] = x[2] & y[2];

assign #delay_gt p[0] = x[0] | y[0];

assign #delay_gt p[1] = x[1] | y[1];

assign #delay_gt p[2] = x[2] | y[2];

assign #(2*delay_gt) c0 = g[0] | (p[0] & cin);

assign #(2*delay_gt) c1 = g[1] | (p[1] & g[0]) |

(p[1] & p[0] & cin);

assign #(2*delay_gt) cout = g[2] | (p[2] & g[1]) |

(p[2] & p[1] & g[0]) | (p[2] & p[1] &

p[0] & cin);

assign #delay_gt sum[0] = (p[0] ^ g[0]) ^ cin;

assign #delay_gt sum[1] = (p[1] ^ g[1]) ^ c0;

assign #delay_gt sum[2] = (p[2] ^ g[2]) ^ c1;

endmodule

Chapter 3

LISTING 3.1 Example of an HDL Behavioral Description

VHDL Description

entity half_add is

port (I1, I2 : in bit; O1, O2 : out bit);

-- Since we are using type bit, no need for attaching a

-- Library.

-- If we use std_logic, we should attach the IEEE

-- Library.

end half_add;

architecture behave_ex of half_add is

begin

process (I1, I2)

begin

O1 <= I1 xor I2 after 10 ns; -- statement 1

O2 <= I1 and I2 after 10 ns; -- statement 2

-- The above two statements are signal-assignment

-- statements with 10 nanoseconds delays.

--

--Other behavioral (sequential) statements can be added

-- here

end process;

end behave_ex;

Verilog Description

module half_add (I1, I2, O1, O2);

input I1, I2;

output O1, O2;

reg O1, O2;

/* Since O1 and O2 are outputs and they are

written inside “always,” they should be

declared as reg */

always @(I1, I2)

begin

#10 O1 = I1 ^ I2; // statement 1.

#10 O2 = I1 & I2; // statement 2.

/*The above two statements are

signal-assignment statements with 10 simulation screen units

delay*/

/*Other behavioral (sequential) statements can be added here*/

end

endmodule

LISTING 3.2 VHDL Code for Behavioral Description of D-Latch Using VariableAssignment

Statements

entity DLTCH_var is

port (d, E : in bit; Q, Qb : out bit);

-- Since we are using type bit, no need for attaching a

-- Library. If std_logic is used, IEEE library should be

--attached

end DLTCH_var;

architecture DLCH_VAR of DLTCH_var is

begin

VAR : process (d, E)

variable temp1, temp2 : bit;

begin

if E = ‘1’ then

temp1 := d; -- This is a variable assignment statement.

temp2 := not temp1; -- This is a variable assignment

-- statement.

end if;

Qb <= temp2; -- Value of temp2 is passed to Qb

Q <= temp1; -- Value of temp1 is passed to Q

end process VAR;

end DLCH_VAR;

LISTING 3.3 VHDL Code for Behavioral Description of a D-Latch Using Signal-

Assignment Statements

entity Dltch_sig is

port (d, E : in bit; Q : buffer bit; Qb : out bit);

--Q is declared as a buffer because it is an

--input/output signal; it appears on both the left

-- and right hand sides of assignment statements.

end Dltch_sig;

architecture DL_sig of Dltch_sig is

begin

process (d, E)

begin

if E = ‘1’ then

Q <= d; -- signal assignment

Qb <= not Q; -- signal assignment

end if;

end process;

end DL_sig;

LISTING 3.4 Verilog Code for Behavioral Description of a D-Latch

module D_latch (d, E, Q, Qb);

input d, E;

output Q, Qb;

reg Q, Qb;

always @ (d, E)

begin

if (E == 1)

begin

Q = d;

Qb = ~ Q;

end

end

endmodule

LISTING 3.5 HDL Description of a 2x1 Multiplexer Using IF-Else

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity MUX_if is

port (A, B, SEL, Gbar : in std_logic;

Y : out std_logic);

end MUX_if;

architecture MUX_bh of MUX_if is

begin

process (A, B, SEL, Gbar)

-- A, B, SEL, and Gbar are the sensitivity list of the process.

variable temp : std_logic;

-- Above statement is declaring temp as a variable; it

-- will be calculated as if it is the output of the

-- multiplexer.

begin

if Gbar = ‘0’ then

if SEL = ‘1’ then

temp := B;

else

temp := A;

end if;

--Now assign the variable temp to the output

Y <= temp;

else

Y <= ‘Z’;

end if;

end process;

end MUX_bh;

Verilog Description

module mux2x1 (A, B, SEL, Gbar, Y);

input A, B, SEL, Gbar;

output Y;

reg Y;

always @ (SEL, A, B, Gbar)

begin

if (Gbar == 1)

Y = 1’bz;

else

begin

if (SEL)

Y = B;

else

Y = A;

end

end

endmodule

LISTING 3.6 HDL Description of a 2x1 Multiplexer Using Else-IF

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity MUXBH is

port (A, B, SEL, Gbar : in std_logic;

Y : out std_logic);

end MUXBH;

architecture MUX_bh of MUXBH is

begin

process (SEL, A, B, Gbar)

variable temp : std_logic;

begin

if (Gbar = ‘0’) and (SEL = ‘1’) then

temp := B;

elsif (Gbar = ‘0’) and (SEL = ‘0’)then

temp := A;

else

temp := ‘Z’; -- Z is high impedance.

end if;

Y <= temp;

end process;

end MUX_bh;

Verilog Description

module MUXBH (A, B, SEL, Gbar, Y);

input A, B, SEL, Gbar;

output Y;

reg Y; /* since Y is an output and appears inside

always, Y has to be declared as reg( register) */

always @ (SEL, A, B, Gbar)

begin

if (Gbar == 0 & SEL == 1)

begin

Y = B;

end

else if (Gbar == 0 & SEL == 0)

Y = A;

else

Y = 1’bz; //Y is assigned to high impedance

end

endmodule

LISTING 3.7 HDL Code for a Positive Edge-Triggered JK Flip-Flop Using the

case Statement

VHDL Description

library ieee;

use ieee.std_logic_1164.all;

entity JK_FF is

port(JK : in bit_vector (1 downto 0);

clk : in std_logic; q, qb : out bit);

end JK_FF;

architecture JK_BEH of JK_FF is

begin

P1 : process (clk)

variable temp1, temp2 : bit;

begin

if rising_edge (clk) then

case JK is

when “01” => temp1 := ‘0’;

when “10” => temp1 := ‘1’;

when “00” => temp1 := temp1;

when “11” => temp1 := not temp1;

end case;

q <= temp1;

temp2 := not temp1;

qb <= temp2;

end if;

end process P1;

end JK_BEH;

Verilog Description

module JK_FF (JK, clk, q, qb);

input [1:0] JK;

input clk;

output q, qb;

reg q, qb;

always @ (posedge clk)

begin

case (JK)

2’d0 : q = q;

2’d1 : q = 0;

2’d2 : q = 1;

2’d3 : q =~ q;

endcase

qb =~ q;

end

endmodule

LISTING 3.8 HDL Code for a Three-Bit Binary Counter Using the case

Statement

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity CT_CASE is

port (clk, clr : in std_logic;

q : buffer std_logic_vector (2 downto 0));

end CT_CASE;

architecture ctr_case of CT_CASE is

begin

ctr : process(clk)

variable temp : std_logic_vector (2 downto 0) := “101”;

--101 is the initial value, so the counter starts from

-- 110

begin

if rising_edge (clk) then

if clr = ‘0’ then

case temp is

when “000” => temp := “001”;

when “001” => temp := “010”;

when “010” => temp := “011”;

when “011” => temp := “100”;

when “100” => temp := “101”;

when “101” => temp := “110”;

when “110” => temp := “111”;

when “111” => temp := “000”;

when others => temp := “000”;

end case;

else

temp := “000”;

end if;

end if;

q <= temp;

end process ctr;

end ctr_case;

Verilog Description

module CT_CASE (clk, clr, q);

input clk, clr;

output [2:0] q;

reg [2:0] q;

initial

/* The above initial statement is to force

the counter to start from initial count q=110 */

q = 3’b101;

always @ (posedge clk)

begin

if (clr == 0)

begin

case (q)

3’d0 : q = 3’d1;

3’d1 : q = 3’d2;

3’d2 : q = 3’d3;

3’d3 : q = 3’d4;

3’d4 : q = 3’d5;

3’d5 : q = 3’d6;

3’d6 : q = 3’d7;

3’d7 : q = 3’d0;

endcase

end

else

q = 3’b000;

end

endmodule

LISTING 3.9 HDL Code for Genotypes and Phenotypes Using the case Statement:

VHDL and Verilog

This program takes the blood genotypes (alleles) of a male and a female

and generates the possible blood phenotypes of their offspring. The statement

report (VHDL) or display (Verilog) is used to print the phenotype

on the screen of the simulator. The male allele is allelm, and allelf is the

female allele. Both allelm and allelf are decoded as 00 for genotype A,

01 for B, or 10 for O. Phenotype A is decoded as 000, B as 001, AB as 010,

O as 011, and an illegal allele entry as 111. Figure 3.12 shows the simulation

waveform for genotypes and phenotypes of human blood.

VHDL Description

library ieee;

use ieee.std_logic_1164.all;

entity Bld_type is

port (allelm, allelf : in bit_vector (1 downto 0);

pheno : out bit_vector (2 downto 0));

end Bld_type;

architecture GEN_BLOOD of Bld_type is

begin

Bld : process (allelm, allelf)

variable geno : bit_vector(3 downto 0);

begin

geno := allelm & allelf;

-- The operator (&) concatenates the two 2-bit vectors

-- allelf and allelm into one 4-bit vector geno.

case geno is

when “0000” => pheno <= “000”;

report “phenotype is A “;

--report statement is close to printf in C language.

--The statement here prints on the screen whatever

--written between the quotations.

when “0001” => pheno <= “010”;

report “phenotype is AB “;

when “0010” => pheno <= “000”;

report “phenotype is A “;

when “0100” => pheno <= “010”;

report “phenotype is AB “;

when (“0101”) => pheno <= “001”;

report “phenotype is B “;

when (“0110”) => pheno <= “001”;

report “phenotype is B “;

when “1000” => pheno <= “000”;

report “phenotype is A “;

when (“1001”) => pheno <= “001”;

report “phenotype is B “;

when “1010” => pheno <= “011”;

report “phenotype is O “;

when others =>pheno <= “111”;

report “illegal allele entry “;

end case;

end process;

end GEN_BLOOD;

Verilog Description

module bld_type (allelm, allelf, pheno);

input [1:0] allelm, allelf;

output [2:0] pheno;

reg [2:0] pheno;

reg [3:0] geno;

always @ (allelm, allelf)

begin

geno = {allelm , allelf};

/* { , } concatenates the two 2-bit vectors

allelm and allelf into one 4-bit vector geno */

case (geno)

4’d0 : begin pheno = 3’d0;

$display (“phenotype is A “); end

4’d1 : begin pheno = 3’d2;

$display (“phenotype is AB “); end

/* $display statement is close to printf in C language.

The statement here prints on the screen whatever

written between the quotations.*/

4’d2 : begin pheno = 3’d0;

$display (“phenotype is A “); end

4’d4 : begin pheno = 3’d2;

$display (“phenotype is AB “); end

4’d5 : begin pheno = 3’d1;

$display (“phenotype is B “); end

4’d6 : begin pheno = 3’d10;

$display (“phenotype is B “); end

4’d8 : begin pheno = 3’d0;

$display (“phenotype is A “); end

4’d9 : begin pheno = 3’d1;

$display (“phenotype is B “); end

4’d10 : begin pheno = 3’d3;

$display (“phenotype is O “); end

default: begin pheno = 3’d7;

$display (“illegal allele entry “); end

endcase

end

endmodule

LISTING 3.10 HDL Code for Genotypes and Phenotypes Using the case

Statement and Character Type

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity bld_charctr is

port ( allelem, allelef : in string(1 to 1) ;

pheno : out string (1 to 2));

end bld_charctr;

architecture Bld_beh of bld_charctr is

begin

process (allelem, allelef)

variable geno: string (1 to 2);

begin

geno := (allelem & allelef);

case (geno ) is

when “AA” => pheno <= “A “;

when “AB” => pheno <= “AB”;

when “AO” => pheno <= “A “;

when “BA” => pheno <= “AB”;

when “BB” => pheno <= “B “;

when “BO” => pheno <= “B “;

when “OA” => pheno <= “A “;

when “OB” => pheno <= “B “;

when “OO” => pheno <= “O “;

when others => pheno <= “??”;

end case;

end process;

end Bld_beh;

Verilog Description

module Bld_typeCharctr(allelm, allelf, pheno);

input [8:1] allelm, allelf;

output [2*8:1] pheno;

reg [2*8:1] pheno; /*Since phenol is two characters;

two ASCII characters are allocated to it.*/

reg [2*8:1] geno;

always @ (allelm, allelf)

begin

geno = {allelm , allelf};

case (geno)

“AA”: pheno = “A “;

“AB”: pheno = “AB”;

“AO”: pheno = “A “;

“BB”: pheno = “B “;

“BA”: pheno = “AB”;

“BO”: pheno = “B “;

“OA”: pheno = “A “;

“OB”: pheno = “B “;

“OO”: pheno = “O “;

default : pheno = “??”; //?? means invalid entry

endcase

end

endmodule

LISTING 3.11 Verilog Description for a Four-Bit Priority Encoder Using

casex

module Encoder_4 (Int_req, Rout_addrs);

input [3:0] Int_req;

output [3:0] Rout_addrs;

reg [3:0] Rout_addrs;

always @ (Int_req)

begin

casex (Int_req)

4’bxxx1 : Rout_addrs=4’d1;

4’bxx10 : Rout_addrs=4’d2;

4’bx100 : Rout_addrs=4’d4;

4’b1000 : Rout_addrs= 4’d8;

default : Rout_addrs=4’d0;

endcase

end

endmodule

LISTING 3.12 Implementation of the wait-for Statement to Generate

Clocks

VHDL

Library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity waittestVHDL is

port ( a,b,c : out std_logic);

end waittestVHDL;

architecture Behavioral of waittestVHDL is

begin

p1 :process

variable a1: std_logic := ‘0’;

begin

a <= a1;

wait for 10 ns;

a1 := not a1;

end process;

p2 :process

variable b1: std_logic := ‘0’;

begin

b <= b1;

wait for 20 ns;

b1 := not b1;

end process;

p3 :process

variable c1: std_logic := ‘0’;

begin

c <= c1;

wait for 40 ns;

c1 := not c1;

end process;

END;

Verilog

module waitstatement(a,b,c);

output a,b,c;

reg a,b,c;

initial

begin

// Initialize Inputs

a = 0;

b = 0;

c = 0;

end

always

begin

#10 ;

a = ~ a;

end

always

begin

#20 ;

b = ~ b;

end

always

begin

#40 ;

c = ~ c;

end

endmodule

LISTING 3.13 HDL Code for a Four-Bit Counter With Synchronous Clear:

VHDL and Verilog

VHDL Description

library ieee;

use ieee.std_logic_1164.all;

entity CNTR_LOP is

port (clk, clr : in std_logic; q :

buffer std_logic_vector (3 downto 0));

end CNTR_LOP;

architecture CTR_LOP of CNTR_LOP is

begin

ct : process(clk)

variable temp :

std_logic_vector (3 downto 0) := “0000”;

variable result : integer := 0;

begin

if rising_edge (clk) then

if (clr = ‘0’) then

result := 0;

-- change binary to integer

lop1 : for i in 0 to 3 loop

if temp(i) = ‘1’ then

result := result + 2**i;

end if;

end loop;

-- increment result to describe a counter

result := result + 1;

-- change integer to binary

for j in 0 to 3 loop

if (result MOD 2 = 1) then

temp (j) := ‘1’;

else temp (j) := ‘0’;

end if;

-- integer division by 2

result := result/2;

end loop;

else temp := “0000”;

end if;

q <= temp;

end if;

end process ct;

end CTR_LOP;

Verilog Description

module CNTR_LOP (clk, clr, q);

input clk, clr;

output [3:0] q;

reg [3:0] q;

integer i, j, result;

initial

begin

q = 4’b0000; //initialize the count to 0

end

always @ (posedge clk)

begin

if (clr == 0)

begin

result = 0;

//change binary to integer

for (i = 0; i < 4; i = i + 1)

begin

if (q[i] == 1)

result = result + 2**i;

end

result = result + 1;

for (j = 0; j < 4; j = j + 1)

begin

if (result %2 == 1)

q[j] = 1;

else

q[j] = 0;

result = result/2;

end

end

else q = 4’b0000;

end

endmodule

LISTING 3.14 HDL Code for a Four-Bit Counter with Synchronous Hold:

VHDL and Verilog

VHDL Description

library ieee;

use ieee.std_logic_1164.all;

entity CNTR_Hold is

port (clk, hold : in std_logic;

q : buffer std_logic_vector (3 downto 0));

end CNTR_Hold;

architecture CNTR_Hld of CNTR_Hold is

begin

ct : process (clk)

variable temp : std_logic_vector

(3 downto 0) := “0000”;

-- temp is initialized to 0 so count starts at 0

variable result : integer := 0;

begin

if rising_edge (clk) then

result := 0;

-- change binary to integer

lop1 : for i in 0 to 3 loop

if temp(i) = ‘1’ then

result := result + 2**i;

end if;

end loop;

-- increment result to describe a counter

result := result + 1;

-- change integer to binary

lop2 : for i in 0 to 3 loop

-- exit the loop if hold = 1

exit when hold = ‘1’;

-- “when” is a predefined word

if (result MOD 2 = 1) then

temp (i) := ‘1’;

else

temp (i) := ‘0’;

end if;

--Successive division by 2

result := result/2;

end loop;

q <= temp;

end if;

end process ct;

end CNTR_Hld;

Verilog 4-Bit Counter with Synchronous Hold Description

module CT_HOLD (clk, hold, q);

input clk, hold;

output [3:0] q;

reg [3:0] q;

integer i, result;

initial

begin

q = 4’b0000; //initialize the count to 0

end

always @ (posedge clk)

begin

result = 0;

//change binary to integer

for (i = 0; i <= 3; i = i + 1)

begin

if (q[i] == 1)

result = result + 2**i;

end

result = result + 1;

for (i = 0; i <= 3; i = i + 1)

begin

if (hold == 1)

i = 4; //4 is out of range, exit.

else

begin

if (result %2 == 1)

q[i] = 1;

else

q[i] = 0;

result = result/2;

end

end

end

endmodule

LISTING 3.15 HDL Code for Logical Shifting of a Register Using the

Loop Statement

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity shift_register is

port(start : in std_logic; shft: in std_logic;

N: in natural;

q : out std_logic_vector(7 downto 0))

end shift_register;

--N is number of shifts selected by the user

architecture shift_righLift of shift_register is

begin

st: process (start)

variable vq : std_logic_vector (7 downto 0)

:= “11001110”;

--initial values for the vector is selected to be

-- 1100110

begin

if (start =’1’) then

lop2: for j in 1 to N loop

lop1: for i in 0 to 6 loop

if shft =’0’ then

--shft = 0 is logical right shift; =1 logical left

-- shift

vq(i) := vq(i+1);

vq(7) :=’0’;

else

vq(7-i ) := vq(6-i);

vq(0) := ‘0’;

end if;

end loop lop1;

end loop lop2;

end if;

q <= vq;

end process st;

end shift_righLift;

Verilog Description

module shft_regVerilog(start,shft, N,q);

input start,shft;

input [7:1] N;

//N is number of requested shifts

output [7:0]q;

reg [7:0]q;

integer i,j;

initial

q = 8’b01100110;

/*initial values for the vector is selected to be

1100110 */

always @ (posedge start)

begin

lop2: for (j= 1; j <= N; j = j +1)

begin

lop1: for (i= 0; i <= 6; i = i +1)

begin

if (shft == 1’b0 )

/*shft = 0 is logical right shift; =1 logical left

Shift */

begin

$display (“ shft = %d”, shft);/*This is a system task

to display The value of shift on the console’s

screen of the simulator*/

$display (“ i= %d”, i);

$display (“q[i] = %b”, q[i]);

$display (“q[i+1] = %b”, q[i+1]);

q[i] = q[i+1];

q[7] =1’b0; $display (“ q = %b”, q);end

else

begin q[7-i] = q[6-i];

q[0] = 1’b0; end

$display (“ shft = %d”, shft);

end

end

end

endmodule

LISTING 3.16 HDL Code for Calculating the Factorial of Positive Integers:

VHDL and Verilog

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

--The above library statements can be omitted;

--however no error if it is not omitted.

--The basic VHDL has type “natural.”

entity factr is

port(N : in natural; z : out natural);

end factr;

architecture factorl of factr is

begin

process (N)

variable y, i : natural;

begin

y := 1;

i := 0;

while (i < N) loop

i := i + 1;

y := y * i;

end loop;

z <= y;

end process;

end factorl;

Verilog Description

module factr (N, z);

input [5:0] N;

output [15:0] z;

reg [15:0] z;

/* Since z is an output, and it will appear inside

“always,” then Z has to be declared “reg” */

integer i;

always @ (N)

begin

z = 16’d1;

i = 0;

while (i < N)

begin

i = i + 1;

z = i * z;

end

end

endmodule

LISTING 3.17 4x4-Bit Booth Algorithm: VHDL and Verilog

VHDL Description

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

entity booth is

port (X, Y : in signed (3 downto 0);

Z : buffer signed (7 downto 0));

end booth;

architecture booth_4 of booth is

begin

process (X, Y)

variable temp : signed (1 downto 0);

variable sum : signed (7 downto 0);

variable E1 : unsigned (0 downto 0);

variable Y1 : signed (3 downto 0);

begin

sum := “00000000”; E1 := “0”;

for i in 0 to 3 loop

temp := X(i) & E1(0);

Y1 := - Y;

case temp is

when “10” => sum (7 downto 4) :=

sum (7 downto 4) + Y1;

when “01” => sum (7 downto 4) :=

sum (7 downto 4) + Y;

when others => null;

end case;

sum := sum srl 1; --This is a logical

--shift of one position to the right

sum (7) := sum(6);

--The above two statements perform arithmetic

--shift where the sign of the

--number is preserved after the shift.

E1(0) := x(i);

end loop;

if (y = “1000”) then

--If Y = 1000; then according to our code,

--Y1 = 1000 (-8 not 8 because Y1 is 4 bits only).

--The statement sum = -sum adjusts the answer.

sum := - sum;

end if;

z <= sum;

end process;

end booth_4;

Verilog Description

module booth (X, Y, Z);

input signed [3:0] X, Y;

output signed [7:0] Z;

reg signed [7:0] Z;

reg [1:0] temp;

integer i;

reg E1;

reg [3:0] Y1;

always @ (X, Y)

begin

Z = 8’d0;

E1 = 1’d0;

for (i = 0; i < 4; i = i + 1)

begin

temp = {X[i], E1};

//The above statement is catenation

Y1 = - Y;

//Y1 is the 2’ complement of Y

case (temp)

2’d2 : Z [7 : 4] = Z [7 : 4] + Y1;

2’d1 : Z [7 : 4] = Z [7 : 4] + Y;

default : begin end

endcase

Z = Z >>> 1;

/*The above statement is arithmetic shift of one position to

the right*/

E1 = X[i];

end

if (Y == 4’d8)

/*If Y = 1000; then according to our code,

Y1 = 1000 (-8 not 8, because Y1 is 4 bits only).

The statement sum = - sum adjusts the answer.*/

begin

Z = - Z;

end

end

endmodule

LISTING 3.18 Antidiuretic Hormone Mechanism: VHDL and Verilog

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity ADH_BEH is

port (clk : in std_logic; BP : in natural;

ADH : out natural);

-- Assume BP takes only positive integer values

end;

architecture ADH_XT of ADH_BEH is

begin

ADHP : process (clk)

variable resADH : natural := 0;

begin

if (clk = ‘1’) then

if Bp <= 20 then resADH := 100;

elsif Bp > 45 then resADH := 0;

else

resADH := Bp * (-4) + 180;

end if;

end if;

ADH <= resADH;

end process ADHP;

end ADH_XT;

Verilog Description

module ADH_BEH (clk, BP, ADH);

input clk;

input [8:0] BP;

// Assume BP takes only positive integer values

output [8:0] ADH;

reg [8:0] ADH;

always @ (clk)

begin

if (clk == 1)

begin

if (BP <= 20) ADH = 100;

else if (BP > 45.0) ADH = 0;

else

ADH = BP * (-4) + 180.0;

end

end

endmodule

Chapter 4

LISTING 4.1 HDL Structural Description

VHDL Description

--This code is not complete; binding statements should

--be aaded to recognize components

-- xor2 and and2 as 2-input

-- xor and and gate respectively.

--

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity system is

port (a, b : in std_logic;

sum, cout : out std_logic);

end system;

architecture struct_exple of system is

--start declaring all different types of components

component xor2

port (I1, I2 : in std_logic;

O1 : out std_logic);

end component;

component and2

port (I1, I2 : in std_logic;

O1 : out std_logic);

end component;

begin

--Start of instantiation statements

X1 : xor2 port map (a, b, sum);

A1 : and2 port map (a, b, cout);

end struct_exple;

Verilog Description

module system (a, b, sum, cout);

input a, b;

output sum, cout;

xor X1 (sum, a, b);

/* X1 is an optional identifier; it can be omitted.*/

and a1 (cout, a, b);

/* a1 is optional identifier; it can be omitted.*/

endmodule

LISTING 4.2 HDL Code of Half Adder: VHDL and Verilog

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity xor2 is

port(I1, I2 : in std_logic; O1 : out std_logic);

end xor2;

architecture Xor2_0 of xor2 is

begin

O1 <= I1 xor I2;

end Xor2_0;

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity and2 is

port (I1, I2 : in std_logic; O1 : out std_logic);

end and2;

architecture and2_0 of and2 is

begin

O1 <= I1 and I2;

end and2_0;

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity half_add is

port (a, b : in std_logic; S, C : out std_logic);

end half_add;

architecture HA_str of half_add is

component xor2

port (I1, I2 : in std_logic; O1 : out std_logic);

end component;

component and2

port (I1, I2 : in std_logic; O1 : out std_logic);

end component;

begin

X1 : xor2 port map (a, b, S);

A1 : and2 port map (a, b, C);

end HA_str;

Verilog Description

Module system (a, b, sum, cout);

input a, b;

output sum, cout;

xor X1 (sum, a, b);

/* X1 is an optional identifier; it can be omitted.*/

and a1 (cout, a, b);

/* a1 is optional identifier; it can be omitted.*/

Endmodule

LISTING 4.3 Binding Between Entity and Architecture in VHDL

entity one is

port (I1, I2 : in std_logic; O1 : out std_logic);

end one;

architecture A of one is

signal s : std_logic;

..........

end A;

architecture B of one is

signal x : std_logic;

.......

end B;

LISTING 4.4 Binding Between Entity and Component in VHDL

entity orgate is

port (I1, I2 : in std_logic; O1 : out std_logic);

end orgate;

architecture Or_dataflow of orgate is

begin

O1 <= I1 or I2;

end Or_dataflow;

entity system is

port (x, y, z : in std_logic;

out r : std_logic_vector (3 downto 0);

end system;

architecture system_str of system is

component orgate

port (I1, I2 : in std_logic; O1 : std_logic);

end component;

begin

orgate port map (x, y, r(0));

.......

end system_str;

LISTING 4.5 Binding Between Library and Module in VHDL

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity system is

port (I1, I2 : in std_logic;

O1 : out std_logic_vector (3 downto 0));

end system;

architecture lib_bound of system is

signal s : std_logic;

.............

end lib_bound;

LISTING 4.6 Binding Between a Library and Component in VHDL

--First, write the code that will be bound to another

-- module

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity bind2 is

port (I1, I2 : in std_logic; O1 : out std_logic);

end bind2;

architecture xor2_0 of bind2 is

begin

O1 <= I1 xor I2;

end xor2_0;

architecture and2_0 of bind2 is

begin

O1 <= I1 and I2;

end and2_0;

architecture and2_4 of bind2 is

begin

O1 <= I1 and I2 after 4 ns;

end and2_4;

-- in a known location. Now, open another module

--

--where the above information is to be used.

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity half_add is

port (a, b : in std_logic; S, C : out std_logic);

end half_add;

architecture HA_str of half_add is

component xor2

port (I1, I2 : in std_logic; O1 : out std_logic);

end component;

component and2

port (I1, I2 : in std_logic; O1 : out std_logic);

end component;

for all : xor2 use entity work.bind2 (xor2_0);

for all : and2 use entity work.bind2 (and2_4);

begin

X1 : xor2 port map (a, b, S);

A1 : and2 port map (a, b, C);

end HA_str;

LISTING 4.7 Binding Between Two Modules in Verilog

module one (O1, O2, a, b);

input [1:0] a;

input [1:0] b;

output [1:0] O1, O2;

two M0 (O1[0], O2[0], a[0], b[0]);

two M1 (O1[1], O2[1], a[1], b[1]);

endmodule

module two (s1, s2, a1, b1);

input a1;

input b1;

output s1, s2;

xor (s1, a1, b1);

and (s2, a1, b1);

endmodule

LISTING 4.8 VHDL Code for Several Gates

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity bind1 is

port (I1 : in std_logic; O1 : out std_logic);

end bind1;

architecture inv_0 of bind1 is

begin

O1 <= not I1; --This is an inverter with zero delay

end inv_0;

architecture inv_7 of bind1 is

begin

O1 <= not I1 after 7 ns; --This is an inverter with a

-- 7-ns delay

end inv_7;

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity bind2 is

port (I1, I2 : in std_logic; O1 : out std_logic);

end bind2;

architecture xor2_0 of bind2 is

begin

O1 <= I1 xor I2; --This is exclusive-or with zero

-- delay.

end xor2_0;

architecture and2_0 of bind2 is

begin

O1 <= I1 and I2; --This is a two input and gate with

-- zero delay.

end and2_0;

architecture and2_7 of bind2 is

begin

O1 <= I1 and I2 after 7 ns; -- This is a two input and

-- gate with 7-ns delay.

end and2_7;

architecture or2_0 of bind2 is

begin

O1 <= I1 or I2; -- This is a two input or gate with

-- zero delay.

end or2_0;

architecture or2_7 of bind2 is

begin

O1 <= I1 or I2 after 7 ns; -- This is a two input or

-- gate with 7-ns delay.

end or2_7;

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity bind3 is

port (I1, I2, I3 : in std_logic; O1 : out std_logic);

end bind3;

architecture and3_0 of bind3 is

begin

O1 <= I1 and I2 and I3; -- This is a three input and

-- gate with zero delay.

end and3_0;

architecture and3_7 of bind3 is

begin

O1 <= I1 and I2 and I3 after 7 ns; --This is a three

-- input and gate with 7-ns delay.

--

end and3_7;

architecture or3_0 of bind3 is

begin

O1 <= I1 or I2 or I3; --This is a three input OR gate

--with zero delay.

end or3_0;

architecture or3_7 of bind3 is

begin

O1 <= I1 or I2 or I3 after 7 ns; --This is a three

--input or gate with 7-ns delay.

end or3_7;

LISTING 4.9 HDL Description of a 2x1 Multiplexer with Active Low Enable:

VHDL and Verilog

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity mux2x1 is

port (A, B, SEL, Gbar : in std_logic;

Y : out std_logic);

end mux2x1;

architecture mux_str of mux2x1 is

--Start components Declaration

component and3

port (I1, I2, I3 : in std_logic; O1 : out std_logic);

end component;

--Only different types of components need be declared.

--Since the multiplexer has two identical AND gates,

--only one is declared.

component or2

port (I1, I2 : in std_logic; O1 : out std_logic);

end component;

component Inv

port (I1 : in std_logic; O1 : out std_logic);

end component;

signal S1, S2, S3, S4, S5 : std_logic;

for all : and3 use entity work.bind3 (and3_7);

for all : Inv use entity work.bind1 (inv_7);

for Or1 : or2 use entity work.bind2 (or2_7);

begin

--Start instantiation

A1 : and3 port map (A,S2, S1, S4);

A2 : and3 port map (B,S3, S1, S5);

IV1 : Inv port map (SEL, S2);

IV2 : Inv port map (Gbar, S1);

IV3 : Inv port map (S2, S3);

or1 : or2 port map (S4, S5, Y);

end mux_str;

Verilog Description

module mux2x1 (A, B, SEL, Gbar, Y);

input A, B, SEL, Gbar;

output Y;

and #7 (S4, A, S2, S1);

or #7 (Y, S4, S5);

and #7 (S5, B, S3, S1);

not #7 (S2, SEL);

not #7 (S3, S2);

not #7 (S1, Gbar);

endmodule

LISTING 4.10 VHDL Behavioral Description of a Tri-State Buffer

entity bind2 is

port (I1, I2 : in std_logic; O1 : out std_logic);

end bind2;

...........

--Add the following architecture to

--the entity bind2 of Listing 4.8

architecture bufif1 of bind2 is

begin

buf : process (I1, I2)

variable tem : std_logic;

begin

if (I2 =’1’) then

tem := I1;

else

tem := ‘Z’;

end if;

O1 <= tem;

end process buf;

end bufif1;

LISTING 4.11 HDL Description of a 2x4 Decoder with Tri-State Output

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity decoder2x4 is

port (I : in std_logic_vector(1 downto 0);

Enable : in std_logic;

D : out std_logic_vector (3 downto 0));

end decoder2x4;

architecture decoder of decoder2x4 is

component bufif1

port (I1, I2 : in std_logic; O1 : out std_logic);

end component;

component inv

port (I1 : in std_logic; O1 : out std_logic);

end component;

component and2

port (I1, I2 : in std_logic; O1 : out std_logic);

end component;

for all : bufif1 use entity work.bind2 (bufif1);

for all : inv use entity work.bind1 (inv_0);

for all : and2 use entity work.bind2 (and2_0);

signal s0, s1, s2, s3 : std_logic;

signal Ibar : std_logic_vector (1 downto 0);

-- The above signals have to be declared before they

-- can be used

begin

B0 : bufif1 port map (s0, Enable, D(0));

B1 : bufif1 port map (s1, Enable, D(1));

B2 : bufif1 port map (s2, Enable, D(2));

B3 : bufif1 port map (s3, Enable, D(3));

iv0 : inv port map (I(0), Ibar(0));

iv1 : inv port map (I(1), Ibar(1));

a0 : and2 port map (Ibar(0), Ibar(1), s0);

a1 : and2 port map (I(0), Ibar(1), s1);

a2 : and2 port map (Ibar(0), I(1), s2);

a3 : and2 port map (I(0), I(1), s3);

end decoder;

Verilog Description

module decoder2x4 (I, Enable, D);

input [1:0] I;

input Enable;

output [3:0] D;

wire [1:0] Ibar;

bufif1 (D[0], s0, Enable);

bufif1 (D[1], s1, Enable);

bufif1 (D[2], s2, Enable);

bufif1 (D[3], s3, Enable);

not (Ibar[0], I[0]);

not (Ibar[1], I[1]);

and (s0, Ibar[0], Ibar[1]);

and (s1, I[0], Ibar[1]);

and (s2, Ibar[0], I[1]);

and (s3, I[0], I[1]);

endmodule

LISTING 4.12 VHDL Description

--This code is to be appended to Listing 4.8

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity bind22 is

Port (I1, I2 : in std_logic;

O1, O2 : out std_logic);

end bind22;

architecture HA of bind22 is

component xor2

port (I1, I2 : in std_logic; O1 : out std_logic);

end component;

component and2

port (I1, I2 : in std_logic; O1 : out std_logic);

end component;

for A1 : and2 use entity work.bind2 (and2_0);

for X1 : xor2 use entity work.bind2 (xor2_0);

begin

X1 : xor2 port map (I1, I2, O1);

A1 : and2 port map (I1, I2, O2);

end HA;

LISTING 4.13 HDL Description of a Full Adder (Figures 4.6a and 4.6b)

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity FULL_ADDER is

Port (x, y, cin : in std_logic;

sum, carry : out std_logic);

end FULL_ADDER;

architecture full_add of FULL_ADDER is

component HA

Port (I1, I2 : in std_logic; O1, O2 : out std_logic);

end component;

component or2

Port (I1, I2 : in std_logic; O1 : out std_logic);

end component;

for all : HA use entity work.bind22 (HA);

for all : or2 use entity work.bind2 (or2_0);

signal s0, c0, c1 : std_logic;

begin

HA1 : HA port map (y, cin, s0, c0);

HA2 : HA port map (x, s0, sum, c1);

r1 : or2 port map (c0, c1, carry);

end full_add;

Verilog Description

module FULL_ADDER (x, y, cin, sum, carry);

input x, y, cin;

output sum, carry;

HA H1 (y, cin, s0, c0);

HA H2 (x, s0, sum, c1);

//The above two statements bind module HA

//to the present module FULL_ADDER

or (carry, c0, c1);

endmodule

module HA (a, b, s, c);

input a, b;

output s, c;

xor (s, a, b);

and (c, a, b);

endmodule

LISTING 4.14 HDL Description of a Three-Bit Ripple-Carry Adder: VHDL

and Verilog

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity three_bit_adder is

port(x, y : in std_logic_vector (2 downto 0);

cin : in std_logic;

sum : out std_logic_vector (2 downto 0);

cout : out std_logic);

end three_bit_adder;

architecture three_bitadd of three_bit_adder is

component FULL_ADDER

port (I1, I2, I3 : in std_logic;

O1, O2 : out std_logic);

end component;

for all : FULL_ADDER

use entity work.bind32 (full_add);

signal carry : std_logic_vector (1 downto 0);

begin

M0 : FULL_ADDER port map (x(0), y(0), cin, sum(0), carry(0));

M1 : FULL_ADDER port map (x(1), y(1), carry(0), sum(1),

carry(1));

M2 : FULL_ADDER port map (x(2), y(2), carry(1), sum(2), cout);

end three_bitadd;

Verilog Description

module three_bit_adder (x, y, cin, sum, cout);

input [2:0] x, y;

input cin;

output [2:0] sum;

output cout;

wire [1:0] carry;

FULL_ADDER M0 (x[0], y[0], cin, sum[0], carry[0]);

FULL_ADDER M1 (x[1], y[1], carry[0], sum[1], carry[1]);

FULL_ADDER M2 (x[2], y[2], carry[1], sum[2], cout);

/* It is assumed that the module FULL_ADDER

(Listing 4.13) is attached by the simulator to

the module three_bit_adder so, no need to

rewrite the module FULL_ADDER.*/

endmodule

LISTING 4.15 Verilog Description of Carry-Save Adder (Figure 4.7)

module carry_saveadder(a1,b1,c1,d1,sum_total,final_carryout);

input[2:0] a1, b1,c1,d1;

output [3:0]sum_total;

output final_carryout;

full_adder FA1(a1[0],b1[0],c1[0], sm0,cr0);

//FA1 is a user-selected label

full_adder FA2(a1[1],b1[1],c1[1], sm1,cr1);

full_adder FA3(a1[2],b1[2],c1[2], sm2,cr2);

full_adder FA4(sm0,d1[0],1’b0, smm0,crr0);

full_adder FA5(sm1,d1[1],cr0, smm1,crr1);

full_adder FA6(sm2,d1[2],cr1, smm2,crr2);

assign sum_total[0] =smm0;

full_adder FA7(crr0,smm1,1’b0, sum_total[1],crrr0);

full_adder FA8(crr1,smm2,crrr0, sum_total[2],crrr1);

full_adder FA9(crr2,crrr1,cr2, sum_total[3],cc);

assign final_carryout = cc;

endmodule

module full_adder (a,b,c,Sum,Carryout);

input a,b,c;

output Sum, Carryout;

not (a_bar,a); // this is an inverter

not (b_bar,b);

not (c_bar,c);

and a1 (s0,a_bar,b_bar, c);/*This is And gate with

optional name a1*/

and a2 (s1,a_bar,b, c_bar);

and a3 (s2,a,b_bar, c_bar);

and a4 (s3,a,b,c);

or o1(Sum, s0,s1,s2,s3);

and a5 ( s5,a,b);

and a6 ( s6,a,c);

and a7 ( s7,b,c);

or o2( Carryout,s5,s6,s7);

endmodule

LISTING 4.16 HDL Description of a Three-Bit Comparator Using Adders

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity three_bit_cmpare is

port (X, Y : in std_logic_vector (2 downto 0);

xgty, xlty, xeqy : buffer std_logic);

end three_bit_cmpare;

architecture cmpare of three_bit_cmpare is

--Some simulators will not allow mapping between

--buffer and out. In this

--case, change all out to buffer.

component full_adder

port (I1, I2, I3 : in std_logic;

O1, O2 : out std_logic);

end component;

component Inv

port (I1 : in std_logic; O1 : out std_logic);

end component;

component nor2

port (I1, I2 : in std_logic; O1 : out std_logic);

end component;

component and3

port (I1, I2, I3 : in std_logic; O1 : out std_logic);

end component;

for all : full_adder use entity work.bind32 (full_add);

for all : Inv use entity work.bind1 (inv_0);

for all : nor2 use entity work.bind2 (nor2_0);

for all : and3 use entity work.bind3 (and3_7);

--To reduce hazards, an AND gate is

--implemented with a 7-ns delay.

signal sum, Yb : std_logic_vector (2 downto 0);

signal carry : std_logic_vector (1 downto 0);

begin

in1 : inv port map (Y(0), Yb(0));

in2 : inv port map (Y(1), Yb(1));

in3 : inv port map (Y(2), Yb(2));

F0 : full_adder port map (X(0), Yb(0),

‘0’, sum(0), carry(0));

F1 : full_adder port map (X(1), Yb(1), carry(0),

sum(1), carry(1));

F2 : full_adder port map (X(2), Yb(2), carry(1),

sum(2), xgty);

a1 : and3 port map (sum(0), sum(1), sum(2), xeqy);

n1 : nor2 port map (xeqy, xgty, xlty);

end cmpare;

Verilog Description

module three_bit_cmpare (X, Y, xgty, xlty, xeqy);

input [2:0] X, Y;

output xgty, xlty, xeqy;

wire [1:0] carry;

wire [2:0] sum, Yb;

not (Yb[0], Y[0]);

not (Yb[1], Y[1]);

not (Yb[2], Y[2]);

FULL_ADDER M0 (X[0], Yb[0], 1’b0, sum[0],

carry[0]);

FULL_ADDER M1 (X[1], Yb[1], carry[0], sum[1],

carry[1]);

FULL_ADDER M2 (X[2], Yb[2], carry[1], sum[2],

xgty);

and #7 (xeqy, sum[0], sum[1], sum[2]);

/* To reduce hazard use an AND gate with a delay of 7 units*/

nor (xlty, xeqy, xgty);

endmodule

LISTING 4.17 HDL Description of an SR-Latch with NOR Gates

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity SR_latch is

port (R, S : in std_logic;

Q, Qbar : buffer std_logic);

--Q, Qbar are declared buffer because

--they behave as input and output.

end SR_latch;

architecture SR_strc of SR_latch is

--Some simulators would not allow mapping between

--buffer and out. In this

--case, change all out to buffer.

component nor2

end component;

for all : nor2 use entity work.bind2 (nor2_0);

begin

n1 : nor2 port map (S, Q, Qbar);

n2 : nor2 port map (R, Qbar, Q);

end SR_strc;

Verilog Description

module SR_Latch (R, S, Q, Qbar);

input R, S;

output Q, Qbar;

nor (Qbar, S,Q);

nor (Q, R, Qbar);

endmodule

LISTING 4.18 HDL Description of a D-Latch with Active Low Clear

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity D_LatchWclr is

port (D, E,clrbar : in std_logic;

Q, Qbar : buffer std_logic);

end;

architecture D_latch_str of D_LatchWclr is

--be sure to use buffer rather than output in all

-- components; some simulators will not map output

--to buffer.

component and3

port (I1, I2, I3 : in std_logic;

O1 : buffer std_logic);

end component;

component nor2

port (I1, I2 : in std_logic; O1 : buffer std_logic);

end component;

component inv

port (I1 : in std_logic; O1 : buffer std_logic);

end component;

for all : and3 use entity work.bind3 (and3_4);

for all : nor2 use entity work.bind2 (nor2_4);

for all : inv use entity work.bind1 (inv_1);

signal Eb, s1, s2 : std_logic;

begin

a1 : and3 port map (D, E, clrbar, s1);

a2 : and3 port map (Eb, D,clrbar, s2);

in1 : inv port map (E, Eb);

in2 : inv port map (Qbar, Q);

n2 : nor2 port map (s1, s2, Qbar);

end D_latch_str;

Verilog Description

module D_latchWclr(D, E,clrbar, Q, Qbar);

input D, E, clrbar;

output Q, Qbar;

/* assume 4 ns delay for and gate and nor gate,

and 1 ns for inverter */

//The clear is active low; if clrbar = 0, Q=0

and #4 gate1 (s1, D, E, clrbar);

/* the name “gate1” is optional; we could have

written and #4 (s1, D, E) */

and #4 gate2 (s2, Eb, Q, clrbar);

not #1 (Eb, E);

nor #4 (Qbar, s1, s2);

not #1 (Q, Qbar);

endmodule

LISTING 4.19 HDL Description of a Master-Slave D Flip-Flop with Active

Low Clear

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity D_FFMasterWclr is

Port (D, clk, clrbar : in std_logic;

Q, Qbar : buffer std_logic);

end D_FFMasterWclr ;

architecture D_FF_str of D_FFMasterWclr is

component inv

port (I1 : in std_logic; O1 : buffer std_logic);

end component;

component D_latchWclrbar

port (I1, I2, I3 : in std_logic;

O1, O2 : buffer std_logic);

end component;

for all : D_latchWclrbar use

entity work. bind32(D_latch_Wclr);

for all : inv use entity work.bind1 (inv_1);

signal clkb, clk2, Q0, Qb0 : std_logic;

begin

D0 : D_latchWclrbar port map (D, clkb,clrbar, Q0, Qb0);

D1 : D_latchWclrbar port map (Q0, clk2, clrbar, Q, Qbar);

in1 : inv port map (clk, clkb);

in2 : inv port map (clkb, clk2);

end D_FF_str;

Verilog Description

module D_FFMasterWclr(D, clk,clrbar, Q, Qbar);

input D, clk, clrbar;

output Q, Qbar;

not #1 (clkb, clk);

not #1 (clk2, clkb);

D_latchWclr D0 (D, clkb,clrbar, Q0, Qb0);

D_latchWclr D1 (Q0, clk2,clrbar, Q, Qbar);

endmodule

LISTING 4.20 HDL Description of a Master-Slave JK Flip-Flop: VHDL and

Verilog

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity JK_FLFL is

port (J, K, clk, clrbar : in std_logic;

Q, Qbar : buffer std_logic);

-- Q and Qbar are declared buffer so they can be input

-- or output

end JK_FLFL;

architecture JK_Master of JK_FLFL is

component and2

port (I1, I2 : in std_logic; O1 : buffer std_logic);

end component;

component or2

port (I1, I2 : in std_logic; O1 : buffer std_logic);

end component;

component inv

port (I1 : in std_logic; O1 : buffer std_logic);

end component;

component D_FFMasterWclr

port (D, clk, clrbar : in std_logic;

Q, Qbar : buffer std_logic);

end component;

for all : and2 use entity work.bind2 (and2_4);

for all : or2 use entity work.bind2 (or2_4);

for all : inv use entity work.bind1 (inv_1);

for all : D_FFMasterWclr use

entity work. D_FFMasterWclr (D_FF_str);

signal s1, s2, Kb, DD : std_logic;

begin

a1 : and2 port map (J, Qbar, s1);

a2 : and2 port map (Kb, Q, s2);

in1 : inv port map (K, Kb);

or1 : or2 port map (s1, s2, DD);

DFF : D_FFMasterWclr port map (DD, clk,clrbar, Q, Qbar);

end JK_Master;

Verilog Description

module JK_FF (J, K, clk,clrbar, Q, Qbar);

input J, K, clk, clrbar;

output Q, Qbar;

wire s1, s2;

and #4 (s1, J, Qbar);

and #4 (s2, Kb, Q);

not #1 (Kb, K);

or #4 (DD, s1, s2);

D_FFMasterWclr D0 (DD, clk,clrbar, Q, Qbar);

endmodule

module D_FFMasterWclr(D, clk,clrbar, Q, Qbar);

/* no need to rewrite this module here if it has

been already attached to the above module (JK_FF). */

input D, clk, clrbar;

output Q, Qbar;

not #1 (clkb, clk);

not #1 (clk2, clkb);

D_latchWclr D0 (D, clkb,clrbar, Q0, Qb0);

D_latchWclr D1 (Q0, clk2,clrbar, Q, Qbar);

endmodule

module D_latchWclr(D, E,clrbar, Q, Qbar);

/* no need to rewrite this module here if it has

been already attached to the above module (JK_FF). */

input D, E, clrbar;

output Q, Qbar;

/* assume 4 ns delay for and gate and nor gate,

and 1 ns for inverter */

//The clear is active low; if clrbar = 0, Q=0

and #4 gate1 (s1, D, E, clrbar);

/* the name “gate1” is optional; we could have

written and #4 (s1, D, E) */

and #4 gate2 (s2, Eb, Q, clrbar);

not #1 (Eb, E);

nor #4 (Qbar, s1, s2);

not #1 (Q, Qbar);

endmodule

LISTING 4.21 HDL Description of an SRAM Memory Cell: VHDL and Verilog

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity memory is

port (Sel, RW, Din : in std_logic;

O1: buffer std_logic );

end memory;

architecture memory_str of memory is

--Some simulators will not allow mapping between

--buffer and out. In this

--case, change all out to buffer.

component and3

port (I1, I2, I3 : in std_logic; O1 : out std_logic);

end component;

component inv

port (I1 : in std_logic; O1 : out std_logic);

end component;

component or2

port (I1, I2 : in std_logic; O1 : out std_logic);

end component;

component bufif1

port (I1, I2 : in std_logic; O1 : out std_logic);

end component;

component SR_Latch

port (I1, I2 : in std_logic;

O1, O2 : buffer std_logic);

end component;

for all : and3 use entity work.bind3 (and3_0);

for all : inv use entity work.bind1 (inv_0);

for all : or2 use entity work.bind2 (or2_0);

for all : bufif1 use entity work.bind2 (bufif1);

for all : SR_Latch use entity work.bind22 (SR_Latch);

signal RWb, Dinb, S, S1, R, O11, Q : std_logic;

begin

in1 : inv port map (RW, RWb);

in2 : inv port map (Din, Dinb);

a1 : and3 port map (Sel, RWb, Din, S);

a2 : and3 port map (Sel, RWb, Dinb, R);

SR1 : SR_Latch port map (S, R, Q, open);

--open is a predefined word;

--it indicates that the port is left open.

a3 : and3 port map (Sel, RW, Q, S1);

or1 : or2 port map (S1, S, O11);

buf1 : bufif1 port map (O11, Sel, O1);

end memory_str;

Verilog Description

module memory (Sel, RW, Din, O1);

input Sel, RW, Din;

output O1;

not (RWb, RW);

not (Dinb, Din);

and (S, Sel, RWb, Din);

and (R, Sel, RWb, Dinb);

SR_Latch RS1 (R, S, Q, Qbar);

and (S1, Sel, RW, Q);

or (O11, S1, S);

bufif1 (O1, O11, Sel);

endmodule

LISTING 4.22 Verilog Description of a Three-Bit Universal Shift Register

module shft_regsterUniv(clk, clrbar,

s0,s1,P,DSR,DSL,Q,Qb);

input clk, clrbar,s0,s1,DSR,DSL;

output [2:0] Q,Qb,P;

not (s0bar, s0);

not (s0t,s0bar);

not (s1bar, s1);

not (s1t, s1bar);

and #4 a0(aa0, DSR, s1bar,s0t);

and #4 a1(aa1, s0t, s1t,P[2]);

and #4 a2(aa2, s0bar, s1t,Q[1]);

and #4 a3(aa3, s0bar, s1bar,Q[2]);

or #4 or2 (D2,aa0,aa1,aa2,aa3);

D_FFMasterWclr DFM0(D2,clk,clrbar,Q[2],Qb[2]);

and #4 a4(aa4, Q[2], s1bar,s0t);

and #4 a5(aa5, s0t, s1t,P[1]);

and #4 a6(aa6, s0bar, s1t,Q[0]);

and #4 a7(aa7, s0bar, s1bar,Q[1]);

or #4 or1 (D1,aa4,aa5,aa6,aa7);

D_FFMasterWclr DFM1(D1,clk, clrbar,Q[1],Qb[1]);

and #4 a8(aa8, Q[1], s1bar,s0t);

and #4 a9(aa9, s0t, s1t,P[0]);

and #4 a10(aa10, s0bar, s1t,DSL);

and #4 a11(aa11, s0bar, s1bar,Q[0]);

or #4 or0 (D0,aa8,aa9,aa10,aa11);

D_FFMasterWclr DFM2(D0,clk,clrbar,Q[0], Qb[0]);

endmodule

LISTING 4.23 HDL Description of a Three-Bit Synchronous Counter Using

Master-Slave JK Flip-Flops: VHDL and Verilog

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity CTStatemachine is

port( clk, clrbar : in std_logic;

Q, Qbar: buffer std_logic_vector (2 downto 0));

end CTStateMachine;

architecture ct_3 of CTStateMachine is

component and2

port (I1, I2 : in std_logic; O1 : buffer std_logic);

end component;

component JK_FLFL

port (J, K, clk, clrbar : in std_logic;

Q, Qbar : buffer std_logic);

end component;

for all : and2 use entity work.bind2 (and2_4);

for all : JK_FLFL use entity work.

JK_FLFL (JK_Master );

--Be sure to attach the entity-architectures

-- shown above

signal J2,K2 : std_logic;

begin

JK0 : JK_FLFL port map (‘1’, ‘1’, clk, clrbar, Q(0), Qbar(0));

JK1 : JK_FLFL port map (q(0), q(0), clk, clrbar, Q(1), Qbar(1));

A1: and2 port map (q(0), q(1), J2);

A2: and2 port map (q(0), q(1), K2);

JK2 : JK_FLFL port map (J2, K2, clk, clrbar, Q(2), Qbar(2));

end ct_3;

Verilog Description

module CTstatemachine(clk, clrbar, q, qb);

input clk, clrbar;

output [2:0] q, qb;

JK_FF FF0(1’b1, 1’b1, clk, clrbar, q[0], qb[0]);

assign J1 = q[0]; /* a buffer could have been used here

and in all assign statement in this module*/

assign K1 = q[0];

JK_FF FF1 (J1, K1, clk, clrbar, q[1], qb[1]);

and A1 (J2, q[0], q[1]);

assign K2 = J2;

JK_FF FF2(J2, K2, clk, clrbar, q[2], qb[2]);

endmodule

LISTING 4.24 HDL Description of a Three-Bit Synchronous Even Counter

with Hold: VHDL and Verilog

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity CTR_EVEN is

port (H, clk, clrbar : in std_logic;

Q, Qbar : buffer std_logic_vector (2 downto 0));

-- Input clrbar is added to help in testing;

-- set clrbar to low initially when testing

--to clear the output and then set it back to high

end CTR_EVEN;

architecture Counter_even of CTR_EVEN is

--Some simulators will not allow mapping between

--buffer and out. In this

--case, change all out to buffer.

component inv

port (I1 : in std_logic; O1 : buffer std_logic);

end component;

component and2

port (I1, I2 : in std_logic; O1 : buffer std_logic);

end component;

component or2

port (I1, I2 : in std_logic; O1 : buffer std_logic);

end component;

component and3

port (I1, I2, I3 : in std_logic;

O1 : buffer std_logic);

end component;

component or3

port (I1, I2, I3 : in std_logic;

O1 : buffer std_logic);

end component;

component D_FFMasterWclr

port (D, clk, clrbar : in std_logic;

Q, Qbar : buffer std_logic);

end component;

for all : D_FFMasterWclr use

entity work. D_FFMasterWclr (D_FF_str);

for all : inv use entity work.bind1 (inv_0);

for all : and2 use entity work.bind2 (and2_0);

for all : and3 use entity work.bind3 (and3_0);

for all : or2 use entity work.bind2 (or2_0);

for all : or3 use entity work.bind3 (or3_0);

signal Hbar, a1, a2, a3, a4,

a5, OR11, OR22 : std_logic;

begin

DFF0 : D_FFMasterWclr port map (‘0’, clk, clrbar, Q(0),Qbar(0));

inv1 : inv port map (H, Hbar);

an1 : and2 port map (Hbar, Qbar(1), a1);

an2 : and3 port map (H, Q(1), Qbar(0), a2);

r1 : or2 port map (a2, a1, OR11);

DFF1 : D_FFMasterWclr port map (OR11, clk, clrbar,

Q(1), Qbar(1));

an3 : and3 port map (Q(2), Qbar(1), Qbar(0), a3);

an4 : and3 port map (Qbar(0), H, Q(2), a4);

an5 : and3 port map (Hbar, Qbar(2), Q(1), a5);

r2 : or3 port map (a3, a4, a5, OR22);

DFF2 : D_FFMasterWclr port map (OR22, clk, clrbar,

Q(2), Qbar(2));

end Counter_even;

Verilog Description

module CTR_EVEN(H, clk, clrbar, Q, Qbar);

// Input clrbar is added to help in testing;

//set clrbar to low initially when testing

//to clear the output and then set it back to high

input H, clk, clrbar;

output [2:0] Q, Qbar;

D_FFMasterWclr DFF0 (1’b0, clk, clrbar, Q[0], Qbar[0]);

not (Hbar, H);

and (a1, Qbar[1], Hbar);

and (a2, H, Q[1], Qbar[0]);

or (OR1, a1, a2);

D_FFMasterWclr DFF1 (OR1, clk, clrbar, Q[1], Qbar[1]);

and (a3, Q[2], Qbar[1], Qbar[0]);

and (a4, Qbar[0], H, Q[2]);

and (a5, Hbar, Qbar[2], Q[1]);

or (OR2, a3, a4, a5);

D_FFMasterWclr DFF2 (OR2, clk, clrbar, Q[2], Qbar[2]);

endmodule

LISTING 4.25 HDL Description of a 3-Bit Synchronous Up/Down Counter

with Clear and Terminal Count—VHDL and Verilog

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity up_down is

port (clr, Dir, clk : in std_logic;

TC : buffer std_logic;

Q, Qbar : buffer std_logic_vector (2 downto 0));

end up_down;

architecture Ctr_updown of up_down is

--Some simulators will not allow mapping between

--buffer and out. In this

--case, change all out to buffer.

component inv

port (I1 : in std_logic; O1 : buffer std_logic);

end component;

component and2

port (I1, I2 : in std_logic; O1 : buffer std_logic);

end component;

component or2

port (I1, I2 : in std_logic; O1 : buffer std_logic);

end component;

component or3

port (I1, I2,I3 : in std_logic; O1 : buffer std_logic);

end component;

component and3

port (I1, I2, I3 : in std_logic;

O1 : buffer std_logic);

end component;

component and4

port (I1, I2, I3, I4 : in std_logic;

O1 : buffer std_logic);

end component;

component JK_FLFL

port (J, K, clk, clrbar : in std_logic;

Q, Qbar : buffer std_logic);

end component;

for all : JK_FLFL use entity work.

JK_FLFL (JK_Master );

for all : inv use entity work.bind1 (inv_1);

for all : and2 use entity work.bind2 (and2_4);

for all : and3 use entity work.bind3 (and3_4);

for all : and4 use entity work.bind4 (and4_4);

for all : or2 use entity work.bind2 (or2_4);

for all : or3 use entity work.bind3 (or3_4);

--Be sure that all the reference entities

--above such as JK_FLFL

--are attached in the project.

signal clrbar, Dirbar, J1, K1, J2, K2 : std_logic;

signal s : std_logic_vector (5 downto 0);

begin

in1 : inv port map (clr, clrbar);

in2 : inv port map (Dir, Dirbar);

an2 : and2 port map (Dir, Q(0), s(1));

an3 : and3 port map (Dirbar, Qbar(1), Qbar(0), s(2));

an4 : and3 port map (Dir, Q(1), Q(0), s(3));

an5 : and4 port map (Dir, Q(1), Q(0), Q(2), s(4));

an6 : and4 port map (Dirbar, Qbar(1),

Qbar(0), Qbar(2), s(5));

r0 : or3 port map (s(0), s(1), Q(1), J1);

r1 : or2 port map (s(0), s(1), K1);

r2 : or2 port map (s(2), s(3), J2);

K2 <= J2;

r3 : or2 port map (s(4), s(5), TC);

JKFF0 : JK_FLFL port map

(‘1’, ‘1’, clk, clrbar, Q(0), Qbar(0));

JKFF1 : JK_FLFL port map

(J1, K1, clk, clrbar, Q(1), Qbar(1));

JKFF2 : JK_FLFL port map

(J2, K2, clk, clrbar, Q(2), Qbar(2));

end Ctr_updown;

Verilog Description

module up_down(clr, Dir, clk, Q, Qbar, TC);

input clr, Dir, clk;

output [2:0] Q, Qbar;

output TC;

not #1 (clrbar, clr);

not #1 (Dirbar, Dir);

and #4 a1(s0, Dirbar, Qbar[0]);

and #4 a2(s1, Dir, Q[0]);

and #4 a3(s2, Dirbar, Qbar[0], Qbar[1]);

and #4 a4(s3, Q[0], Q[1], Dir);

and #4 a5(s4, Dirbar, Qbar[0], Qbar[1], Qbar[2]);

and #4 a6(s5, Q[0], Q[1], Q[2],Dir);

or #4 r1(J1, s0, Q[1], s1);

or #4 r2(K1, s0, s1);

or #4 r3(J2, s2, s3);

assign K2 = J2;// a buffer can be

//used to generate the above statement

or #4 r4(TC, s4, s5);

JK_FF JKFF0 (1’b1, 1’b1, clk, clrbar, Q[0], Qbar[0]);

JK_FF JKFF1 (J1, K1, clk, clrbar, Q[1], Qbar[1]);

JK_FF JKFF2 (J2, K2, clk,clrbar, Q[2], Qbar[2]);

/*Be sure that all the reference entities above

such as JK_FLFL are attached in the project.*/

endmodule

LISTING 4.26 HDL Description of a Three-Bit Synchronous Decade Counter

with Terminal Count: VHDL and Verilog

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity decade_ctr is

port (clk, clr : in std_logic;

Q, Qbar : buffer std_logic_vector (3 downto 0);

TC : buffer std_logic);

end decade_ctr;

architecture decade_str of decade_ctr is

--Some simulators will not allow mapping between

--buffer and out. In this

--case, change all out to buffer.

component inv

port (I1 : in std_logic; O1 : buffer std_logic);

end component;

component buf

port (I1 : in std_logic; O1 : buffer std_logic);

end component;

component and2

port (I1, I2 : in std_logic; O1 : buffer std_logic);

end component;

component and3

port (I1, I2, I3 : in std_logic;

O1 : buffer std_logic);

end component;

component and4

port (I1, I2, I3, I4 : in std_logic;

O1 : buffer std_logic);

end component;

component or2

port (I1, I2 : in std_logic; O1 : buffer std_logic);

end component;

component or3

port (I1, I2, I3 : in std_logic;

O1 : buffer std_logic);

end component;

component D_FFMasterWclr

port (D, clk, clrbar : in std_logic;

Q, Qbar : buffer std_logic);

end component;

for all : D_FFMasterWclr use entity

work. D_FFMasterWclr (D_FF_str);

for all : inv use entity work.bind1 (inv_1);

for all : buf use entity work.bind1 (buf_1);

for all : and2 use entity work.bind2 (and2_4);

for all : and3 use entity work.bind3 (and3_4);

for all : and4 use entity work.bind4 (and4_4);

for all : or2 use entity work.bind2 (or2_4);

for all : or3 use entity work.bind3 (or3_4);

signal s : std_logic_vector (6 downto 0);

signal D : std_logic_vector (3 downto 0);

signal clrbar : std_logic;

begin

i1 : inv port map( clr, clrbar);

b1 : buf port map (Qbar(0), D(0));

DFF0 : D_FFMasterWclr port map (D(0), clk, clrbar,

Q(0), Qbar(0));

--Assume AND gates and OR gates have 4 ns propagation

--delay and invert has 1 ns.

a1 : and3 port map (Qbar(3), Qbar(1), Q(0), s(0));

a2 : and2 port map (Q(1), Qbar(0), s(1));

r1 : or2 port map (s(0), s(1), D(1));

DFF1 : D_FFMasterWclr port map (D(1), clk, clrbar,

Q(1), Qbar(1));

a3 : and2 port map (Q(2), Qbar(1), s(2));

a4 : and2 port map (Q(2), Qbar(0), s(3));

a5 : and3 port map (Q(1), Q(0), Qbar(2), s(4));

r2 : or3 port map (s(2), s(3), s(4), D(2));

DFF2 : D_FFMasterWclr port map (D(2), clk, clrbar,

Q(2), Qbar(2));

a6 : and2 port map (Q(3), Qbar(0), s(5));

a7 : and4 port map (Q(0), Q(1), Q(2), Qbar(3), s(6));

r3 : or2 port map (s(5), s(6), D(3));

DFF3 : D_FFMasterWclr port map (D(3), clk, clrbar,

Q(3), Qbar(3));

a8 : and4 port map (Q(0), Qbar(1), Qbar(2), Q(3), TC);

end decade_str;

Verilog Description

module decade_ctr(clk, clrbar,Q, Qbar, TC );

input clk,clrbar;

//use clrbar input to clear the counter when simulting

output [3:0] Q, Qbar;

output TC;

wire [3:0] D;

wire [6:0] s;

buf #1 (D[0], Qbar[0]);

D_FFMasterWclr DFF0 (D[0], clk, clrbar, Q[0], Qbar[0]);

/*Assume and gates and or gates have 4 ns propagation

delay and invert has 1 ns.*/

and #4 (s[0], Qbar[3], Qbar[1], Q[0]);

and #4 (s[1], Q[1], Qbar[0]);

or #4 (D[1], s[0], s[1]);

D_FFMasterWclr FF1 (D[1], clk, clrbar, Q[1], Qbar[1]);

and #4 (s[2],Q[2], Qbar[1]);

and #4 (s[3],Q[2], Qbar[0]);

and #4 (s[4],Q[1], Q[0], Qbar[2]);

or #4 (D[2], s[2], s[3], s[4]);

D_FFMasterWclr FF2 (D[2], clk,clrbar, Q[2], Qbar[2]);

and #4 (s[5], Q[3], Qbar[0]);

and #4 (s[6], Q[0], Q[1], Q[2], Qbar[3]);

or #4 (D[3], s[5], s[6]);

D_FFMasterWclr FF3 (D[3], clk,clrbar, Q[3], Qbar[3]);

and #4 (TC, Q[0], Qbar[1], Qbar[2], Q[3]);

endmodule

LISTING 4.28 HDL Description of N-Bit Magnitude Comparator Using the

generate Statement: VHDL and Verilog

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity compr_genr is

generic (N : integer := 3);

port (X, Y : in std_logic_vector (N downto 0);

xgty, xlty, xeqy : buffer std_logic);

end compr_genr;

architecture cmpare_str of compr_genr is

--Some simulators will not allow mapping between

--buffer and out. In this

--case, change all out to buffer.

component full_adder

port (I1, I2, I3 : in std_logic;

O1, O2 : buffer std_logic);

end component;

component inv

port (I1 : in std_logic; O1 : buffer std_logic);

end component;

component nor2

port (I1, I2 : in std_logic; O1 : buffer std_logic);

end component;

component and2

port (I1, I2 : in std_logic; O1 : buffer std_logic);

end component;

signal sum, Yb : std_logic_vector (N downto 0);

signal carry, eq : std_logic_vector (N + 1 downto 0);

for all : full_adder use entity work.bind32 (full_add);

for all : inv use entity work.bind1 (inv_1);

for all : nor2 use entity work.bind2 (nor2_7);

for all : and2 use entity work.bind2 (and2_7);

begin

carry(0) <= ‘0’;

eq(0) <= ‘1’;

G1 : for i in 0 to N generate

v1 : inv port map (Y(i), Yb(i));

FA : full_adder port map (X(i), Yb(i), carry(i),

sum(i), carry(i+1));

a1 : and2 port map (eq(i), sum(i), eq(i+1));

end generate G1;

xgty <= carry(N+1);

xeqy <= eq(N+1);

n1 : nor2 port map (xeqy, xgty, xlty);

end cmpare_str;

Verilog Description

module Compr_genr(X, Y, xgty, xlty, xeqy);

parameter N = 3;

input [N:0] X, Y;

output xgty, xlty, xeqy;

wire [N:0] sum, Yb;

wire [N+1 : 0] carry, eq;

assign carry[0] = 1’b0;

assign eq[0] = 1’b1;

generate

genvar i;

for (i = 0; i <= N; i = i + 1)

begin : u

not (Yb[i], Y[i]);

/* The above statement is equivalent to assign Yb = ~Y if outside

the generate loop */

FULL_ADDER FA(X[i], Yb[i], carry [i], sum [i], carry[i+1]);

/*be sure that the module FULL_ADDER

is entered (attached) in the project*/

and (eq[i+1], sum[i], eq[i]);

end

endgenerate

assign xgty = carry[N+1];

assign xeqy = eq[N+1];

nor (xlty, xeqy, xgty);

endmodule

LISTING 4.29 HDL Description of an N-Bit Asynchronous Down Counter Using

generate: VHDL and Verilog

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity asynch_ctr is

Generic (N : integer := 3);

-- This is a 3-bit counter. If a different number of

-- bits is needed, simply change the

-- value of N here only.

port (clk, clrbar : in std_logic;

Q, Qbar : buffer std_logic_vector (N-1 downto 0));

end asynch_ctr;

architecture CT_strgnt of asynch_ctr is

--Some simulators will not allow mapping between

--buffer and out. In this

--case, change all out to buffer.

component JK_FLFL

port (J, K, clk, clrbar : in std_logic;

Q, Qbar : buffer std_logic);

end component;

for all : JK_FLFL use entity work.

JK_FLFL (JK_Master );

-- For bind32, see Listing 4.17a

signal h, l : std_logic;

signal s : std_logic_vector (N downto 0);

begin

h <= ‘1’;

l <= ‘0’;

s <= (Q & clk);

-- s is the concatenation of Q and clk. We need

-- this concatenation to

-- describe the clock of each JK flip-flop.

Gnlop : for i in (N-1) downto 0 generate

G1 : JK_FLFL port map (h, h, s(i), clrbar,

Q(i), Qbar(i));

end generate GnLop;

end CT_strgnt;

Verilog Description

module asynch_ctr(clk,clrbar, Q, Qbar);

parameter N = 3;

/* This is a 3-bit counter. If a different number of

bits is needed, simply change the value

of N here only.*/

input clk, clrbar;

output [N-1:0] Q, Qbar;

wire [N:0] s;

assign s = {Q, clk};

/* s is the concatenation of Q and clk.

This concatenation is needed to describe the clock

of each JK flip-flop. */

generate

genvar i;

for (i = 0; i < N; i = i + 1)

begin : u

JK_FF JKFF0 (1’b1, 1’b1, s[i],clrbar, Q[i],

Qbar[i]);

// JK_FF is as shown in Listing 4.17b

end

endgenerate

LISTING 4.30 HDL Description of N-Bit Memory Word Using generate:

VHDL and Verilog

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity Memory_word is

Generic (N : integer := 7);

port (Data_in : in std_logic_vector (N downto 0);

sel, R_W : in std_logic; Data_out : out

std_logic_vector (N downto 0));

end Memory_word;

architecture Word_generate of Memory_word is

component memory_cell

Port (Sel, RW, Din : in std_logic;

O1 : buffer std_logic );

end component;

for all : memory_cell use entity

work.memory (memory_str);

begin

G1 : for i in 0 to N generate

M : memory_cell port map (sel, R_W, Data_in(i),

Data_out(i));

end generate;

end Word_generate;

Verilog Description

module Memory_Word (Data_in, sel, R_W, Data_out);

parameter N = 7;

input [N:0] Data_in;

input sel, R_W;

output [N:0] Data_out;

generate

genvar i;

for (i = 0; i <= N; i = i + 1)

begin : u

memory M1 (sel, R_W, Data_in [i], Data_out[i]);

end

endgenerate

endmodule

LISTING 4.31 VHDL Code for Components Used for Binding in Chapter 4

--Some simulators will not allow mapping between

--buffer and out. In this

--case, change all out to buffer as it is done here.

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity bind1 is

port (O1 : buffer std_logic; I1 : in std_logic);

end bind1;

architecture inv_0 of bind1 is

begin

O1 <= not I1;

end inv_0;

architecture inv_1 of bind1 is

begin

O1 <= not I1 after 1 ns;

end inv_1;

architecture inv_7 of bind1 is

begin

O1 <= not I1 after 7 ns;

end inv_7;

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity bind2 is

port (O1 : buffer std_logic; I1, I2 : in std_logic);

end bind2;

architecture xor2_0 of bind2 is

begin

O1 <= I1 xor I2;

end xor2_0;

architecture and2_0 of bind2 is

begin

O1 <= I1 and I2;

end and2_0;

architecture and2_4 of bind2 is

begin

O1 <= I1 and I2 after 4 ns;

end and2_4;

architecture and2_7 of bind2 is

begin

O1 <= I1 and I2 after 7 ns;

end and2_7;

architecture or2_0 of bind2 is

begin

O1 <= I1 or I2;

end or2_0;

architecture or2_7 of bind2 is

begin

O1 <= I1 or I2 after 7 ns;

end or2_7;

architecture nor2_0 of bind2 is

begin

O1 <= I1 nor I2;

end nor2_0;

architecture nor2_7 of bind2 is

begin

O1 <= I1 nor I2 after 7 ns;

end nor2_7;

architecture nor2_4 of bind2 is

begin

O1 <= I1 nor I2 after 4 ns;

end nor2_4;

architecture bufif1 of bind2 is

begin

buf : process (I1, I2)

variable tem : std_logic;

begin

if (I2 =’1’)then

tem := I1;

else

tem := ‘Z’;

end if;

O1 <= tem;

end process buf;

end bufif1;

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity bind3 is

port (O1 : buffer std_logic;

I1, I2, I3 : in std_logic);

end bind3;

architecture and3_0 of bind3 is

begin

O1 <= I1 and I2 and I3;

end and3_0;

architecture and3_4 of bind3 is

begin

O1 <= I1 and I2 and I3 after 4 ns;

end and3_4;

architecture and3_7 of bind3 is

begin

O1 <= I1 and I2 and I3 after 7 ns;

end and3_7;

architecture or3_0 of bind3 is

begin

O1 <= I1 or I2 or I3;

end or3_0;

architecture or3_7 of bind3 is

begin

O1 <= I1 or I2 or I3 after 7 ns;

end or3_7;

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity bind22 is

Port (O1, O2 : buffer std_logic;

I1, I2 : in std_logic);

end bind22;

architecture HA of bind22 is

component xor2

port (I1, I2 : in std_logic; O1 : buffer std_logic);

end component;

component and2

port (I1, I2 : in std_logic; O1 : buffer std_logic);

end component;

for A1 : and2 use entity work.bind2 (and2_0);

for X1 : xor2 use entity work.bind2 (xor2_0);

begin

X1 : xor2 port map (I1, I2, O1);

A1 : and2 port map (I1, I2, O2);

end HA;

architecture SR_Latch of bind22 is

component nor2

port (I1, I2 : in std_logic; O1 : buffer std_logic);

end component;

for all : nor2 use entity work.bind2 (nor2_0);

begin

n1 : nor2 port map (I1, O1, O2);

n2 : nor2 port map (I2, O2, O1);

end SR_Latch;

architecture D_latch of bind22 is

component and2

port (I1, I2 : in std_logic; O1 : buffer std_logic);

end component;

component nor2

port (I1, I2 : in std_logic; O1 : buffer std_logic);

end component;

component inv

port (I1 : in std_logic; O1 : buffer std_logic);

end component;

for all : and2 use entity work.bind2 (and2_4);

for all : nor2 use entity work.bind2 (nor2_4);

for all : inv use entity work.bind1 (inv_1);

signal I2b, s1, s2 : std_logic;

begin

a1 : and2 port map (I1, I2, s1);

a2 : and2 port map (I2b, O1, s2);

in1 : inv port map (I2, I2b);

in2 : inv port map (O2, O1);

n2 : nor2 port map (s1, s2, O2);

end D_latch;

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity bind32 is

port (I1, I2, I3 : in std_logic;

O1, O2 : buffer std_logic);

end bind32;

architecture full_add of bind32 is

component HA

port (I1, I2 : in std_logic;

O1, O2 : buffer std_logic);

end component;

component or2

port (I1, I2 : in std_logic; O1 : buffer std_logic);

end component;

for all : HA use entity work.bind22 (HA);

for all : or2 use entity work.bind2 (or2_0);

signal s0, c0, c1 : std_logic;

begin

HA1 : HA port map (I2, I3, s0, c0);

HA2 : HA port map (I1, s0, O1, c1);

r1 : or2 port map (c0, c1, O2);

end full_add;

architecture D_latch_Wclr of bind32 is

component and3

port (I1, I2, I3 : in std_logic;

O1 : buffer std_logic);

end component;

component nor2

port (I1, I2 : in std_logic; O1 : buffer std_logic);

end component;

component inv

port (I1 : in std_logic; O1 : buffer std_logic);

end component;

for all : and3 use entity work.bind3 (and3_4);

for all : nor2 use entity work.bind2 (nor2_4);

for all : inv use entity work.bind1 (inv_1);

signal I2b, s1, s2 : std_logic;

begin

a1 : and3 port map (I1, I2, I3, s1);

a2 : and3 port map (I2b, O1,I3, s2);

in1 : inv port map (I2, I2b);

in2 : inv port map (O2, O1);

n2 : nor2 port map (s1, s2, O2);

end D_latch_Wclr;

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity D_LatchWclr is

port (D, E,clrbar : in std_logic;

Q, Qbar : buffer std_logic);

end;

architecture D_latch_str of D_LatchWclr is

component and3

port (I1, I2, I3 : in std_logic;

O1 : buffer std_logic);

end component;

component nor2

port (I1, I2 : in std_logic; O1 : buffer std_logic);

end component;

component inv

port (I1 : in std_logic; O1 : buffer std_logic);

end component;

for all : and3 use entity work.bind3 (and3_4);

for all : nor2 use entity work.bind2 (nor2_4);

for all : inv use entity work.bind1 (inv_1);

signal Eb, s1, s2 : std_logic;

begin

a1 : and3 port map (D, E, clrbar, s1);

a2 : and3 port map (Eb, D,clrbar, s2);

in1 : inv port map (E, Eb);

in2 : inv port map (Qbar, Q);

n2 : nor2 port map (s1, s2, Qbar);

end D_latch_str;

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity D_FFMasterWclr is

Port (D, clk, clrbar : in std_logic;

Q, Qbar : buffer std_logic);

end D_FFMasterWclr ;

architecture D_FF_str of D_FFMasterWclr is

component inv

port (I1 : in std_logic; O1 : buffer std_logic);

end component;

component D_latchWclrbar

port (I1, I2, I3 : in std_logic;

O1, O2 : buffer std_logic);

end component;

for all : D_latchWclrbar use entity

work. bind32(D_latch_Wclr);

for all : inv use entity work.bind1 (inv_1);

signal clkb, clk2, Q0, Qb0 : std_logic;

begin

D0 : D_latchWclrbar port map (D, clkb,clrbar, Q0, Qb0);

D1 : D_latchWclrbar port map (Q0, clk2, clrbar, Q,

Qbar);

in1 : inv port map (clk, clkb);

in2 : inv port map (clkb, clk2);

end D_FF_str;

Chapter 5

LISTING 5.1 VHDL Behavioral Code for NMOS and PMOS Switches

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity mos is

Port (O1 : out std_logic; I1, I2 : in std_logic);

end mos;

architecture nmos_behavioral of mos is

-- All switches presented here do not include any

-- time parameters, such as rise time and fall time.

-- They only mimic the logical functions of their

-- Verilog counterparts.

begin

switch : process (I1, I2)

variable temp : std_logic;

begin

case I2 is

when ‘0’=> temp := ‘Z’;

when ‘1’ => temp := I1;

when others => case I1 is

when ‘0’ => temp := ‘L’;

when ‘1’ => temp := ‘H’;

when others => temp := I1;

end case;

end case;

O1 <= temp;

end process switch;

end nmos_behavioral;

architecture pmos_behavioral of mos is

begin

switch : process (I1, I2)

variable temp : std_logic;

begin

case I2 is

when ‘1’=> temp := ‘Z’;

when ‘0’ => temp := I1;

when others => case I1 is

when ‘0’ => temp := ‘L’;

when ‘1’ => temp := ‘H’;

when others => temp := I1;

end case;

end case;

O1 <= temp;

end process switch;

end pmos_behavioral;

LISTING 5.2 VHDL Code for NMOS and PMOS Switches as Components

architecture nmos of mos is

component nmos

port (O1 : out std_logic; I1, I2 : in std_logic);

end component;

component pmos

port (O1 : out std_logic; I1, I2 : in std_logic);

end component;

for all : pmos use entity work.mos (pmos_behavioral);

for all : nmos use entity work.mos (nmos_behavioral);

LISTING 5.3 HDL Code for an Inverter: VHDL and Verilog

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity Inverter is

port (y : out std_logic; a : in std_logic);

end Inverter;

architecture Invert_switch of Inverter is

component nmos

port (O1 : out std_logic; I1, I2 : in std_logic);

end component;

component pmos

port (O1 : out std_logic; I1, I2 : in std_logic);

end component;

for all : pmos use entity work.mos (pmos_behavioral);

for all : nmos use entity work.mos (nmos_behavioral);

constant vdd : std_logic := ‘1’;

constant gnd : std_logic := ‘0’;

begin

p1 : pmos port map (y, vdd, a);

n1 : nmos port map (y, gnd, a);

end Invert_switch;

Verilog Description

module invert (y, a);

input a;

output y;

supply1 vdd; /*supply1 is a predefined word for the

high voltage.*/

supply0 gnd; /*supply0 is a predefined word for the

ground.*/

pmos p1 (y, vdd, a); /*the name “p1” is optional; it

can be omitted.*/

nmos n1 (y, gnd, a); /*the name “n1” is optional; it can

be omitted. */

endmodule

LISTING 5.4 HDL Code for a Two-Input AND Gate: VHDL and Verilog

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity and2gate is

port (y : out std_logic; a, b : in std_logic);

end and2gate;

architecture and_switch of and2gate is

component nmos

port (O1 : out std_logic; I1, I2 : in std_logic);

end component;

component pmos

port (O1 : out std_logic; I1, I2 : in std_logic);

end component;

for all : pmos use entity work.mos (pmos_behavioral);

for all : nmos use entity work.mos (nmos_behavioral);

constant vdd : std_logic := ‘1’;

constant gnd : std_logic := ‘0’;

signal s1 : std_logic;

begin

n1 : nmos port map (s1, vdd, a);

n2 : nmos port map (y, s1, b);

p1 : pmos port map (y, gnd, a);

p2 : pmos port map (y, gnd, b);

end and_switch;

Verilog Description

module and2gate (y, a, b);

input a, b;

output y;

supply1 vdd;

supply0 gnd;

nmos (s1, vdd, a);

nmos (y, s1, b);

pmos (y, gnd, a);

pmos (y, gnd, b);

endmodule

LISTING 5.5 HDL Code of a Two-Input OR Gate: VHDL and Verilog

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity or2gate is

port (y : out std_logic; a, b : in std_logic);

end or2gate;

architecture or_switch of or2gate is

component nmos

port (O1 : out std_logic; I1, I2 : in std_logic);

end component;

component pmos

port (O1 : out std_logic; I1, I2 : in std_logic);

end component;

for all : pmos use entity work.mos (pmos_behavioral);

for all : nmos use entity work.mos (nmos_behavioral);

constant vdd : std_logic := ‘1’;

constant gnd : std_logic := ‘0’;

signal s1 : std_logic;

begin

n1 : nmos port map (y, vdd, a);

n2 : nmos port map (y, vdd, b);

p1 : pmos port map (y, s1, a);

p2 : pmos port map (s1, gnd, b);

end or_switch;

Verilog Description

module OR2gate (a, b, y);

input a, b;

output y;

supply1 vdd;

supply0 gnd;

nmos (y, vdd, a);

nmos (y, vdd, b);

pmos (y, s1, a);

pmos (s1, gnd, b);

endmodule

LISTING 5.6 HDL Code for a Two-Input NAND Gate: VHDL and Verilog

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity nand2gate is

port (y : out std_logic; a, b : in std_logic);

end nand2gate;

architecture nand_switch of nand2gate is

component nmos

port (O1 : out std_logic; I1, I2 : in std_logic);

end component;

component pmos

port (O1 : out std_logic; I1, I2 : in std_logic);

end component;

for all : pmos use entity work.mos (pmos_behavioral);

for all : nmos use entity work.mos (nmos_behavioral);

constant vdd : std_logic := ‘1’;

constant gnd : std_logic := ‘0’;

signal s1 : std_logic;

begin

n1 : nmos port map (s1, gnd, b);

n2 : nmos port map (y, s1, a);

p1 : pmos port map (y, vdd, a);

p2 : pmos port map (y, vdd, b);

end nand_switch;

Verilog Description

module NAND2gate (a, b, y);

input a, b;

output y;

supply1 vdd;

supply0 gnd;

nmos (s1, gnd, b);

nmos (y, s1, a);

pmos (y, vdd, a);

pmos (y, vdd, b);

endmodule

LISTING 5.7 HDL Code for a Two-Input NOR Gate: VHDL and Verilog

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity nor2gate is

port (y : out std_logic; a, b : in std_logic );

end nor2gate;

architecture nor_switch of nor2gate is

component nmos

port (O1 : out std_logic; I1, I2 : in std_logic);

end component;

component pmos

port (O1 : out std_logic; I1, I2 : in std_logic);

end component;

for all : pmos use entity work.mos (pmos_behavioral);

for all : nmos use entity work.mos (nmos_behavioral);

constant vdd : std_logic := ‘1’;

constant gnd : std_logic := ‘0’;

signal s1 : std_logic;

begin

n1 : nmos port map (y, gnd, a);

n2 : nmos port map (y, gnd, b);

p1 : pmos port map (s1, vdd, a);

p2 : pmos port map (y, s1, b);

end nor_switch;

Verilog Description

module nor2gate (a, b, y);

input a, b;

output y;

supply1 vdd;

supply0 gnd;

nmos (y, gnd, a);

nmos (y, gnd, b);

pmos (s1, vdd, a);

pmos (y, s1, b);

endmodule

LISTING 5.8 HDL Code for a Two-Input AND Gate with Strong Output:

VHDL and Verilog

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity and2Sgate is

port (y : out std_logic; a, b : in std_logic);

end and2Sgate;

architecture and_strong of and2Sgate is

component nmos

port (O1 : out std_logic; I1, I2 : in std_logic);

end component;

component pmos

port (O1 : out std_logic; I1, I2 : in std_logic);

end component;

for all : pmos use entity work.mos (pmos_behavioral);

for all : nmos use entity work.mos (nmos_behavioral);

constant vdd : std_logic := ‘1’;

constant gnd : std_logic := ‘0’;

signal s1, y1 : std_logic;

begin

-- NAND

n1 : nmos port map (s1, gnd, b);

n2 : nmos port map (y1, s1, a);

p1 : pmos port map (y1, vdd, a);

p2 : pmos port map (y1, vdd, b);

-- Invert

n3 : nmos port map (y, gnd, y1);

p3 : pmos port map (y, vdd, y1);

end and_strong;

Verilog Description

module and2Sgate (a, b, y);

input a, b;

output y;

supply1 vdd;

supply0 gnd;

//NAND

nmos (s1, gnd, a);

nmos (y1, s1, b);

pmos (y1, vdd, a);

pmos (y1, vdd, b);

//inverter

nmos (y, gnd, y1);

pmos (y, vdd, y1);

endmodule

LISTING 5.9 HDL Code of a Two-Input OR Gate with Strong Output: VHDL

and Verilog

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity OR2Sgate is

port (y : out std_logic; a, b : in std_logic);

end OR2Sgate;

architecture orgate_strong of OR2Sgate is

component nmos

port (O1 : out std_logic; I1, I2 : in std_logic);

end component;

component pmos

port (O1 : out std_logic; I1, I2 : in std_logic);

end component;

for all : pmos use entity work.mos (pmos_behavioral);

for all : nmos use entity work.mos (nmos_behavioral);

constant vdd : std_logic := ‘1’;

constant gnd : std_logic := ‘0’;

signal s1, y1 : std_logic;

begin

--NOR

n1 : nmos port map (y1, gnd, a);

n2 : nmos port map (y1, gnd, b);

p1 : pmos port map (s1, vdd, a);

p2 : pmos port map (y1, s1, b);

--Invert

n3 : nmos port map (y, gnd, y1);

p3 : pmos port map (y, vdd, y1);

end orgate_strong;

Verilog Description

module OR2Sgate (a, b, y);

input a, b;

output y;

supply1 vdd;

supply0 gnd;

//NOR

nmos (y1, gnd, a);

nmos (y1, gnd, b);

pmos (s1, vdd, a);

pmos (y1, s1, b);

//inverter

nmos (y, gnd, y1);

pmos (y, vdd, y1);

endmodule

LISTING 5.10 HDL Code for a Three-Input NAND Gate: VHDL and Verilog

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity nand3gate is

port (y : out std_logic; a, b, c : in std_logic);

end nand3gate;

architecture nand3_switch of nand3gate is

component nmos

port (O1 : out std_logic; I1, I2 : in std_logic);

end component;

component pmos

port (O1 : out std_logic; I1, I2 : in std_logic);

end component;

for all : pmos use entity work.mos (pmos_behavioral);

for all : nmos use entity work.mos (nmos_behavioral);

constant vdd : std_logic := ‘1’;

constant gnd : std_logic := ‘0’;

signal s1, s2 : std_logic;

begin

n1 : nmos port map (s1, gnd, a);

n2 : nmos port map (s2, s1, b);

n3 : nmos port map (y, s2, c);

p1 : pmos port map (y, vdd, a);

p2 : pmos port map (y, vdd, b);

p3 : pmos port map (y, vdd, c);

end nand3_switch;

Verilog Description

module nand3gate (a, b, c, y);

input a, b, c;

output y;

supply1 vdd;

supply0 gnd;

nmos (s1, gnd, a);

nmos (s2, s1, b);

nmos (y, s2, c);

pmos (y, vdd, a);

pmos (y, vdd, b);

pmos (y, vdd, c);

endmodule

LISTING 5.11 HDL Code for a Three-Input NOR Gate: VHDL and Verilog

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity nor3gate is

port (y : out std_logic; a, b, c : in std_logic);

end nor3gate;

architecture nor3_switch of nor3gate is

component nmos

port (O1 : out std_logic; I1, I2 : in std_logic);

end component;

component pmos

port (O1 : out std_logic; I1, I2 : in std_logic);

end component;

for all : pmos use entity work.mos (pmos_behavioral);

for all : nmos use entity work.mos (nmos_behavioral);

constant vdd : std_logic := ‘1’;

constant gnd : std_logic := ‘0’;

signal s1, s2 : std_logic;

begin

n1 : nmos port map (y, gnd, a);

n2 : nmos port map (y, gnd, b);

n3 : nmos port map (y, gnd, c);

p1: pmos port map (s1, vdd, a);

p2: pmos port map (s2, s1, b);

p3: pmos port map (y, s2, c);

end nor3_switch;

Verilog Description

module nor3gate (a, b, c, y);

input a, b, c;

output y;

supply1 vdd;

supply0 gnd;

nmos (y, gnd, a);

nmos (y, gnd, b);

nmos (y, gnd, c);

pmos (s1, vdd, a);

pmos (s2, s1, b);

pmos (y, s2, c);

endmodule

LISTING 5.12 HDL Code for the Logic y = abc+de : VHDL and Verilog

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity simple_logic is

port (y : out std_logic; a, b, c, d : in std_logic);

end simple_logic;

architecture ABC of simple_logic is

component nmos

port (O1 : out std_logic; I1, I2 : in std_logic);

end component;

component pmos

port (O1 : out std_logic; I1, I2 : in std_logic);

end component;

for all : pmos use entity work.mos (pmos_behavioral);

for all : nmos use entity work.mos (nmos_behavioral);

constant vdd : std_logic := ‘1’;

constant gnd : std_logic := ‘0’;

signal s1, s2, s3 : std_logic;

begin

n1 : nmos port map (s1, gnd, c);

n2 : nmos port map (s2, s1, b);

n3 : nmos port map (y, s2, a);

n4 : nmos port map (y, gnd, d);

p1 : pmos port map (y, s3, a);

p2 : pmos port map (y, s3, b);

p3 : pmos port map (y, s3, c);

p4 : pmos port map (s3, vdd, d);

end ABC;

Verilog Description

module simple_logic (a, b, c, d, e, y);

input a, b, c, d, e;

output y;

supply1 vdd;

supply0 gnd;

nmos (s1, gnd, c);

nmos (s2, s1, b);

nmos (y, s2, a);

nmos (s3, gnd, e);

nmos (y, s3, d);

pmos (y, s4, a);

pmos (y, s4, b);

pmos (y, s4, c);

pmos (s4, vdd, d);

pmos (s4, vdd, e);

endmodule

LISTING 5.13 HDL Code for a XNOR Gate: VHDL and Verilog

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity XOR_XNOR is

port (y : out std_logic; a, b : in std_logic);

end XOR_XNOR;

architecture XNORgate of XOR_XNOR is

component nmos

port (O1 : out std_logic; I1, I2 : in std_logic);

end component;

component pmos

port (O1 : out std_logic; I1, I2 : in std_logic);

end component;

for all : pmos use entity work.mos (pmos_behavioral

for all : nmos use entity work.mos (nmos_behavioral);

constant vdd : std_logic := ‘1’;

constant gnd : std_logic := ‘0’;

signal abar, bbar, s1, s2, s3 : std_logic;

begin

-- Invert a and b. If the complement of a and b

--are available, then there is no need for the

--following two pair (nmos and pmos) switches.

--

p1 : pmos port map (abar, vdd, a);

n1 : nmos port map (abar, gnd, a);

p2 : pmos port map (bbar, vdd, b);

n2 : nmos port map (bbar, gnd, b);

--Write the pull-down combination

n3 : nmos port map (s1, gnd, a);

n4 : nmos port map (y, s1, bbar);

n5 : nmos port map (s2, gnd, abar);

n6 : nmos port map (y, s2, b);

--Write the pull-up combination

p3 : pmos port map (y, s3, a);

p4 : pmos port map (y, s3, bbar);

p5 : pmos port map (s3, vdd, abar);

p6 : pmos port map (s3, vdd, b);

end XNORgate;

Verilog Description

module XOR_XNOR (a, b, y);

input a, b;

output y;

supply1 vdd;

supply0 gnd;

/* Invert a and b. If the complement of a and b

are available, then there is no need for the

following two pair (nmos and pmos) switches */

pmos (abar, vdd, a);

nmos (abar, gnd, a);

pmos (bbar, vdd, b);

nmos (bbar, gnd, b);

// Write the pull-down combination

nmos (s1, gnd, a);

nmos (y, s1, bbar);

nmos (s2, gnd, abar);

nmos (y, s2, b);

// Write the pull-up combination

pmos (y, s3, a);

pmos (y, s3, bbar);

pmos (s3, vdd, abar);

pmos (s3, vdd, b);

endmodule

LISTING 5.14 HDL Code for a XNOR Gate: VHDL and Verilog

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity XNOR_degrade is

Port (y : out std_logic; a, b : in std_logic);

end XNOR_degrade;

architecture XNORgate of XNOR_degrade is

component nmos

port (O1 : out std_logic; I1, I2 : in std_logic);

end component;

component pmos

port (O1 : out std_logic; I1, I2 : in std_logic);

end component;

for all : pmos use entity work.mos (pmos_behavioral

for all : nmos use entity work.mos (nmos_behavioral);

constant vdd : std_logic := ‘1’;

constant gnd : std_logic := ‘0’;

signal s0 : std_logic;

begin

p1 : pmos port map (s0, vdd, b);

p2 : pmos port map (y, s0, a);

n1 : nmos port map (s0, gnd, b);

n2 : nmos port map (y, b, a);

end XNORgate;

Verilog Description

module gate (a, b, y);

input a, b;

output y;

supply1 vdd;

supply0 gnd;

pmos p1 (s0, vdd, b);

pmos p2 (y, s0, a);

nmos n1 (s0, gnd, b);

nmos n2 (y, b, a);

endmodule

LISTING 5.15 HDL Code for a 2x1 Multiplexer with Active High Enable and

Complement Output: VHDL and Verilog

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity mux2x1 is

port (a, b, Sel, E : in std_logic;

ybar : out std_logic);

end mux2x1;

architecture mux2x1switch of mux2x1 is

component nmos

port (O1 : out std_logic; I1, I2 : in std_logic);

end component;

component pmos

port (O1 : out std_logic; I1, I2 : in std_logic);

end component;

for all : pmos use entity work.mos (pmos_behavioral);

for all : nmos use entity work.mos (nmos_behavioral);

constant vdd : std_logic := ‘1’;

constant gnd : std_logic := ‘0’;

signal Selbar, s0, s1, s2, s3 : std_logic;

begin

--Invert signal Sel. If the complement of Sel is

--available then, there is no need for

--the following pair of transistors.

v1 : pmos port map (Selbar, vdd, Sel);

--All instantiation statements should be

--labeled

v2 : nmos port map (Selbar, gnd, Sel);

--Write the pull-down combination

n1 : nmos port map (s0, gnd, E);

n2 : nmos port map (s1, s0, Sel);

n3 : nmos port map (ybar, s1, a);

n4 : nmos port map (s2, s0, Selbar);

n5 : nmos port map (ybar, s2, b);

--Write the pull-up combination

p1 : pmos port map (ybar, vdd, E);

p2 : pmos port map (ybar, s3, Sel);

p3 : pmos port map (ybar, s3, a);

p4 : pmos port map (s3, vdd, Selbar);

p5 : pmos port map (s3, vdd, b);

end mux2x1switch;

Verilog Description

module mux2x1 (a, b, Sel, E, ybar);

input a, b, Sel, E;

output ybar;

supply1 vdd;

supply0 gnd;

/* Invert signal Sel. If the complement of Sel

is available then, there is no need for

the following pair of transistors */

pmos (Selbar, vdd, Sel);

nmos (Selbar, gnd, Sel);

//Write the pull-down combination

nmos n1 (s0, gnd, E);

nmos n2 (s1, s0, Sel);

nmos n3 (ybar, s1, a);

nmos n4 (s2, s0, Selbar);

nmos n5 (ybar, s2, b);

//Write the pull-up combination

pmos p1 (ybar, vdd, E);

pmos p2 (ybar, s3, Sel);

pmos p3 (ybar, s3, a);

pmos p4 (s3, vdd, Selbar);

pmos p5 (s3, vdd, b);

endmodule

LISTING 5.16 HDL Code for an SR-Latch: VHDL and Verilog

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity SR_Latch is

port (S, R : in std_logic; Q, Qbar : inout std_logic);

end SR_Latch;

architecture SR of SR_Latch is

component nmos

port (O1 : inout std_logic; I1, I2 : in std_logic);

--port O1 is selected here to be inout to match

--its use in the latch circuit

end component;

component pmos

port (O1 : inout std_logic; I1, I2 : in std_logic);

--port O1 is selected here to be inout

--to match its use in the latch circuit

end component;

for all : pmos use entity work.mos (pmos_behavioral);

for all : nmos use entity work.mos (nmos_behavioral);

--In this example only, the mode of Output port O1

--in the entity “mos” should

--be declared as inout instead of out.

constant vdd : std_logic := ‘1’;

constant gnd : std_logic := ‘0’;

signal s0, s1, s2 : std_logic;

begin

n1 : nmos port map (Qbar, gnd, S);

n2 : nmos port map (Qbar, gnd, Q);

p1 : pmos port map (s0, vdd, Q);

p2 : pmos port map (Qbar, s0, S);

n3 : nmos port map (Q, gnd, Qbar);

n4 : nmos port map (Q, gnd, R);

p3 : pmos port map (s1, vdd, R);

p4 : pmos port map (Q, s1, Qbar);

end SR;

Verilog Description

module SR_latch (S, R, Q, Qbar);

input S, R;

output Q, Qbar;

supply1 vdd;

supply0 gnd;

nmos n1 (Qbar, gnd, S);

nmos n2 (Qbar, gnd, Q);

pmos p1 (s0, vdd, Q);

pmos p2 (Qbar, s0, S);

nmos n3 (Q, gnd, Qbar);

nmos n4 (Q, gnd, R);

pmos p3 (s1, vdd, R);

pmos p4 (Q, s1, Qbar);

endmodule

LISTING 5.17 VHDL Code for the CMOS Switch in Figure 5.19.

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity CMOS is

port (output : out std_logic;

input, gn, gp : in std_logic);

end CMOS;

architecture macro of CMOS is

--All switches presented here do not include any

--time parameters, such as

--rise time and fall times. They only mimic the

--logical functions of their

--Verilog counterparts.

component nmos

port (O1 : out std_logic; I1, I2 : in std_logic);

end component;

component pmos

port (O1 : out std_logic; I1, I2 : in std_logic);

end component;

for all : pmos use entity work.mos (pmos_behavioral);

for all : nmos use entity work.mos (nmos_behavioral);

begin

n1 : nmos port map (output, input, gn);

p1 : pmos port map (output, input, gp);

end macro;

LISTING 5.18 Verilog Code for a D-Latch Using NMOS and PMOS Switches

module D_latch (D, E, Q, Qbar);

input D, E;

output Q, Qbar;

supply1 vdd;

supply0 gnd;

pmos (Ebar, vdd, E);

nmos (Ebar, gnd, E);

nmos n1 (s0, gnd, D);

nmos n2 (Qbar, s0, E);

nmos n3 (s1, gnd, Q);

nmos n4 (Qbar, s1, Ebar);

pmos p1 (Qbar, s2, D);

pmos p2 (Qbar, s2, E);

pmos p3 (s2, vdd, Q);

pmos p4 (s2, vdd, Ebar);

endmodule

LISTING 5.19 HDL Code for a D-Latch Using CMOS Switches: VHDL and

Verilog

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity D_Latch is

port (D, E : in std_logic; Q, Qbar : inout std_logic);

-- Referring to Figure 5.22, signal Q is

--input and output and has multiple

--sources (the inverter and the CMOS switches,

--so Q has to be declared as

--inout. All other ports are also adjusted in

--the following components to be inout.

end D_Latch;

architecture DlatchCmos of D_Latch is

component CMOS

port (output : out std_logic;

input, gn, gp : in std_logic);

end component;

component invert

port (y : out std_logic; a : in std_logic );

end component;

for all : CMOS use entity work.CMOS (macro);

for all : invert use entity work.

inverter( Invert_switch);

signal Ebar, s1 : std_logic;

begin

c1 : cmos port map (Q, D, E, Ebar);

c2 : cmos port map (Q, s1, Ebar, E);

inv1 : invert port map (Ebar, E);

inv2 : invert port map (Qbar, Q);

inv3 : invert port map (s1, Qbar);

end DlatchCmos;

Verilog Description

module D_latch (D, E, Q, Qbar);

input D, E;

output Q, Qbar;

cmos (Q, D, E, Ebar);

cmos (Q, s1, Ebar, E);

invert inv1 (Ebar, E);

invert inv2 (Qbar, Q);

invert inv3 (s1, Qbar);

endmodule

module invert (y, a);

input a;

output y;

supply1 vdd;

supply0 gnd;

pmos p1 (y, vdd, a);

nmos n1 (y, gnd, a);

endmodule

LISTING 5.20 HDL Code for the Logic y = abc+de : VHDL and Verilog

module simple_logic (a, b, c, d, e, y);

input a, b, c, d, e;

output y;

supply1 vdd;

supply0 gnd;

tranif1 (s1, gnd, c);

tranif1 (s2, s1, b);

tranif1 (y, s2, a);

tranif1 (s3, gnd, e);

tranif1 (y, s3, d);

tranif0 (y, s4, a);

tranif0 (y, s4, b);

tranif0 (y, s4, c);

tranif0 (s4, vdd, d);

tranif0 (s4, vdd, e);

endmodule

Chapter 6

LISTING 6.1 HDL Description of a Full Adder Using procedure and task:

VHDL and Verilog

VHDL Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity full_add is

port (x, y, cin : in std_logic; sum, cout : out std_logic);

end full_add;

architecture two_halfs of full_add is

-- The full adder is built from two half adders

procedure Haddr (sh, ch : out std_logic;

ah, bh : in std_logic) is

--This procedure describes a half adder

begin

sh := ah xor bh;

ch := ah and bh;

end Haddr;

begin

addfull : process (x, y, cin)

variable sum1, c1, c2, tem1, tem2 : std_logic;

begin

Haddr (sum1, c1, y, cin);

Haddr (tem1, c2, sum1, x);

--The above two statements are calls to

--the procedure Haddr

sum <= tem1;

cout <= tem2;

end process;

end two_halfs;

Verilog Description

module Full_add (x, y, cin, sum, cout);

//The full adder is built from two half adders

input x, y, cin;

output sum, cout;

reg sum, sum1, c1, c2, cout;

always @ (x, y, cin)

begin

Haddr (sum1, c1, y, cin);

Haddr (sum, c2, sum1, x);

//The above two statements are calls to the task Haddr.

cout = c1 | c2;

end

task Haddr;

//This task describes the half adder

output sh, ch;

input ah, bh;

begin

sh = ah ^ bh;

ch = ah & bh;

end

endtask

endmodule

LISTING 6.2 HDL Description of an N-Bit Ripple-Carry Adder Using procedure

and task: VHDL and Verilog

VHDL N-Bit Ripple-Carry Adder Using procedure

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity adder_ripple is

generic (N : integer := 3);

port (x, y : in std_logic_vector (N downto 0);

cin : in std_logic;

sum : out std_logic_vector (N downto 0);

cout : out std_logic);

end adder_ripple;

architecture adder of adder_ripple is

procedure Faddr (sf, cof : out std_logic;

af, bf, cinf : in std_logic) is

--This procedure describes a full adder

begin

sf := af xor bf xor cinf;

cof := (af and bf) or (af and cinf) or (bf and cinf);

end Faddr;

begin

addrpl : process (x, y, cin)

variable c1, c2, tem1, tem2 : std_logic;

variable cint : std_logic_vector (N+1 downto 0);

variable sum1 : std_logic_vector (N downto 0);

begin

cint(0) := cin;

for i in 0 to N loop

Faddr (sum1(i), cint(i+1), x(i), y(i), cint(i));

--The above statement is a call to the procedure Faddr

end loop;

sum <= sum1;

cout <= cint(N+1);

end process;

end adder;

Verilog N-Bit Ripple-Carry Adder Using task

module adder_ripple (x, y, cin, sum, cout);

parameter N = 3;

input [N:0] x, y;

input cin;

output [N:0] sum;

output cout;

reg [N+1:0] cint;

reg [N:0] sum;

reg cout;

integer i;

always @ (x, y, cin)

begin

cint[0] = cin;

for (i = 0; i <= N; i = i + 1)

begin

Faddr (sum[i], cint[i+1], x[i], y[i], cint[i]);

//The above statement is a call to task Faddr

end

cout = cint[N+1];

end

task Faddr;

//The task describes a full adder

output sf, cof;

input af, bf, cinf;

begin

sf = af ^ bf ^ cinf;

cof = (af & bf) | (af & cinf) | (bf & cinf);

end

endtask

endmodule

LISTING 6.3 HDL Code for Converting an Unsigned Binary to an Integer Using

procedure and task: VHDL and Verilog

VHDL: Converting an Unsigned Binary to an Integer Using procedure

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all; --This Library is for

-- type “unsigned”

entity Bin_Int is

generic (N : natural := 3);

port (X_bin : unsigned (N downto 0);

Y_int : out natural; Z : out std_logic);

--Y is always positive

end Bin_Int;

architecture convert of Bin_Int is

procedure bti (bin : in unsigned; int : out natural;

signal Z : out std_logic) is

-- the procedure bti is to change binary to integer

-- Flag Z is chosen to be a signal rather than a variable

-- Since the binary vector is always positive,

-- use type natural for the output of the procedure.

variable result : natural;

begin

result := 0;

for i in bin’Range loop

--bin’Range represents the range of the unsigned vector bin

--Range is a predefined attribute

if bin(i) = ‘1’ then

esult := result + 2**i;

end if;

end loop;

int := result;

if (result = 0) then

Z <= ‘1’;

else

Z <= ‘0’;

end if;

end bti;

begin

process (X_bin)

variable tem : natural;

begin

bti (X_bin, tem, Z);

Y_int <= tem;

end process;

end convert;

Verilog: Converting an Unsigned Binary to an Integer Using task

module Bin_Int (X_bin, Y_int, Z);

parameter N = 3;

input [N:0] X_bin;

output integer Y_int;

output Z;

reg Z;

always @ (X_bin)

begin

bti (Y_int, Z, N, X_bin);

end

task bti;

parameter P = N;

output integer int;

output Z;

input N;

input [P:0] bin;

integer i, result;

begin

int = 0;

//change binary to integer

for (i = 0; i <= P; i = i + 1)

begin

if (bin[i] == 1)

int = int + 2**i;

end

if (int == 0)

Z = 1’b1;

else

Z = 1’b0;

end

endtask

endmodule

LISTING 6.4 HDL Code for Converting a Fraction Binary to Real Using procedure

and task: VHDL and Verilog

VHDL: Converting a Fraction Binary to Real Using procedure

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity Bin_real is

generic (N : integer := 3);

port (X_bin : in std_logic_vector (0 to N); Y : out real);

end Bin_real;

architecture Bin_real of Bin_real is

procedure binfloat (a : in std_logic_vector (0 to 3);

float : out real) is

--This procedure converts fraction expressed

--in fixed-point binary to real

variable tem,j : real;

begin

tem := 0.0;

j := 1.0;

for i in 0 to N loop

j := j/ 2.0;

if (a(i) = ‘1’) then

tem := tem + j;

end if;

end loop;

float := tem;

end binfloat;

begin

rel : process (X_bin)

variable temp : real;

begin

binfloat (X_bin, temp);

Y <= temp;

end process rel;

end Bin_real;

Verilog: Converting a Fraction Binary to Real Using task

module Bin_real (X_bin);

parameter N = 3;

input [N:0] X_bin;

real Z;

always @ (X_bin)

begin

binfloat (X_bin, Z);

end

task binfloat;

parameter P = N;

input [0:P] a;

output real float;

integer i;

begin

float = 0.0;

for (i = 0; i <= P; i = i + 1)

begin

if (a[i] == 1)

float = float + 1.0 / 2**(i+1);

// The above statement multiplies each bit by its weight.

//

end

end

endtask

endmodule

LISTING 6.5 HDL Code for Converting an Unsigned Integer to Binary Using

procedure and task: VHDL and Verilog

VHDL: Converting an Unsigned Integer to Binary Using procedure

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity Int_Bin is

generic (N : integer := 3);

port (X_bin : out std_logic_vector (N downto 0);

Y_int : in integer;

flag_even : out std_logic);

end Int_Bin;

architecture convert of Int_Bin is

procedure itb (bin : out std_logic_vector;

signal flag : out std_logic;

N : in integer; int : inout integer) is

-- The procedure itb is to convert the integer to binary

-- The dimension of bin does not have to be specified

-- at the above declaration statement; the procedure

-- can determine the dimension of bin later in its body.

begin

if (int MOD 2 = 0) then

--The above statement checks int to see if it is even.

flag <= ‘1’;

else

flag <= ‘0’;

end if;

for i in 0 to N loop

if (int MOD 2 = 1) then

bin (i) := ‘1’;

else

bin (i) := ‘0’;

end if;

-- perform integer division by 2

int := int/2;

end loop;

end itb;

begin

process (Y_int)

variable tem : std_logic_vector (N downto 0);

variable tem_int : integer ;

begin

tem_int := Y_int;

itb (tem, flag_even, N, tem_int);

X_bin <= tem;

end process;

end convert;

Verilog: Converting an Unsigned Integer to Binary Using task

module Int_Bin (X_bin, flag_even, Y_int );

/*In general Verilog, in contrast to VHDL, does not

strictly differentiate between integers and binaries;

for example, if bin is declared as a binary of width 4,

bin = bin/2 can be written, and the Verilog, but not

VHDL, performs this division as if bin is integer.

In the following, the corresponding VHDL program in

Listing 6.5a is just translated to practice with

the command task */

parameter N = 3;

output [N:0] X_bin;

output flag_even;

input [N:0] Y_int;

reg [N:0] X_bin;

reg flag_even;

always @ (Y_int)

begin

itb (Y_int, N, X_bin, flag_even);

end

task itb;

parameter P = N;

input integer int;

input N;

output [P:0] bin;

output flag;

integer j;

begin

if (int %2 == 0)

//The above statement checks int to see if it is even.

flag = 1’b1;

else

flag = 1’b0;

for (j = 0; j <= P; j = j + 1)

begin

if (int %2 == 1)

bin[j] = 1;

else

bin[j] = 0;

int = int/2;

end

end

endtask

endmodule

LISTING 6.6 HDL Code for Converting a Signed Binary to an Integer Using

procedure and task: VHDL and Verilog.

VHDL: Converting a Signed Binary to an Integer Using procedure

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

entity signed_btoIn is

generic (N : integer := 3);

port (X_bin : in signed (N downto 0); Y_int : out integer;

even_parity : out std_logic );

end signed_btoIn;

architecture convert of signed_btoIn is

procedure sbti (binsg : in signed; M : in integer;

int : out integer; signal even : out std_logic) is

--The procedure sbti is to change signed binary to integer

-- and also to find whether the parity of the binary

--is odd or even.

--The dimension of “sbin” does not have to be specified

--at the declaration statement; it can be declared later

--in the body of the procedure.

variable result, parity : integer;

begin

result := 0;

for i in 0 to M loop

if binsg(i) = ‘1’ then

result := result + 2**i;

parity := parity + 1;

end if;

end loop;

if (binsg(M) = ‘1’) then

result := result - 2**(M+1);

end if;

int := result;

if (parity mod 2 = 1) then

even <= ‘0’;

else

even <= ‘1’;

end if;

end sbti;

begin

process (X_bin)

variable tem : integer;

begin

sbti (X_bin, N, tem, even_parity);

Y_int <= tem;

end process;

end convert;

Verilog: Converting a Signed Binary to an Integer Using task

module signed_btoIn(X_bin, Y_int, even_parity);

/*In general, Verilog (in contrast to VHDL) does not

strictly differentiate between integers and binaries;

for example if bin is declared as binary of width 4,

write bin = bin/2, and the Verilog (but not VHDL) will

perform this division. In the following, just translate

the corresponding VHDL counterpart program. */

parameter N = 3;

input signed [N:0] X_bin;

output integer Y_int;

output even_parity;

reg even_parity;

always @ (X_bin)

begin

sbti (Y_int, even_parity, N, X_bin);

end

task sbti;

parameter P = N;

output integer int;

output even;

input N;

input [P:0] bin;

integer i;

reg parity;

begin

int = 0;

parity = 0;

//change binary to integer

for (i = 0; i <= P; i = i + 1)

begin

if (bin[i] == 1)

begin

int = int + 2**i;

parity = parity + 1;

end

end

if ((parity % 2) == 1)

even = 0;

else

even = 1;

if (bin [P] == 1)

int = int - 2**(P+1);

end

endtask

endmodule

LISTING 6.7 VHDL Code for Converting an Integer to a Signed Binary Using

procedure

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use ieee.numeric_std.all;

entity signed_IntToBin is

generic (N : integer := 3);

port (X_bin : out signed (N downto 0); Y_int : in integer);

end signed_IntToBin;

architecture convert of signed_IntToBin is

procedure sitb (sbin : out signed; M, int : in integer) is

-- The procedure sitb is to convert integer into signed

-- binary. The dimension of “sbin” does not have to be

-- specified at the declaration statement; it can be

--declared later in the body of the procedure.

variable temp_int : integer;

variable flag : std_logic;

variable bin : signed (M downto 0);

begin

if (int < 0) then

temp_int := - int;

--if flag = 1, the number is negative

else

temp_int := int;

end if;

for i in 0 to M loop

if (temp_int MOD 2 = 1) then

bin (i) := ‘1’;

else

bin (i) := ‘0’;

end if;

--integer division by 2

temp_int := temp_int/2;

end loop;

if (flag = ‘1’) then

sbin := - bin;

else sbin := bin;

end if;

end sitb;

begin

process (Y_int)

variable tem : signed (N downto 0);

begin

sitb(tem, N, Y_int);

X_bin <= tem;

end process;

end convert;

LISTING 6.8 HDL Code for Signed-Vector Multiplication Using procedure

and task: VHDL and Verilog

VHDL: Signed-Vector Multiplication Using procedure

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use ieee.numeric_std.all;

entity Vector_Booth is

generic (N : integer := 3);

port (a0, a1, a2, b0, b1, b2 : in signed (N downto 0);

d : out signed (3*N downto 0));

end Vector_Booth;

architecture multiply of Vector_Booth is

procedure booth (X, Y : in signed (3 downto 0);

Z : out signed (7 downto 0)) is

--Booth algorithm here is restricted to 4x4 bits.

--It can be adjusted to multiply any NxN bits.

variable temp : signed (1 downto 0);

variable sum : signed (7 downto 0);

variable E1 : unsigned (0 downto 0);

variable Y1 : signed (3 downto 0);

begin

sum := “00000000”; E1 := “0”;

for i in 0 to 3 loop

temp := X(i) & E1(0);

Y1 := -Y;

case temp is

when “10” => sum (7 downto 4) :=

sum (7 downto 4) + Y1;

when “01” => sum (7 downto 4) :=

sum (7 downto 4) + Y;

when others => null;

end case;

sum := sum srl 1;

sum (7) := sum(6);

E1(0) := x(i);

end loop;

if (y = “1000”) then

--If Y = 1000; then according to the code,

--Y1 = 1000 (-8 not 8 because Y1 is 4 bits only).

--The statement sum = -sum adjusts the answer.

sum := -sum;

end if;

Z := sum;

end booth;

procedure sitb (sbin : out signed; M, int : in integer) is

-- The procedure sitb is to convert integer into signed

-- binary. The dimension of “sbin” does not have to be

-- specified at the declaration statement; it can be

--declared later in the body of the procedure.

variable temp_int : integer;

variable flag : std_logic;

variable bin : signed (M downto 0);

begin

if (int < 0) then

temp_int := -int;

flag := ‘1’;

else

temp_int := int;

end if;

for i in 0 to M loop

if (temp_int MOD 2 = 1) then

bin (i) := ‘1’;

else

bin (i) := ‘0’;

end if;

temp_int := temp_int/2;

end loop;

if (flag = ‘1’) then

sbin := -bin;

else

sbin := bin;

end if;

end sitb;

procedure sbti (binsg : in signed; M : in integer;

int : out integer) is

-- The procedure sbti is to change signed binary to

-- integer.No need to specify the dimension of “sbin”

-- at the declaration statement; it can be declared

-- later in the body of the procedure.

variable result : integer;

begin

result := 0;

for i in 0 to M loop

if binsg(i) = ‘1’ then

result := result + 2**i;

end if;

end loop;

if (binsg(M) = ‘1’) then

result := result - 2**(M+1);

end if;

int := result;

end sbti;

begin

process (a0, b0, a1, b1, a2, b2)

variable tem0, tem1, tem2 : signed ((2*N + 1) downto 0);

variable d_temp : signed (3*N downto 0);

variable temi0, temi1, temi2, temtotal : integer;

begin

--Find the partial products a0b0, a1b1, a2b2

booth (a0, b0, tem0);

booth (a1, b1, tem1);

booth (a2, b2, tem2);

-- Change the partial products to integers

sbti (tem0, (2*N+1), temi0);

sbti (tem1, (2*N+1), temi1);

sbti (tem2, (2*N+1), temi2);

-- Find the total integer sum of partial products

temtotal := temi0 + temi1 + temi2;

-- Change the integer to binary

sitb (d_temp, 3*N, temtotal);

d <= d_temp;

end process;

end multiply;

Verilog: Signed-Vector Multiplication Using task

module Vector_Booth (a0, a1, a2, b0, b1, b2, d);

parameter N = 3;

input signed [N:0] a0, a1, a2, b0, b1, b2;

output signed [3*N : 0] d;

reg signed [2*N+1 : 0] tem0, tem1, tem2;

reg signed [3*N : 0] d;

always @ (a0, b0, a1, b1, a2, b2)

begin

booth (a0, b0, tem0);

//booth is a task to multiply a0 x b0 = tem0

booth (a1, b1, tem1);

booth (a2, b2, tem2);

d = tem0 + tem1 + tem2;

end

task booth;

input signed [3:0] X, Y;

output signed [7:0] Z;

reg signed [7:0] Z;

reg [1:0] temp;

integer i;

reg E1;

reg [3:0] Y1;

begin

Z = 8’d0;

E1 = 1’d0;

for (i = 0; i < 4; i = i + 1)

begin

temp = {X[i], E1}; //This is catenation

Y1 = -Y; //Y1 is the 2’ complement of Y

case (temp)

2’d2 : Z [7:4] = Z [7:4] + Y1;

2’d1 : Z [7:4] = Z [7:4] + Y;

default : begin end

endcase

Z = Z >> 1; /*This is a logical shift of one position to

the right*/

Z[7] = Z[6];

/*The above two statements perform arithmetic shift

where the sign of the number is preserved after

the shift.*/

E1 = X[i];

end

if (Y == 4’b1000) Z = -Z;

/* If Y = 1000, then Y1 = 1000 (should be 8 not -8).

This error is because Y1 is 4 bits only.

The statement Z = -Z adjusts the value of Z. */

end

endtask

endmodule

LISTING 6.9 HDL Code for Signed 3x3-Bit Multiplication Using Combinational

Array: VHDL and Verilog

VHDL

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity boothArrayProc is

port ( start: in std_logic;

x : in std_logic_vector (2 downto 0);

y: in std_logic_vector (4 downto 0);

p:out std_logic_vector(4 downto 0));

--Input y is 3-bit plus two additional sign extension

-- bits.

end boothArrayProc;

architecture boothar of boothArrayProc is

procedure COMPR (n1 : in std_logic_vector;

i : in integer range 0 to 3; c1, c2: out std_logic) is

begin

c1 := n1(i+1) xor n1(i);

c2 := n1(i+1) and not n1(i);

end procedure COMPR;

procedure Fulladdr (a, b,c : in std_logic;

sum, Carryout : out std_logic) is

begin

Sum := (not a and not b and c) or

(not a and b and not c) or

(a and not b and not c) or

(a and b and c);

Carryout := (a and b) or (a and c) or (b and c);

end procedure Fulladdr;

procedure Fullsub (a, b,c : in std_logic; Diff,

Borrow : out std_logic) is

begin

Diff := (not a and not b and c) or

(not a and b and not c) or

(a and not b and not c) or

(a and b and c);

Borrow := ((not a) and c) or ((not a) and b) or (b and c);

end procedure Fullsub;

begin

B1: process (start,x,y)

variable s1, s2: std_logic;

variable T : std_logic_vector(3 downto 0);

variable f, E, z: std_logic_vector(4 downto 0):= “00000”;

variable carry_temp: std_logic_vector(5 downto 0);

variable i,j,layer, j1 :integer range 0 to 6 ;

begin

if (start = ‘1’) then

f := “00000”; E := “00000”;z := “00000”;

carry_temp := “000000”;

T := (x & ‘0’);-- T now represents all the inputs to

--cells COMP including the initial

--zero on the first COMP cell.

for layer in 0 to 2 loop

carry_temp(0) := ‘0’;

COMPR (T,layer,s1,s2);

if (s1 = ‘0’) then

for j in 0 to (4-layer) loop

z(j + layer) := f(j + layer);

end loop;

elsif (s2 = ‘0’) then

for j in 0 to (4-layer) loop

Fulladdr (f(j+layer), y(j), carry_temp(j),

z(j+ layer), carry_temp(j+1));

end loop;

else

for j in 0 to (4-layer) loop

Fullsub (f(j+layer), y(j),

carry_temp(j), z(j+layer), carry_temp(j+1));

end loop;

end if;

for j1 in 0 to (4-layer) loop

f(j1 + layer) := z(j1 + layer);end loop;

end loop;

end if;

p <= z;

end process B1;

end boothar;

Verilog

module bootharrayTask(start, x, y, p);

input start;

input [2:0] x;

input [4:0]y;

output [4:0] p;

/*input y is 3-bit with additional 2 bits as sign extension*/

reg [4:0] f,E,p;

reg [3:0] T;

integer layer,i, j,j1;

reg [5:0] carry_temp;

reg s1, s2;

always @ (start,x,y)

begin

if (start == 1’b1)

begin

T = {x,1’b0};/* T now represents all the inputs to

cells COMP including the initial

zero on the first COMP cell.*/

f = 5’d0; E = 5’d0; p = 5’d0; carry_temp = 6’d0;

for (layer = 0; layer <= 2; layer = layer + 1)

begin

carry_temp [0] = 1’b0;

COMPR (T,layer,s1,s2);

if (s1 == 1’b0)

begin

for (j = 0; j <= (4-layer); j = j+1)

begin

p[j + layer] = f[j + layer];

end

end

else if (s2 == 1’b0)

begin

for (j = 0; j <= (4-layer); j = j+1)

begin

Fulladdr (f[j+layer], y[j], carry_temp[j],

p[j+ layer], carry_temp[j+1]);

end

end

else

begin

for (j = 0; j <= (4-layer); j = j+1)

begin

Fullsub (f[j+layer], y[j], carry_temp[j],

p[j+ layer], carry_temp[j+1]);

end

end

for (j1 = 0; j1 <= (4-layer); j1 = j1+1)

begin

f[j1 + layer] = p[j1 + layer];

end

end

end

end

task COMPR;

input [3:0]n1;

input integer i;

output c1, c2;

begin

c1 = n1[i+1] ^ n1[i];

c2 = n1[i+1] & ( ~ n1[i]);

end

endtask

task Fulladdr;

input a, b,c;

output sum, Carryout;

reg sum, Carryout;

begin

sum = (~a & ~b & c) |

(~a & b & ~ c) |

(a & ~ b & ~ c) |

(a & b & c);

Carryout = (a & b) | (a & c) | (b & c);

end

endtask

task Fullsub;

input a, b,c;

output Diff, Borrow;

begin

Diff = (~a & ~b & c) |

(~a & b & ~ c) |

(a & ~ b & ~ c) |

(a & b & c);

Borrow = ((~ a) & c) | ((~ a) & b) | (b & c);

end

endtask

endmodule

LISTING 6.10 HDL Description for Enzyme Activity Using procedure and

task: VHDL and Verilog

VHDL: Enzyme Activity Using procedure

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use ieee.std_logic_arith.all;

-- A description of enzyme-substrate binding mechanism.

-- S= (Vmax *S)/(S + M) where S is the substrate

-- concentration, M is the dissociation constant, and Vmax

-- is the maximum rate of reaction. In this example,

-- Vmax = 1. The inputs are S and M in binary (fraction);

-- the output v is in Q4 format. This means that v is

-- always less than one, with the binary point placed to

-- the left of the most significant bit. For example,

-- if v = 1010, the decimal equivalent is .5 + .125 =

-- 0.625. To calculate v, convert S and M to real fraction,

-- find the real value of (S/(S + M)), convert this real

-- value to Q4 by multiplying it with 2**4 = 16, and

-- convert the integer to binary.

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use ieee.std_logic_arith.all;

entity enzyme_beh is

port (S : in std_logic_vector (0 to 3);

M : in std_logic_vector(0 to 3);

v : out real ; intg_bin : out integer;

start: in bit);

end enzyme_beh;

architecture enzyme of enzyme_beh is

procedure flt (a : in std_logic_vector (0 to 3);

float : out real) is

--This procedure converts fraction expressed

--in fixed-point binary (Q4) to real

variable tem,j : real;

begin

tem := 0.0;

j := 1.0;

for i in 0 to 3 loop

j := j/ 2.0;

if (a(i) = ‘1’) then

tem := tem + j;

end if;

end loop;

float := tem;

end flt;

procedure rltointg (a1 : in real ; Bin1 : out integer ) is

--This procedure converts real to integer

--The procedure does not round off

variable tem,j : real := 0.0;

variable tmp : integer := 0;

begin

while (tmp <= 256) loop

--Assume the maximum integer value is 256; if not,

--adjust accordingly

if (a1 > j) then

tmp := tmp + 1;

j := j + 1.0;

else

exit;

end if;

end loop;

Bin1 := tmp;

end rltointg;

begin

P1 : process(S, M, start)

variable temp1, temp11, vmax : real;

variable temp2: integer := 0;

begin

if (start = ‘1’) then

vmax := 1.0;

flt( S,temp1);

flt( M,temp11);

temp1 := Vmax * ( temp1/(temp1 + temp11));

v <= temp1;

temp1 := temp1 *16.0;

rltointg (temp1, temp2);

intg_bin <= temp2;

end if;

end process P1;

end enzyme;

Verilog: Enzyme Activity Using task

/* A description of enzyme-substrate binding mechanism.

S= (Vmax *S)/(S + M), where S is the substrate

concentration, M is the dissociation constant,

and Vmax is the maximum rate of reaction. In this

example, Vmax =1. The inputs are S and M in binary

(integer);

the output v is in Q4 format. This means that v is

always less than 1, with the binary point placed to the

left of the most significant bit. For example, if

v = 1010, the

decimal equivalent is .5 + .125 = 0.625. To calculate V,

find the real value of (S /(S + M)), convert this real

value to Q4 by multiplying it with 2**4 =16, and convert

the integer to binary. */

module enzyme_beh (S, M, V);

input [3:0] S, M;

output [3:0] V;

integer vmax;

reg [3:0] V;

real vr;

always @ (S, M)

begin

vmax = 1;

vr = vmax * (1.0 * S) / (S * 1.0 + M * 1.0);

vr = vr * 2**4;

rti (vr, V);

end

task rti;

/* This task can be replaced by just one statement, v1= r.

Verilog, in contrast to VHDL, can handle different

types of the assignment statement. Verilog finds the

equivalent integer value v1 for the real r. The task has

been designed here only to match the same steps done in

VHDL. */

input real r;

output [3:0] v1;

real temp;

begin

temp = r;

v1 = 4’b0000;

while (temp >= 0.5)

begin

v1 = v1 + 1;

temp = r - 1.0 * v1;

end

end

endtask

endmodule

LISTING 6.11 Example of a VHDL Function

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity Func_exm is

port (a1, b1 : in std_logic; d1 : out std_logic);

end Func_exm;

architecture Behavioral of Func_exm is

function exp (a, b : in std_logic) return std_logic is

variable d : std_logic;

begin

d := a xor b;

return d;

end function exp;

begin

process (a1, b1)

begin

d1 <= exp (a1, b1);

--The above statement is a function call

end process;

end Behavioral;

LISTING 6.12 Verilog Function That Calculates exp = a XOR b

module Func_exm (a1, b1, d1);

input a1, b1;

output d1;

reg d1;

always @ (a1, b1)

begin

/*The following statement calls the function exp

and stores the output in d1.*/

d1 = exp (a1, b1);

end

function exp ;

input a, b;

begin

exp = a ^ b;

end

endfunction

endmodule

LISTING 6.13 HDL Function to Find the Greater of Two Signed Numbers:

VHDL and Verilog

VHDL Function to Find the Greater of Two Signed Numbers

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use ieee.numeric_std.all;

entity greater_2 is

port (x, y : in signed (3 downto 0);

z : out signed (3 downto 0));

end greater_2;

architecture greater_2 of greater_2 is

function grt (a, b : signed (3 downto 0)) return signed is

-- The above statement declares a function by the name grt.

-- The inputs are 4-bit signed numbers.

variable temp : signed (3 downto 0);

begin

if (a >= b) then

temp := a;

else

temp := b;

end if;

return temp;

end grt;

begin

process (x, y)

begin

z <= grt (x, y); --This is a function call.

end process;

end greater_2;

Verilog Function to Find the Greater of Two Signed Numbers

module greater_2 (x, y, z);

input signed [3:0] x;

input signed [3:0] y;

output signed [3:0] z;

reg signed [3:0] z;

always @ (x, y)

begin

z = grt (x, y); //This is a function call.

end

function [3:0] grt;

/*The above statement declares a function by the name grt;

grt is also the output of the function*/

input signed [3:0] a, b;

/*The above statement declares two inputs to the function;

both are 4-bit signed numbers.*/

begin

if (a >= b)

grt = a;

else

grt = b;

end

endfunction

endmodule

LISTING 6.14 HDL Code for y = , 0 < x < 1—VHDL and Verilog

VHDL Floating Sum Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity segma is

port (x : in std_logic_vector (0 to 3);

y: out std_logic_vector (7 downto 0));

end segma;

architecture segm_beh of segma is

procedure flt (a : in std_logic_vector (0 to 3);

float : out real) is

--This procedure converts fraction expressed

-- in fixed-point binary to real

variable tem,j : real;

begin

tem := 0.0;

j := 1.0;

for i in 0 to 3 loop

j := j/ 2.0;

if (a(i) = ‘1’) then

tem := tem + j;

end if;

end loop;

float := tem;

end flt;

procedure rltointg (a1 : in real ; Bin1 : out integer ) is

--This procedure converts real to integer

--The procedure does not round off

variable tem,j : real := 0.0;

variable tmp : integer := 0;

begin

while (tmp <= 256) loop

--Assume the maximum integer value is 256;

--if not, adjust accordingly

if (a1 > j) then

tmp := tmp + 1;

j := j + 1.0;

else

exit;

end if;

end loop;

Bin1 := tmp;

end rltointg;

procedure itb (bin : out std_logic_vector;

N : in integer; int : in integer) is

--This procedure is to convert integer to binary

variable temp_int : integer := int;

begin

for i in 0 to N loop

if (temp_int MOD 2 = 1) then

bin(i) := ‘1’;

else bin(i) := ‘0’;

end if;

temp_int := temp_int/2;

end loop;

end itb;

function exp (a : in std_logic_vector (0 to 3))

return std_logic_vector is

variable z1 : real;

variable intgr : integer;

variable tem : std_logic_vector (7 downto 0);

begin

flt (a, z1);

z1 := 1.0 - z1 + z1 * z1 - z1 * z1 * z1;

z1 := z1 * 256.0; -- 256 is for 8 bits

rltointg (z1, intgr);

itb (tem, 7, intgr);

return tem;

end exp;

begin

sg1 : process (x)

variable tem1 : std_logic_vector (7 downto 0);

variable tem2: integer;

begin

tem1 := exp(x);

y <= tem1;

end process sg1;

end segm_beh;

Verilog Floating Sum Description

module segma1(x,y);

input [0:3] x;

// x is a fraction in Q4 format, 0 < x < 1.

output [7:0] y;

reg [7:0] y;

always @ (x)

begin

y = exp (x);

end

function [7:0] rti;

//This function convers real to integer with rounding off

input real r;

begin

rti = r;

end

endfunction

function [7:0] exp;

input [0:3] a;

real z1;

integer i;

begin

z1 = 0.0;

for (i = 0; i <= 3; i = i + 1)

begin

if (a[i] == 1)

z1 = z1 + 1.0 / 2**(i+1);

/*The above statement multiplies

each bit by its weight */

end

z1 = 1.0 - z1 + z1**2 - z1**3;

z1 = z1 * 2**8;

exp = rti(z1);

end

endfunction

endmodule

LISTING 6.15 Verilog Code for Conversion to Single-Precision IEEE 754

Floating-Point Representation

module IEEEflt( start, IEEE_flt);

input start;

output [0:31] IEEE_flt;//This is the 32-bit IEEE

reg [7:0] j;

reg [0:31] IEEE_flt;

real x = 24.0; /* x is the iput number in decimal

format; assume all numbers entered here are

positive; see Exercise at the end of the Chapter */

real Mant;integer i;

always @ (start,x,j)

begin

if (start == 1’b1)

begin

j =8’d127;//The exponent is in excess 127

if (x != 0.0)

begin

while (x >= 2.0)

begin

x = x/ 2.0;

j = j +1;

end

while (x < 1.0)

begin

x = x* 2.0;

j = j -1;

end

end

end

Mant = x-1.0;

for (i =0 ; i <= 22; i = i + 1)

begin

IEEE_flt [9+i] = 1’b0;

Mant = Mant * 2.0;

if ( Mant >= 1.0)

begin

IEEE_flt [9+i] = 1’b1;

Mant = Mant-1.0;

end

end

IEEE_flt [0] = 1’b0;

IEEE_flt [1:8] = j[7:0];

end

endmodule

Chapter 7

LISTING 7.1 An Example of a VHDL Package

package conversions is

type wkdays is (mon, tue, wed, th, fr);

procedure convert (a : in bit; b : out integer);

function incr (b : std_logic_vector) return std_logic_vector;

end conversions;

package body of conversions is written as:

procedure convert (a : in bit; b : out integer) is

begin

.......

end convert;

function incr (b : std_logic_vector) return std_logic_vector is

begin

...

end incr;

end conversions;

LISTING 7.2 An Example of a VHDL Package

library ieee;

use ieee.std_logic_1164.all;

package codes is

type op is (add, mul, divide, none);

end;

use work.codes;

entity ALUS2 is

port (a, b : in std_logic_vector (3 downto 0);

cin : in std_logic; opc : in op;

z : out std_logic_vector (7 downto 0);

cout : buffer std_logic);

end ALUS2;

LISTING 7.3 HDL Code for Finding the Greatest Element of an Array: VHDL

and Verilog

VHDL: Finding the Greatest Element of an Array

library IEEE;

use IEEE.STD_LOGIC_1164.all;

--Build a package for an array

package array_pkg is

constant N : integer := 4;

--N+1 is the number of elements in the array.

constant M : integer := 3;

--M+1 is the number of bits of each element

--of the array.

subtype wordN is std_logic_vector (M downto 0);

type strng is array (N downto 0) of wordN;

end array_pkg;

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use work.array_pkg.all;

-- The above statement makes the package array_pkg visible

-- in this module.

entity array1 is

generic (N : integer :=4; M : integer := 3);

--N + 1 is the number of elements in the array; M = 1 is

-- the number of bits of each element.

Port (a : inout strng;

z : out std_logic_vector (M downto 0));

end array1;

architecture max of array1 is

begin

com: process (a)

variable grtst : wordN;

begin

--enter the data of the array.

a <= (“0110”, “0111”, “0010”, “0011”, “0001”);

grtst := “0000”;

lop1 : for i in 0 to N loop

if (grtst <= a(i)) then

grtst := a(i);

report “ grtst is less or equal than a”;

-- use the above report statement if you want to

-- monitor the progress of the program.

else

report “grtst is greater than a”;

-- Use the above report statement to monitor the

-- progress of the program

end if;

end loop lop1;

z <= grtst;

end process com;

end max;

Verilog: Finding the Greatest Element of an Array

module array1 (start, grtst);

parameter N = 4;

parameter M = 3;

input start;

output [3:0] grtst;

reg [M:0] a[0:N];

/*The above statement is declaring an array of N + 1 elements;

each element is M bits. */

reg [3:0] grtst;

integer i;

always @ (start)

begin

a[0] = 4’b0110;

a[1] = 4’b0111;

a[2] = 4’b0010;

a[3] = 4’b0011;

a[4] = 4’b0001;

grtst = 4’b0000;

for (i = 0; i <= N; i= i +1)

begin

if (grtst <= a[i])

begin

grtst = a[i];

$display (“ grtst is less or equal than a”);

// use the above statement to monitor the program

end

else

$display (“ grtst is greater than a”);

// use the above statement to monitor the program

end

end

endmodule

LISTING 7.4 Multiplication of Two Signed N-Element Vectors: VHDL and

Verilog

VHDL: Multiplication of Two Signed N-Element Vectors

library IEEE;

use IEEE.STD_LOGIC_1164.all;

use ieee.numeric_std.all;

package booth_pkg is

constant N : integer := 4;

--N + 1 is the number of elements in the array.

constant M: integer := 3;

--M + 1 is the number of bits of each element

--of the array.

subtype wordN is signed (M downto 0);

type strng is array (N downto 0) of wordN;

procedure booth (X, Y : in signed (3 downto 0);

Z : out signed (7 downto 0));

end booth_pkg;

package body booth_pkg is

procedure booth (X, Y : in signed (3 downto 0);

Z : out signed (7 downto 0)) is

--Booth algorithm here is restricted to 4x4 bits.

--It can be adjusted to multiply any NxN bits.

variable temp : signed (1 downto 0);

variable sum : signed (7 downto 0);

variable E1 : unsigned (0 downto 0);

variable Y1 : signed (3 downto 0);

begin

sum := “00000000”; E1 := “0”;

for i in 0 to 3 loop

temp := X(i) & E1(0);

Y1 := -Y;

case temp is

when “10” => sum (7 downto 4) :=

sum (7 downto 4) + Y1;

when “01” => sum (7 downto 4) :=

sum (7 downto 4) + Y;

when others => null;

end case;

sum := sum srl 1;

sum(7) := sum(6);

E1(0) := x(i);

end loop;

if (y = “1000”) then

sum := -sum;

--If Y = 1000; then Y1 is calculated as 1000;

--that is -8, not 8 as expected. This is because Y1 is

--4 bits only. The statement sum = -sum corrects

--this error.

end if;

Z := sum;

end booth;

end booth_pkg;

-- We start writing the multiplication algorithm using

-- the package booth_pkg

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use ieee.numeric_std.all;

use work.booth_pkg.all;

entity vecor_multply is

generic (N : integer := 4; M : integer := 3);

--N + 1 is the number of elements in the array; M + 1 is

-- the number of bits of each element.

Port (a, b : in strng; d : out signed (3*N downto 0));

end vecor_multply;

architecture multply of vecor_multply is

begin

process (a, b)

variable temp : signed (7 downto 0);

variable temp5 : signed (3*N downto 0) := “0000000000000”;

begin

for i in 0 to 4 loop

booth(a(i), b(i), temp);

--accumulate the partial products in the product temp5

temp5 := temp5 + temp;

end loop;

d <= temp5;

end process;

end multply;

Verilog: Multiplication of Two Signed N-Element Vectors

module vecor_multply (start, d);

parameter N = 4;

parameter M = 3;

input start;

output signed [3*N:0] d;

reg signed [M:0] a[0:N];

reg signed [M:0] b[0:N];

reg signed [3*N:0] d;

reg signed [3*N:0] temp;

integer i;

always @ (start)

begin

a[0] = 4’b1100;

a[1] = 4’b0000;

a[2] = 4’b1001;

a[3] = 4’b0011;

a[4] = 4’b1111;

b[0] = 4’b1010;

b[1] = 4’b0011;

b[2] = 4’b0111;

b[3] = 4’b1000;

b[4] = 4’b1000;

d = 0;

for (i = 0; i <= N; i = i + 1)

begin

booth (a[i], b[i], temp);

d = d + temp;

end

end

task booth;

input signed [3:0] X, Y;

output signed [7:0] Z;

reg signed [7:0] Z;

reg [1:0] temp;

integer i;

reg E1;

reg [3:0] Y1;

begin

Z = 8’d0;

E1 = 1’d0;

for (i = 0; i < 4; i = i + 1)

begin

temp = {X[i], E1}; //This is catenation

Y1 = -Y; //Y1 is the 2’complement of Y

case (temp)

2’d2 : Z[7:4] = Z[7:4] + Y1;

2’d1 : Z[7:4] = Z[7:4] + Y;

default : begin end

endcase

Z = Z >> 1;

/*The above statement is a logical shift of

one position to the right*/

Z[7] = Z[6];

/*The above two statements perform arithmetic shift where the

sign of the number is preserved after the shift. */

E1 = X[i];

end

if (Y == 4’b1000)

/* If Y = 1000, then Y1 = 1000 (should be 8 not -8).

This error is because Y1 is 4 bits only.

The statement Z = -Z adjusts the value of Z. */

Z = -Z;

End

endtask

endmodule

LISTING 7.5 HDL Code for a Two-Dimensional Array

VHDL Two-Dimensional Array

library IEEE;

use IEEE.STD_LOGIC_1164.all;

--Build a package to declare the array

package twodm_array is

constant N : integer := 4;

-- N+1 is the number of elements in the array.

-- this is [N+1,N+1] matrix with N+1 rows and N+1 columns

subtype wordg is integer;

type strng1 is array (N downto 0) of wordg;

type strng2 is array (N downto 0) of strng1;

end twodm_array;

--use the package to describe a two-dimensional array

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use work.twodm_array.all;

-- The above statement instantiates the package twodm_array

entity two_array is

Port (N, M : integer; z : out integer);

end two_array;

architecture Behavioral of two_array is

begin

com : process (N, M)

variable t : integer;

constant y : strng2 := ((7, 6, 5, 4, 3), (6, 7, 8, 9, 10),

(30, 31, 32, 33, 34), (40, 41, 42, 43, 44),

(50, 51, 52, 53, 54));

begin

t := y (N)(M);

--Look at the simulation output to identify the elements of the

--array

z <= t;

end process com;

end Behavioral;

Verilog Two-Dimensional Array

module twodmarrays(start,N,M,Z );

parameter N1 = 4;

parameter M1 = 4;

input start;

input [2:0] N,M;

output integer Z;

reg [5:0] Y [0:4] [0:4];

//The following statements generate the array as

//[25,24,23,22,21], [20,19,18,17,16], [15,14,13,12,11],

// [10,9,8,7,6], [5,4,3,2,1] with Y[0][0]=1,Y[0][1]=2

integer i,j,K = 0;

always @ ((start == 1’b1),N,M)

begin

K = 0;

for (i = 0; i <= N1; i= i +1)

begin

for (j = 0; j <= M1; j= j +1)

begin

K= K +1;

Y[i][j]= K;

end

end

Z = Y[N][M];

end

endmodule

LISTING 7.6 VHDL Description: Addition of Two [5×5] Matrices

-- First, write a package to declare a two-dimensional

--array with five elements

library IEEE;

use IEEE.STD_LOGIC_1164.all;

package twodm_array is

constant N : integer := 4;

-- N+1 is the number of elements in the array.

-- This is an NxN matrix with N rows and N columns.

subtype wordg is integer;

type strng1 is array (N downto 0) of wordg;

type strng2 is array (N downto 0) of strng1;

end twodm_array;

--Second, write the code for addition

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use work.twodm_array.all;

entity matrices is

Port (x, y : strng2; z : out strng2);

--strng2 type is 5x5 matrix

end matrices;

architecture sum of matrices is

begin

com : process (x, y)

variable t : integer := 0;

begin

for i in 0 to 4 loop

for j in 0 to 4 loop

t := x(i)(j) + y(i)(j);

z(i)(j) <= t;

end loop;

end loop;

end process com;

end sum;

Verilog Description: Addition of Two [5×5] Matrices

module sumMatrices(start,N,M,Z );

//The program generates two matrices

//X,Y (two dimensional arrays)

//and add them up and store the sum

//in matrix sum

parameter N1 = 4;

parameter M1 = 4;

input start;

input [2:0] N,M;

output integer Z;

reg [6:0] sum [0:4] [0:4];

reg [5:0] Y [0:4] [0:4];

reg [5:0] X [0:4] [0:4];

//initial values in the array are generated by the loop

//statements as:

//[25,24,23,22,21], [20,19,18,17,16], [15,14,13,12,11],

//[10,9,8,7,6], [5,4,3,2,1}with Y[0][0]=1,Y[0][1]=2

integer i,j,K = 0;

always @ ((start == 1’b1),N,M)

begin

K =0;

for (i = 0; i <= N1; i= i +1)

begin

for (j = 0; j <= M1; j= j +1)

begin

K= K +1;

Y[i][j]= K;

X[i][j] = K + 3;

end

end

Z = Y[N][M];

for (i = 0; i <= N1; i= i +1)

begin

for (j = 0; j <= M1; j= j +1)

begin

sum[i][j]= X[i][j] + Y[i][j];

end

end

end

endmodule

LISTING 7.7 HDL Description of an ALU: VHDL and Verilog

VHDL ALU Description

--Here the code for a package for user-defined

--type and function is written.

library ieee;

use ieee.std_logic_1164.all;

use IEEE.STD_LOGIC_1164.ALL,IEEE.NUMERIC_STD.ALL;

package codes_Arithm is

type op is (add, mul, divide, none);

-- type op is for the operation codes for the ALU.

--The operations are: addition, multiplication,

--division, and no operation

function TO_UNSIGN (b : integer) return unsigned;

end;

package body codes_Arithm is

function TO_UNSIGN (b : integer) return unsigned is

--The function converts integers to unsigned. This function

--can be omitted if it is included in the vendor’s package;

--the vendor’s package, if available, should be attached.

variable temp : integer;

variable bin : unsigned (5 downto 0);

begin

temp := b;

for j in 0 to 5 loop

if (temp MOD 2 = 1) then

bin (j) := ‘1’;

else bin (j) := ‘0’;

end if;

temp := temp/2;

end loop;

return bin;

end TO_UNSIGN;

end codes_Arithm;

--Now we write the code for the ALU

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use ieee.numeric_std.all;

use work.codes_arithm.all;

--The above use statement is to set the user-

--defined package “codes_arithm.all” visible to this

-- module.

entity ALU_mixed is

port (a, b : in unsigned (2 downto 0);

cin : in std_logic;

opc : in op; z : out unsigned (5 downto 0));

--opc is of type “op”; type op is defined in the

--user-defined package “codes_arithm”

end ALU_mixed;

architecture ALU_mixed of ALU_mixed is

signal c0, c1 : std_logic;

signal p, g : unsigned (2 downto 0);

signal temp1 : unsigned (5 downto 0);

begin

--The following is a data flow-description of a 3-bit

-- lookahead adder. The sum is stored in the three least

-- significant bits of temp1.

adder.

-- The carry out is stored in temp1(3).

g(0) <= a(0) and b(0);

g(1) <= a(1) and b(1);

g(2) <= a(2) and b(2);

p(0) <= a(0) or b(0);

p(1) <= a(1) or b(1);

p(2) <= a(2) or b(2);

c0 <= g(0) or (p(0) and cin);

c1 <= g(1) or (p(1) and g(0)) or

(p(1) and p(0) and cin);

temp1(3) <= g(2) or (p(2) and g(1)) or (p(2) and p(1)

and g(0)) or (p(2) and p(1) and p(0) and cin);

--temp1(3) is the final carryout of the adders

temp1(0) <= (p(0) xor g(0)) xor cin;

temp1(1) <= (p(1) xor g(1)) xor c0;

temp1(2) <= (p(2) xor g(2)) xor c1;

temp1 (5 downto 4) <= «00»;

process (a, b, cin, opc, temp1)

--The following is a behavioral description for the

-- multiplication and division functions of the ALU.

variable temp : unsigned (5 downto 0);

variable a1, a2, a3 : integer;

begin

a1 := TO_INTEGER (a);

a2 := TO_INTEGER (b);

--The predefined function «TO_INTEGER»

--converts unsigned to integer.

--The function is a member of the VHDL package

-- IEEE.numeric.

case opc is

when mul =>

a3 := a1 * a2;

temp := TO_UNSIGN(a3);

--The function «TO_UNSIGN» is a user-defined function

--written in the user-defined package «codes_arithm.»

when divide =>

a3 := a1 / a2;

temp := TO_UNSIGN(a3);

when add =>

temp := temp1;

when none =>

null;

end case;

z <= temp;

end process;

end ALU_mixed;

Verilog ALU Description

module ALU_mixed (a, b, cin, opc, z);

parameter add = 0;

parameter mul = 1;

parameter divide = 2;

parameter nop = 3;

input [2:0] a, b;

input cin;

input [1:0] opc;

output [5:0] z;

reg [5:0] z;

wire [5:0] temp1;

wire [2:0] g, p;

wire c0, c1;

// The following is data-flow description

// for 3-bit lookahead adder

assign g[0] = a[0] & b[0];

assign g[1] = a[1] & b[1];

assign g[2] = a[2] & b[2];

assign p[0] = a[0] | b[0];

assign p[1] = a[1] | b[1];

assign p[2] = a[2] | b[2];

assign c0 = g[0] | (p[0] & cin);

assign c1 = g[1] | (p[1] & g[0]) | (p[1] & p[0] & cin);

assign temp1[3] = g[2] | (p[2] & g[1]) | (p[2] & p[1]

& g[0]) | (p[2] & p[1] & p[0] & cin);

// temp1[3] is the final carryout of the adders

assign temp1[0] = (p[0] ^ g[0]) ^ cin;

assign temp1[1] = (p[1] ^ g[1]) ^ c0;

assign temp1[2] = (p[2] ^ g[2]) ^ c1;

assign temp1[5:4] = 2’b00;

//The following is behavioral description

always @ (a, b, cin, opc, temp1)

begin

case (opc)

mul : z = a * b;

add : z = temp1;

divide : z = a / b;

nop : z = z;

endcase

end

endmodule

LISTING 7.8 HDL Description of 16×8 SRAM: VHDL and Verilog

VHDL 16×8 SRAM Description

library IEEE;

use IEEE.STD_LOGIC_1164.all;

--Build a package for an array

package array_pkg is

constant N : integer := 15;

--N+1 is the number of elements in the array.

constant M : integer := 7;

--M+1 is the number of bits of each element

--of the array.

subtype wordN is std_logic_vector (M downto 0);

type strng is array (N downto 0) of wordN;

end array_pkg;

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use ieee.numeric_std.all;

use work.array_pkg.all;

entity memory16x8 is

generic (N : integer := 15; M : integer := 7);

--N+1 is the number of words in the memory; M+1 is the

--number of bits of each word.

Port (Memory : inout strng; CS : in std_logic;

ABUS: in unsigned (3 downto 0);

Data_in : in std_logic_vector (7 downto 0);

R_WRbar : in std_logic;

Data_out : out std_logic_vector (7 downto 0));

end memory16x8;

architecture SRAM of memory16x8 is

begin

com : process (CS, ABUS, Data_in, R_WRbar)

variable A : integer range 0 to 15;

begin

if (CS = ‘1’) then

A := TO_INTEGER (ABUS);

-- TO_INTEGER is a built-in function

if (R_WRbar = ‘0’) then

Memory (A) <= Data_in;

else

Data_out <= Memory(A);

end if;

else

Data_out <= “ZZZZZZZZ”;

--The above statement describes high impedance.

end if;

end process com;

end SRAM;

Verilog 16×8 SRAM Description

module memory16x8 (CS, ABUS, Data_in, R_WRbar, Data_out);

input CS, R_WRbar;

input [3:0] ABUS;

input [7:0] Data_in;

output [7:0] Data_out;

reg [7:0] Data_out;

reg [7:0] Memory [0:15];

always @ (CS, ABUS, Data_in, R_WRbar)

begin

if (CS == 1’b1)

begin

if (R_WRbar == 1’b0)

begin

Memory [ABUS] = Data_in;

end

else

Data_out = Memory [ABUS];

end

else

Data_out = 8’bZZZZZZZZ;

//The above statement describes high impedance

end

endmodule

LISTING 7.9 HDL Code for the State Machine in Figure 7.7: VHDL and

Verilog

VHDL State-Machine Description

library IEEE;

use IEEE.STD_LOGIC_1164.all;

--First we write a package that includes type “states.”

package types is

type op is (add, mul, divide, none);

type states is (state0, state1, state2, state3);

end;

-- Now we use the package to write the code for the

-- state machine.

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use work.types.all;

entity state_machine is

port (A, clk : in std_logic; pres_st : buffer states;

Z : out std_logic);

end state_machine;

architecture st_behavioral of state_machine is

begin

FM : process (clk, pres_st, A)

variable present : states := state0;

begin

if (clk = ‘1’ and clk’event) then

case pres_st is

when state0 =>

if A =’1’ then

present := state1;

Z <= ‘0’;

else

present := state0;

Z <= ‘1’;

end if;

when state1 =>

if A =’1’ then

present := state2;

Z <= ‘0’;

else

present := state3;

Z <= ‘0’;

end if;

when state2 =>

if A =’1’ then

present := state3;

Z <= ‘1’;

else

present := state0;

Z <= ‘0’;

end if;

when state3 =>

if A =’1’ then

present := state0;

Z <= ‘0’;

else

present := state2;

Z <= ‘0’;

end if;

end case;

pres_st <= present;

end if;

end process FM;

end st_behavioral;

Verilog State-Machine Description

`define state0 2’b00

`define state1 2’b01

`define state2 2’b10

`define state3 2’b11

// We could have declared these states as parameters.

// See Listing 7.7.

module state_machine (A, clk, pres_st, Z);

input A, clk;

output [1:0] pres_st;

output Z;

reg Z;

reg [1:0] present;

reg [1:0] pres_st;

initial

begin

pres_st = 2’b00;

end

always @ (posedge clk)

begin

case (pres_st)

`state0 :

begin

if (A == 1)

begin

present = `state1;

Z = 1’b0;

end

else

begin

present = `state0;

Z = 1’b1;

end

end

`state1 :

begin

if (A == 1)

begin

present = `state2;

Z = 1’b0;

end

else

begin

present = `state3;

Z = 1’b0;

end

end

`state2 :

begin

if (A == 1)

begin

present = `state3;

Z = 1’b1;

end

else

begin

present = `state0;

Z = 1’b0;

end

end

`state3 :

begin

if (A == 1)

begin

present = `state0;

Z = 1’b0;

end

else

begin

present = `state2;

Z = 1’b0;

end

end

endcase

pres_st = present;

end

endmodule

LISTING 7.10 HDL Code for the Basic-Computer Memory Program

(Table 7.4): VHDL and Verilog

VHDL Basic-Computer Memory Program

--Write the code for Package Comp_Pkg

library IEEE;

use IEEE.STD_LOGIC_1164.all;

use ieee.numeric_std.all;

package Comp_Pkg is

constant N: integer := 15;

--N+1 is the number of elements in the array.

constant M : integer := 7;

--M+1 is the number of bits of each element

--of the array.

subtype wordN is unsigned (M downto 0);

type strng is array (N downto 0) of wordN;

type states is (state0, state1, state2, state3);

end Comp_Pkg;

--Now write the code for the control unit

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

use ieee.numeric_std.all;

use work.Comp_Pkg. all;

entity computer_basic is

generic (N : integer := 15; M : integer := 7);

--N+1 is the number of words in the memory; M+1 is the

--number of bits of each word.

Port (Memory : inout strng;

PC : buffer unsigned (3 downto 0);

clk_master : std_logic;

ACH : buffer unsigned (7 downto 0);

ACL : buffer unsigned (7 downto 0);

Reset : buffer std_logic; ON_OFF : in std_logic);

end computer_basic;

architecture Behavioral_comp of computer_basic is

signal z : unsigned (0 downto 0);

signal clk : std_logic;

begin

z(0) <= ACL(6) xor ACL(5) xor ACL(4) xor

ACL(3) xor ACL(2) xor ACL(1) xor ACL(0);

--Z has to be in vector form to match ACL

clk <= clk_master and ON_OFF;

--The above two statements are data-flow description.

--The following is behavioral description.

cpu : process (Reset, PC, ACL, ACH, clk, Memory, z(0))

variable AR : unsigned (3 downto 0);

variable DR : unsigned (7 downto 0);

variable pres_st, next_st : states;

variable ARI : integer range 0 to 16;

variable IR : unsigned (2 downto 0);

variable PR : unsigned (15 downto 0);

begin

if rising_edge (clk) then

if Reset = ‘1’ then

pres_st := state0;

Reset <= ‘0’;

PC <= «0000»;

end if;

case pres_st is

when state0 =>

next_st := state1;

--This is fetch cycle

AR := PC;

when state1 =>

next_st := state2;

ARI := TO_INTEGER(AR);

--This is fetch cycle

DR := Memory (ARI);

when state2 =>

next_st := state3;

--This is fetch cycle

PC <= PC + 1;

IR := DR (7 downto 5);

AR := DR (3 downto 0);

when state3 =>

--This is execute cycle

case IR is

when «111» =>

--The op code is CLA

ACL <= «00000000»;

next_st := state0;

when «001» =>

--The op code is ADD

ARI := TO_INTEGER(AR);

DR := memory (ARI);

ACL <= ACL + DR;

next_st := state0;

when «010» =>

--The op code is MULT

ARI := TO_INTEGER(AR);

DR := memory (ARI);

PR := ACL * DR;

ACL <= PR (7 downto 0);

ACH <= PR (15 downto 8);

next_st := state0;

when «011» =>

--The op code is DIVID

ARI := TO_INTEGER(AR);

DR := memory (ARI);

ACL <= ACL / DR;

next_st := state0;

when «100» =>

--The op code is XOR

ARI := TO_INTEGER(AR);

DR := memory (ARI);

ACL <= ACL XOR DR;

next_st := state0;

when «110» =>

--The op code is NAND

ARI := TO_INTEGER(AR);

DR := memory (ARI);

ACL <= ACL NAND DR;

next_st := state0;

when «101» =>

--The op code is PRITY

ACL(7) <= z(0);

next_st := state0;

when «000» => null;

-- The op code is HALT

next_st := state3;

when others => null;

end case;

when others => null;

end case;

pres_st := next_st;

end if;

end process cpu;

end Behavioral_comp;

Verilog Basic-Computer Memory Program

module computer_basic (PC, clk_master, ACH, ACL,

Reset, ON_OFF);

parameter state0 = 2’b00;

parameter state1 = 2’b01;

parameter state2 = 2’b10;

parameter state3 = 2’b11;

output [3:0] PC;

input clk_master;

output Reset;

input ON_OFF;

output [7:0] ACH;

output [7:0] ACL;

reg [1:0] pres_st;

reg [1:0] next_st;

reg Reset;

reg [3:0] PC;

reg [3:0] AR;

reg [7:0] DR;

reg [2:0] IR;

reg [7:0] ACH;

reg [7:0] ACL;

reg [15:0] PR;

reg [7:0] Memory [0:15];

assign z = ACL[6] ^ ACL[5] ^ ACL[4]^

ACL[3]^ ACL[2]^ ACL[1]^ ACL[0];

/*The above statement can be written using the reduction

XOR as: assign z = ^ ACL[6:0];*/

//

assign clk = clk_master & ON_OFF;

always @ (Reset, PC, ACL, ACH, posedge(clk), z, pres_st)

begin

if (Reset == 1’b1)

begin

pres_st = state0;

Reset = 1’b0;

PC = 4’d0;

Memory [0] = 8’hE0; Memory [1] = 8’h29;

Memory [2] = 8’h8A; Memory [3] = 8’h4B;

Memory [4] = 8’h6C; Memory [5] = 8’h8D;

Memory [6] = 8’hCE; Memory [7] = 8’hA0;

Memory [8] = 8’h00; Memory [9] = 8’h0C;

Memory [10] = 8’h05; Memory [11] = 8’h04;

Memory [12] = 8’h09; Memory [13] = 8’h03;

Memory [14] = 8’h09;

Memory [15] = 8’h07;

end

case (pres_st)

state0 :

begin

next_st = state1;

AR = PC;

end

state1 :

//This is fetch cycle

begin

next_st = state2;

DR = Memory [AR];

end

state2 :

//This is fetch cycle

begin

next_st = state3;

PC = PC + 1;

IR = DR [7:5];

AR = DR [3:0];

end

state3 :

//This is execute cycle

begin

case (IR)

3’d7 :

//The op code is CLA

begin

ACL = 8’d0;

next_st = state0;

end

3’d1 :

//The op code is ADD

begin

DR = Memory [AR];

ACL = ACL + DR;

next_st = state0;

end

3’d2 :

//The op code is MULT

begin

DR = Memory [AR];

PR = ACL * DR;

ACL = PR [7:0];

ACH = PR [15:8];

next_st = state0;

end

3’d3 :

//The op code is DIVID

begin

DR = Memory [AR];

ACL = ACL / DR;

next_st = state0;

end

3’d4 :

//The op code is XOR

begin

DR = Memory [AR];

ACL = ACL ^ DR;

next_st = state0;

end

3’d6 :

//The op code is NAND

begin

DR = Memory [AR];

ACL = ~(ACL & DR);

next_st = state0;

end

3’d5 :

//The op code is PRITY

begin

ACL[7] = z;

next_st = state0;

end

3’d0 :

//The op code is HALT

begin

next_st = state3;

end

default :

begin

end

endcase

end

default :

begin

end

endcase

pres_st = next_st;

end

endmodule

Chapter 8

LISTING 8.1 VHDL Code for Reading and Processing a Text File Containing

Integers

library ieee;

use ieee.std_logic_1164.all;

use std.textio.all;

entity FREAD_INTG is

port (START : in std_logic;

z, z1, z2, z3 : out integer);

end FREAD_INTG;

architecture FILE_BEHAVIOR of FREAD_INTG is

begin

process (START)

-- declare the infile as a text file

file infile : text;

--declare the variable fstatus (or any other variable name)

--as of type file_open_status

variable fstatus : file_open_status;

variable count : integer;

--declare variable temp as of type line

variable temp : line;

begin

if (START = ‘1’) then

--open the file file_int.txt in read mode

file_open (fstatus, infile, “file_int.txt”, read_mode);

--Read the first line of the file and store the line in temp

readline (infile, temp);

-- temp now has the data: 12 -3 5

-- Read the first integer (12) from the

--line temp and store it in the integer variable count.

read (temp, count);

--count has the value of 12. Multiply by 2 and store in z

z <= 2 * count;

-- Read the second integer from the line temp and

-- store it in count

read (temp, count);

--count now has the value of -3

--Multiply by 5 and store in z1

z1 <= 5 * count;

-- read the third integer in line temp

--and store it in count.

read (temp, count);

--Multiply by 3 and store in z2

z2 <= 3 * count;

--Read the second line and store it in temp

readline (infile, temp);

--temp has only the second line

--Read the first integer of the second line

--and store it in count.

read (temp, count);

--Multiply by 4 and store in z3

z3 <= 4 * count;

--Close the infile

file_close (infile);

end if;

end process;

end FILE_BEHAVIOR;

LISTING 8.2 VHDL Code for Reading a Text File Containing Real Numbers

library ieee;

use ieee.std_logic_1164.all;

use std.textio.all;

entity FREAD_REAL is

port (START : in std_logic;

z, z1, z2, z3 : out real);

end FREAD_REAL;

architecture FILE_BEHAVIOR of FREAD_REAL is

begin

process (START)

file infile : text;

variable fstatus : file_open_status;

variable count : real;

--Variable count has to be of type real

variable temp : line;

begin

if (START = ‘1’) then

--Open the file

file_open (fstatus, infile,

“file_real.txt”, read_mode);

-- Read a line

readline (infile, temp);

--Read one number and store it in real variable count

read (temp, count);

--multiply by 2

z <= 2.0 * count;

--read another number

read (temp, count);

--multiply by 5

z1 <= 5.0 * count;

--read another number

read (temp, count);

--multiply by 3

z2 <= 3.0 * count;

--read another line

readline (infile, temp);

read (temp, count);

--multiply by 4

z3 <= 4.0 * count;

file_close (infile);

end if;

end process;

end FILE_BEHAVIOR;

LISTING 8.3 VHDL Code for Reading an ASCII File

use ieee.std_logic_1164.all;

use std.textio.all;

entity FREAD_character is

port (START : in std_logic;

z, z1, z2, z3 : out character);

end FREAD_character;

architecture FILE_BEHAVIOR of FREAD_character is

begin

process (START)

file infile : text;

variable fstatus : file_open_status;

variable count : character;

-- Variable count has to be of type character

variable temp : line;

begin

if(START = ‘1’) then

file_open (fstatus, infile, “file_chr.txt”, read_mode);

--read a line from the file

readline (infile, temp);

--read a character from the line into count.

--Count has to be of type character.

--

read (temp, count);

--store the character in z

z <= count;

read (temp, count);

z1 <= count;

read (temp, count);

z2 <= count;

readline (infile, temp);

read (temp, count);

z3 <= count;

file_close (infile);

end if;

end process;

end FILE_BEHAVIOR;

LISTING 8.4 VHDL Code for Writing Integers to a File

library ieee;

use ieee.std_logic_1164.all;

use std.textio.all;

entity FWRITE_INT is

port (START : in std_logic);

end FWRITE_INT;

architecture FILE_BEHAVIOR of FWRITE_INT is

begin

process (START)

file outfile : text;

variable fstatus : file_open_status;

--declare temp as of type line

variable temp : line;

variable z,z1,z2,z3 : integer := 6;

begin

if(START = ‘1’ ) then

z := 12; z1 := 23; z2 := -56; z3 := -45;

file_open (fstatus, outfile,

“Wfile_int.txt”, write_mode);

--The generated file “Wfile_int.txt” is in

--the same directory as this VHDL module

--Insert the title of the file Wfile_int.txt.

--Your simulator should support formatted text;

--if not, remove all formatted statements “ “.

write (temp, “This is an integer file”);

--Write the line temp into the file

writeline (outfile, temp);

--store the first integer in line temp

write (temp, z);

-- leave space between the integer numbers.

write (temp, “ “);

write (temp, z1);

-- leave another space between the integer numbers.

write (temp, “ “);

write (temp, z2);

write (temp, “ “);

writeline (outfile, temp);

--Insert the fourth integer value on a new line

write (temp, z3);

writeline (outfile, temp);

file_close(outfile);

end if;

end process;

end FILE_BEHAVIOR;

LISTING 8.5 VHDL Code for Writing a Package Containing a String of Five

Characters

library IEEE;

use IEEE.STD_LOGIC_1164.all;

package array_pkg is

constant N : integer := 4;

--N+1 is the number of elements in the array.

subtype wordChr is character;

type string_chr is array (N downto 0) of wordChr;

end array_pkg;

LISTING 8.6 VHDL Code for Reading a String of Characters into an Array

library ieee;

use ieee.std_logic_1164.all;

use std.textio.all;

--include the package with this module

use work.array_pkg.all;

entity FILE_CHARCTR is

port (START : in std_logic; z : out string_chr);

--string_char is included in the package array_pkg;

--z is a 5-character array

end FILE_CHARCTR;

architecture FILE_BEHAVIOR of FILE_CHARCTR is

begin

process (START)

file infile : text;

variable fstatus : file_open_status;

variable count : string_chr;

variable temp : line;

begin

file_open (fstatus, infile, “myfile1.txt”, read_mode);

readline (infile, temp);

read (temp, count);

--Variable count has been declared as an array of five

-- elements, each element is a single character.

--

z <= count;

file_close (infile);

end process;

end FILE_BEHAVIOR;

Listing 8.7 VHDL Code for Finding the Smallest ASCII Value

--The following package needs to be attached

--to the main module.

library IEEE;

use IEEE.STD_LOGIC_1164.all;

package array_pkg is

constant N : integer := 4;

--N+1 is the number of elements in the array.

subtype wordChr is character;

type string_chr is array (N downto 0) of wordChr;

end array_pkg;

library ieee;

use ieee.std_logic_1164.all;

use std.textio.all;

use work.array_pkg.all;

--Now start writing the code to find the smallest

entity SMALLEST_CHRCTR is

port (START : in std_logic; z : out string_chr);

end SMALLEST_CHRCTR;

architecture BEHAVIOR_SMALLEST of SMALLEST_CHRCTR is

begin

process (START)

file infile : text;

variable fstatus : file_open_status;

variable count, smallest :

string_chr := (‘z ‘, ‘z ‘, ‘z ‘, ‘z ‘, ‘z ‘);

-- The above statement assigns initial values (Z’s) to

-- count and smallest.

variable temp : line;

begin

file_open (fstatus, infile,

“f_smallest.txt”, read_mode);

while (count /= (‘E’, ‘N’, ‘D’, ‘ ‘, ‘ ‘)) loop

readline (infile, temp);

read (temp, count);

if (count < smallest) then

smallest := count;

end if;

end loop;

z <= smallest;

file_close (infile);

end process;

end BEHAVIOR_SMALLEST;

LISTING 8.8 VHDL Code for Finding the Integer Code for a Mnemonic Code

--The following package needs to be

--attached to the main module

library IEEE;

use IEEE.STD_LOGIC_1164.all;

package array_pkg is

constant N : integer := 4;

--N+1 is the number of elements in the array.

subtype wordChr is character;

type string_chr is array (N downto 0) of wordChr;

end array_pkg;

--Start writing the code to find the assigned integer value

library ieee;

use ieee.std_logic_1164.all;

use std.textio.all;

use work.array_pkg.all;

entity OPCODES is

port (assmbly_code : in string_chr; z : out string_chr;

z1 : out integer);

end OPCODES;

architecture BEHAVIOR of OPCODES is

begin

process (assmbly_code)

file infile : text;

variable fstatus : file_open_status;

variable temp : string_chr := (‘ ‘, ‘ ‘, ‘ ‘, ‘ ‘, ‘ ‘);

variable tem_bin : integer;

variable regstr : line;

begin

file_open (fstatus, infile, “cods.txt”, read_mode);

for i in 0 to 8 loop

-– while loop could have been used instead

-- of for loop. See Exercise 8.3.

readline (infile, regstr);

read (regstr, temp);

if (temp = assmbly_code) then

z <= temp;

read (regstr, tem_bin);

z1 <= tem_bin;

exit;

else if (i > 7)then

report (“ERROR: CODE COULD NOT BE FOUND”);

z <= (‘E’, ‘R’, ‘R’, ‘O’, ‘R’);

-- assign -1 to z1 if an error occurs

z1 <= -1;

end if;

end if;

end loop;

file_close(infile);

end process;

end BEHAVIOR;

LISTING 8.9 VHDL Assembler Code

--The following package needs to be attached

--to the main module

library IEEE;

use IEEE.STD_LOGIC_1164.all;

package array_pkg is

constant N : integer := 4;

--N+1 is the number of elements in the array.

subtype wordChr is character;

type string_chr is array (N downto 0) of wordChr;

end array_pkg;

library ieee;

use ieee.std_logic_1164.all;

use std.textio.all;

use work.array_pkg.all;

--Now start the code for the assembly

entity ASSMBLR is

port (START : in bit);

end ASSMBLR;

architecture BEHAVIOR_ASSM of ASSMBLR is

begin

process (START)

file infile : text;

file outfile : text;

variable fstatus, fstatus1 : file_open_status;

variable temp : string_chr := (‘ ‘, ‘ ‘, ‘ ‘, ‘ ‘, ‘ ‘);

variable code, addr : integer;

variable regstr, regstw : line;

variable ctr : integer := -1;

begin

file_open (fstatus, infile, “asm.txt”, read_mode);

file_open (fstatus1, outfile, “outf.txt”, write_mode);

-- Prepare the outfile where the results of the assembler

-- are stored.

write (regstw, “Location Code Address”);

writeline (outfile, regstw);

for i in 0 to 11 loop

--while-loop could have been used instead of for-loop.

readline (infile, regstr);

read (regstr, temp);

if (temp = (‘O’, ‘R’, ‘I’, ‘G’, ‘ ‘)) then

read (regstr, ctr);

elsif (ctr = -1)then

-- If the code of the first line in the file is not ORIG

-- report an error

write (regstw, “ ERROR: FIRST OPCODE SHOULD BE ORIG”);

writeline (outfile, regstw);

exit;

else

read (regstr, addr);

write (regstw, ctr);

write (regstw, “ “);

ctr := ctr + 1;

case temp is

when (‘H’, ‘A’, ‘L’, ‘T’, ‘ ‘) =>

code := 0;

write (regstw, code);

write (regstw, “ “);

write (regstw, addr);

writeline (outfile, regstw);

when (‘A’, ‘D’, ‘D’, ‘ ‘, ‘ ‘) =>

code := 1;

write (regstw, code);

write (regstw, “ “);

write (regstw, addr);

writeline (outfile, regstw);

when (‘M’, ‘U’, ‘L’, ‘T’, ‘ ‘) =>

code := 2;

write (regstw, code);

write (regstw, “ “);

write (regstw, addr);

writeline (outfile, regstw);

when (‘D’, ‘I’, ‘V’, ‘I’, ‘D’) =>

code := 3;

write (regstw, code);

write (regstw, “ “);

write (regstw, addr);

writeline (outfile, regstw);

when (‘X’, ‘O’, ‘R’, ‘ ‘, ‘ ‘) =>

code := 4;

write (regstw, code);

write (regstw, “ “);

write (regstw, addr);

writeline (outfile, regstw);

when (‘P’, ‘R’, ‘I’, ‘T’, ‘Y’) =>

code := 5;

write (regstw, code);

write (regstw, “ “);

write (regstw, addr);

writeline (outfile, regstw);

when (‘N’, ‘A’, ‘N’, ‘D’, ‘ ‘) =>

code := 6;

write (regstw, code);

write (regstw, “ “);

write (regstw, addr);

writeline (outfile, regstw);

when (‘C’, ‘L’, ‘A’, ‘ ‘, ‘ ‘) =>

code := 7;

write (regstw, code);

write (regstw, “ “);

write (regstw, addr);

writeline (outfile, regstw);

when (‘E’, ‘N’, ‘D’, ‘ ‘, ‘ ‘) =>

write (regstw, “END OF FILE “);

writeline (outfile, regstw);

exit;

when others =>

code := -20;

write (regstw, “ERROR “);

write (regstw, code);

writeline (outfile, regstw);

end case;

end if;

end loop;

file_close(infile);

file_close (outfile);

end process;

end BEHAVIOR_ASSM;

LISTING 8.10 Verilog Code for Storing b = 2a in file4.txt

module file_test (a, b);

input [1:0] a;

output [2:0] b;

reg [2:0] b;

integer ch1;

initial

begin

ch1 = $fopen(“file4.txt”);

$fdisplay (ch1, “\n\t\t\t This is file4.txt \n”);

$fdisplay (ch1, “ Input a in Decimal\t

\t Output b in Decimal\t\t Output b in Binary\n “);

/*The above statement when entered in the Verilog module

should be entered in one line without carriage return */

end

always @ (a)

begin

b = 2 * a;

$fmonitor (ch1,”\t%d\t\t\t\t%d\t\t\t\t%b \n”, a,b, b);

end

endmodule

LISTING 8.11 Example of Record Type

Type weather is (rain, sunny, snow, cloudy);

Type weekdays is (Monday, Tuesday, Wednesday,

Thursday, Friday, Saturday, Sunday);

Type forecast is

Record

Tempr : integer range -100 to 100;

Day : weekdays;

Cond : weather;

end record;

LISTING 8.12 VHDL Code for an Example of Record

The following is the code of the package weather_fcst

package weather_fcst is

Type cast is (rain, sunny, snow, cloudy);

Type weekdays is (Monday, Tuesday, Wednesday,

Thursday, Friday, Saturday, Sunday);

Type forecast is

Record

Tempr : real range -100.0 to 100.0;

unit : string (1 to 3);

Day : weekdays;

Cond : cast;

end record;

end package weather_fcst;

-- Now write the program

library ieee;

use ieee.std_logic_1164.all;

use std.textio.all;

use work.weather_fcst.all;

entity WEATHER_FRCST is

port (Day_in : in weekdays;

unit_in : in string (1 to 3);

out_temperature : out real;

out_unit : out string (1 to 3);

out_day : out weekdays; out_cond : out cast);

-- Type string is a predefined

end WEATHER_FRCST;

--Now we write the code

architecture behavoir_record of WEATHER_FRCST is

begin

process (Day_in, unit_in)

variable temp : forecast ;

begin

case Day_in is

when Monday =>

temp.cond := sunny;

if (unit_in = “CEN”) then

temp.tempr := 35.6;

elsif (unit_in = “FEH”) then

temp.tempr := 1.2 * 35.6 + 32.0;

else

report (“invalid units”);

end if;

when Tuesday =>

temp.cond := rain;

if (unit_in = “CEN”) then

temp.tempr := 30.2;

elsif (unit_in = “FEH”) then

temp.tempr := 1.2 * 30.2 + 32.0;

else

report (“invalid units”);

end if;

when Wednesday =>

temp.cond := sunny;

if (unit_in = “CEN”) then

temp.tempr := 37.2;

elsif (unit_in = “FEH”) then

temp.tempr := 1.2 * 37.2 + 32.0;

else

report (“invalid units”);

end if;

when Thursday =>

temp.cond := cloudy;

if (unit_in = “CEN”) then

temp.tempr := 30.2;

elsif (unit_in = “FEH”) then

temp.tempr := 1.2 * 30.2 + 32.0;

else

report (“invalid units”); end if;

when Friday =>

temp.cond := cloudy;

if (unit_in = “FEH”) then

temp.tempr := 33.9;

elsif (unit_in = “FEH”) then

temp.tempr := 1.2 * 33.9 + 32.0;

else

report (“invalid units”);

end if;

when Saturday =>

temp.cond := rain;

if (unit_in = “CEN”) then

temp.tempr := 25.1;

elsif (unit_in = “FEH”) then

temp.tempr := 1.2 * 25.1 + 32.0;

else

report (“invalid units”);

end if;

when Sunday =>

temp.cond := rain;

if (unit_in = “FEH”) then

temp.tempr := 27.1;

elsif (unit_in = “FEH”) then

temp.tempr := 1.2 * 27.1 + 32.0;

else

report (“invalid units”);

end if;

when others =>

temp.tempr := 99.99;

report (“ERROR-NOT VALID DAY”);

end case;

out_temperature <= temp.tempr;

out_unit <= unit_in;

out_day <= Day_in;

out_cond <= temp.cond;

end process;

end behavoir_record;

LISTING 8.13 VHDL Code for Stack Operation

library IEEE;

use IEEE.STD_LOGIC_1164.all;

library IEEE;

use IEEE.STD_LOGIC_1164.all;

package stack_pkg is

constant N : integer := 15;

constant M : integer := 3;

--N+1 is the number of elements in the array.

subtype Memoryword is std_logic_vector (M downto 0);

type Memory is array (N downto 0) of Memoryword;

--The above array represents a 16x4 bits memory

type stack is (push, pop, none);

--The above statement defines three members (push, pop, and

-- none) of the user-defined type stack.

end stack_pkg;

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use ieee.numeric_std.all;

use work.stack_pkg.all;

entity stck_asrt is

generic (N : integer := 15; M : integer := 3);

Port (action : in stack; Data_in : in std_logic_vector

(M downto 0); clk : in std_logic);

end stck_asrt;

architecture Behavioral of stck_asrt is

begin

stk : process (action, data_in, clk)

variable stack_pointer : integer := 0;

variable Mem_comp : Memory;

begin

if (rising_edge (clk)) then

case action is

when push =>

Mem_comp (stack_pointer) := data_in;

stack_pointer := stack_pointer + 1;

--if the operation is push, the stack pointer is

-- incremented as shown above.

--

assert (stack_pointer < 5)

report “ stack is full-program halts”

severity Failure;

--The above three statements state that if the stack

-- pointer is not less than 5, the program halts

--and the message “stack is full-program halts” is

-- displayed.

“

when pop =>

stack_pointer := stack_pointer - 1;

--If pop, the stack pointer is decremented

when others => null;

end case;

end if;

end process;

end Behavioral;

Listing 8.14 VHDL Code for a Four-Bit D-Latch Using Block Statement

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity BLCKstatement is

port(E: in std_logic; D: in std_logic_vector (3 downto 0);

Q: out std_logic_vector ( 3 downto 0));

end BLCKstatement;

architecture blck of BLCKstatement is

begin

Dlatch: block ( E = ‘1’)

begin

Q(0) <= guarded D(0);

Q(1) <= guarded D(1);

Q(2) <= guarded D(2);

Q(3) <= guarded D(3);

end block;

LISTING 8.15 Verilog Code 2x1 Multiplexer with Active Low Enable Using

Combinational User-Defined Primitive

module Mux2x1Prmtv(A, B, SEL, Gbar,Y);

input A,B,SEL,Gbar;

output Y;

multiplexer MUX1 (Y, Gbar, SEL,A,B) ;

endmodule

primitive multiplexer (mux, enable, control, dataA, dataB) ;

output mux;

input enable, control, dataA, dataB;

table

// enable control dataA dataB mux

1 ? ? ? : 0;

0 0 1 ? : 1;

0 0 0 ? : 0;

0 1 ? 1 : 1;

0 1 ? 0 : 0;

0 x 0 0 : 0;

0 x 1 1 : 1;

endtable

endprimitive

LISTING 8.16 Verilog Code for a D-Latch with Active High Clear Using

Sequential User-Defined Primitive

module latchprimitive(E, clr,D, Q, Qbar);

input E, clr,D;

output Q,Qbar;

D_latch D1 (Q, E,clr, D) ;

assign Qbar = ~ Q;

endmodule

primitive D_latch(O1, inp1, inp2, inp3) ;

output O1;

reg O1 ;

input inp1,inp2,inp3;

table

// inp1 inp2 inp3 O1 O1+

1 0 1 : ? : 1 ;

1 0 0 : ? : 0 ;

0 0 ? : ? : - ; // no change

? 1 ? : ? : 0; //if clear signal is=1, Q=0

endtable

endprimitive

LISTING 8.17 Verilog Code for One- and Two-Way Set-Associative Mapping

//single word mapping, one and two-way

module casheMem(start, cachemapping, cpuaddress1,

cpuaddress2 );

parameter N= 5;

parameter N1=11;

input start;

input cachemapping;

input [3:0] cpuaddress1, cpuaddress2;

reg [3:0] M [0:15];

reg [N:0] cache [0:3];

reg [N1:0] cache2 [0:3];

always @(start, cachemapping,cpuaddress1, cpuaddress2)

begin

M[0] = 4’d3; M[1] = 4’d4; M[2] = 4’d9;

M[3] = 4’d10; M[4] = 4’d7; M[5] = 4’d0;

M[6] = 4’d13; M[7] = 4’d15; M[8] = 4’d2;

M[9] = 4’d1; M[10] = 4’d14; M[11] = 4’d8;

M[12] = 4’d6; M[13] = 4’d5; M[14] = 4’d12;

M[15] = 4’d11;

case (cachemapping)

1’b0: begin //one-way set-associative (direct mapping)//

cache[cpuaddress1 [1:0]] =

{cpuaddress1[3:2],M[cpuaddress1]};

end

1’b1 : begin //two-way associative//

if (cpuaddress1 [1:0] == cpuaddress2 [1:0])

cache2[cpuaddress1 [1:0]] = {({cpuaddress1[3:2],

M[cpuaddress1]}),

({cpuaddress2[3:2],M[cpuaddress2]})};

begin

cache2[cpuaddress1 [1:0]]

={cpuaddress1[3:2],M[cpuaddress1]};

cache2[cpuaddress2 [1:0]]

={cpuaddress2[3:2],M[cpuaddress2]};

end

end

endcase;

end

endmodule

LISTING 8.18 Verilog Code for Determining a Hit or Miss

//Determining Hit or Miss without replacement

module hitmiss(cpuaddress, hitORmiss);

parameter N1=11;

input [3:0] cpuaddress;

output [4*8:1] hitORmiss ;

reg [4*8:1] hitORmiss;

reg [N1:0] cache [0:3];

reg [3:0] M [0:15];

reg [N1:0] data;

always @(cpuaddress)

begin

/*fill two-way the cache in order starting from

memory location 0; s0 locations 0000 and 0100

will occupy the first cache location*/

M[0] = 4’d3; M[1] = 4’d4; M[2] = 4’d9;

M[3] = 4’d10; M[4] = 4’d7; M[5] = 4’d0;

M[6] = 4’d13; M[7] = 4’d15; M[8] = 4’d2;

M[9] = 4’d1; M[10] = 4’d14; M[11] = 4’d8;

M[12] = 4’d6; M[13] = 4’d5; M[14] = 4’d12;

M[15] = 4’d11;

cache[0] = 12’b000011010111; cache[1] =12’b000100010000;

cache[2] = 12’b001001011101; cache[3] = 12’b001010011111;

hitORmiss = “miss”;

data = cache[cpuaddress[1:0]];

if ((cpuaddress [3:2] == data [11:10])

| (cpuaddress [3:2] == data [5:4]))

hitORmiss = “hit “;

end

endmodule

LISTING 8.19 Verilog Code for Two-Way Set-Associative Cache System with

Replacement

module FIFOreplace(cpuaddress, hitORmiss);

parameter N1=11;

input [3:0] cpuaddress;

output [4*8:1] hitORmiss ;

reg [4*8:1] hitORmiss;

reg [N1+1:0] cache [0:3];

reg [3:0] M [0:15];

reg [N1+2:0] data;

initial

begin

/*fill the cache in two-way set associative mapping

in order starting from memory

location 0; so locations 0000 and 0100

will occupy the first cache word*/

M[0] = 4’d3; M[1] = 4’d4; M[2] = 4’d9;

M[3] = 4’d10; M[4] = 4’d7; M[5] = 4’d0;

M[6] = 4’d13; M[7] = 4’d15; M[8] = 4’d2;

M[9] = 4’d1; M[10] = 4’d14; M[11] = 4’d8;

M[12] = 4’d6; M[13] = 4’d5; M[14] = 4’d12;

M[15] = 4’d11;

/*bit0 (the least significant bit) of the word in the

cache indicates the age of the data in the first and the

second set. If bit0=0 then the least significant

data (set) is the older and should be replaced; otherwise

if bit0 =1; the most significant data(set) is the older

and should be replaced. */

cache[0] = 13’b0000110101110; cache[1] =13’b0001000100000;

cache[2] = 13’b0010010111010; cache[3] =13’b0010100111110;

end

always @(cpuaddress)

begin

hitORmiss = “miss”;

data = cache[cpuaddress[1:0]];

if ((cpuaddress [3:2] == data [12:11])

| (cpuaddress [3:2] == data [6:5]))

hitORmiss = “hit “;

else

begin

if (data[0] == 1’b0)

begin

data[0] = 1’b1;

data[6:5] = cpuaddress [3:2];

data[4:1] = M[cpuaddress];

cache[cpuaddress[1:0]] = data;

end

else

data[0] = 1’b0;

data[12:11] = cpuaddress [3:2];

data[10:7] = M[cpuaddress];

cache[cpuaddress[1:0]] = data;

end

end

endmodule

LISTING 8.20 HDL Description of a Simple Artificial Neural Network

library IEEE;

use IEEE.STD_LOGIC_1164.all;

-- Write a package to include user-selected type

package types is

type state_machine is (state0, state1, state2, state3);

end;

-- Write the code for the state machine

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use ieee.numeric_std.all;

use work.types.all;

entity neural is

port (clk : in std_logic; I1, I2, Target4,

threshld : in real;

W31_O, W32_O, W41_O, W42_O, W43_O, Theta3_O,

Theta4_O : out real;

output4 : out real; count_O : out natural);

--The weights could have been entered as an array

end neural;

architecture Behavioral of neural is

-- write the firing function

function firing (wt : in real) return real is

variable wt_rl : real;

begin

--The firing function here is a straight line with

--saturation levels at both the positive and negative ends

if (wt <= -2.2) then

wt_rl := 0.06;

elsif (wt > 2.5) then

wt_rl := 1.0;

else

wt_rl := 0.20 * wt + 0.5;

end if;

return wt_rl;

end firing;

begin

--Write the code for the state machine

train : process (I1, I2, Target4, threshld, clk)

variable O1, O2, O3, O4, wtsum, delta3, delta4 : real;

variable eita : real := 0.5;

variable pres_st : state_machine := state0;

--Assign initial values for the weights and theta

variable W31 : real := -1.5;

variable W32 : real := -1.5;

variable W41 : real := -1.0;

variable W42 : real := -1.0;

variable W43 : real := -2.0;

variable Theta3 : real := 1.0;

variable Theta4 : real := 1.0;

variable count : natural := 0;

begin

if (clk = ‘1’ and clk’event) then

case pres_st is

when state0 =>

--Calculate outputs from Equation 2

O1 := I1;

O2 := I2;

--Calculate the weighted sum

wtsum := W31 * O1 + W32*O2 + Theta3;

--Apply the firing function

O3 := firing (wtsum);

wtsum := W41 * O1 + W42 * O2 + W43 * O3 + Theta4;

O4 := firing (wtsum);

pres_st := state1;

when state1 =>

-- Calculate errors

delta4 := (Target4 - O4)* O4 * (1.0 - O4);

delta3 := O3 * (1.0 - O3) * (delta4 * W43);

if (delta4 < threshld) then

--The threshold is a user-selected value

pres_st := state3;

else

pres_st := state2;

--Record the number of iteration

count := count + 1;

count_O <= count;

end if;

when state2 =>

--Update weights

W41 := W41 + eita * delta4 * O1;

W42 := W42 + eita * delta4 * O2;

W43 := W43 + eita * delta4 * O3;

Theta4 := Theta4 + eita * O4;

W31 := W31 + eita * delta3 * O1;

W32 := W32 + eita * delta3 * O2;

Theta3 := Theta3 + eita * O3;

pres_st := state0;

when state3 =>

--Finish; report results

W41_O <= W41;

W42_O <= W42;

W43_O <= W43;

W32_O <= W32;

W31_O <= W31;

Theta3_O <= Theta3;

Theta4_O <= Theta4;

output4 <= O4;

end case;

end if;

end process train;

end Behavioral;

Chapter 9

LISTING 9.1 Invoking a VHDL Entity From a Verilog Module

//This is the Verilog module

module mixed (a, b, c, d);

input a, b;

output c, d;

...........

VHD_enty V1 (a, b, c, d);

/*The above module VHD_enty is the VHDL entity to be

invoked in this module*/

...........

endmodule

--This is the VHDL entity

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity VHD_enty is

port (x, y : in std_logic; O1, O2 : out std_logic);

end VHD_enty;

architecture VHD_enty of VHD_enty is

begin

...........

end VHD_enty;

LISTING 9.2 Invoking a Verilog Module From a VHDL Module

-- This is the VHDL Project

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity Ver_VHD is

port (a, b : in std_logic; c : out std_logic);

end Ver_VHD;

architecture Ver_VHD of Ver_VHD is

component V_modl

port (x, y : in std_logic; z : out std_logic);

-- The name of the Component V_modl should be

-- identical to the name of the

-- Verilog module; also, the ports should be

-- identical in name and mode

-- with the inputs and outputs of the Verilog module

end component;

.......

end Ver_VHD;

//This is the Verilog module

module V_modl (x, y, z);

input x, y;

output z;

endmodule

LISTING 9.3 Mixed-Language Description of a Full Adder

--This is the Verilog module

module Full_Adder1 (x, y, cin, sum, carry);

input x, y, cin;

output sum, carry;

wire c0, c1, s0;

HA H1 (y, cin, s0, c0);

HA H2 (x, s0, sum, c1);

// Description of HA is written in VHDL in the entity HA

or (carry, c0, c1);

endmodule

library IEEE;

use ieee.std_logic_1164.all;

entity HA is

--For correct binding between this VHDL code and the above

--Verilog code, the entity has to be named HA.

port (a, b : in std_logic; s, c : out std_logic);

end HA;

architecture HA_Dtflw of HA is

begin

s <= a xor b;

c <= a and b;

end HA_Dtflw;

LISTING 9.4 Mixed-Language Description of a Nine-Bit Adder

module Nine_bitAdder (a, b, c0, sum_total, carry_out);

input [8:0] a, b;

input c0;

output [8:0] sum_total;

output carry_out;

wire cr0, cr1;

//Invoke the VHDL entity

adders_RL A1 (a [2:0], b [2:0], c0,

sum_total [2:0], cr0);

adders_RL A2 (a [5:3], b [5:3], cr0,

sum_total [5:3], cr1);

adders_RL A3 (a [8:6], b [8:6], cr1,

sum_total [8:6], carry_out);

//adders_RL is the name of the VHDL entity

endmodule

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

-- This is a VHDL data-flow code for a 3-bit

--carry-lookahead adder.

entity adders_RL is

port (x, y : in std_logic_vector (2 downto 0);

cin : in std_logic;

sum : out std_logic_vector (2 downto 0);

cout : out std_logic);

--The entity name is identical to that of the Verilog

-- module. The input and output ports have

-- the same mode as the inputs. and outputs of the Verilog

-- module.

end adders_RL;

architecture lkh_DtFl of adders_RL is

signal c0, c1 : std_logic;

signal p, g : std_logic_vector (2 downto 0);

constant delay_gt : time := 0 ns;

--The gate propagation delay here is equal to 0.

begin

g(0) <= x(0) and y(0) after delay_gt;

g(1) <= x(1) and y(1) after delay_gt;

g(2) <= x(2) and y(2) after delay_gt;

p(0) <= x(0) or y(0) after delay_gt;

p(1) <= x(1) or y(1) after delay_gt;

p(2) <= x(2) or y(2) after delay_gt;

c0 <= g(0) or (p(0) and cin) after 2 * delay_gt;

c1 <= g(1) or (p(1) and g(0)) or (p(1) and

p(0) and cin) after 2 * delay_gt;

cout <= g(2) or (p(2) and g(1)) or (p(2) and p(1)

and g(0)) or(p(2) and p(1) and p(0) and cin)

after 2 * delay_gt;

sum(0) <= (p(0) xor g(0)) xor cin after delay_gt;

sum(1) <= (p(1) xor g(1)) xor c0 after delay_gt;

sum(2) <= (p(2) xor g(2)) xor c1 after delay_gt;

end lkh_DtFl;

LISTING 9.5 Mixed-Language Description of a Three-Bit Adder with a Zero

Flag

module three_bitAdd (A, B, cin, Sum_3, Carry_out, Z_flag);

input [2:0] A, B;

input cin;

output [2:0] Sum_3;

output Carry_out;

output Z_flag;

wire cr0, cr1;

full_add FA0 (A[0], B[0], cin, Sum_3[0], cr0);

full_add FA1 (A[1], B[1], cr0, Sum_3[1], cr1);

full_add FA2 (A[2], B[2], cr1, Sum_3[2], Carry_out);

--The above modules invoke the VHDL entity full_add

assign Z_flag = ~(Sum_3[0] | Sum_3[1] | Sum_3[2] |

Carry_out);

endmodule

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity full_add is

Port (X, Y, cin : in std_logic;

sum, cout : out std_logic);

--This is a 1-bit full adder component built from

--AND-OR-NOT gates; see Figure 9.4.

end full_add;

architecture beh_vhdl of full_add is

--Instantiate the components of a 1-bit adder;

--see Figure 9.4.

component inv

port(I1 : in std_logic; O1 : out std_logic);

end component;

component and2

port(I1, I2 : in std_logic; O1 : out std_logic);

end component;

component and3

port(I1, I2, I3 : in std_logic; O1 : out std_logic);

end component;

component or3

port(I1, I2, I3 : in std_logic; O1 : out std_logic);

end component;

component or4

port(I1, I2, I3, I4 : in std_logic;

O1 : out std_logic);

end component;

for all : inv use entity work.bind1 (inv_0);

for all : and2 use entity work.bind2 (and2_0);

for all : and3 use entity work.bind3 (and3_0);

for all : or3 use entity work.bind3 (or3_0);

for all : or4 use entity work.bind4 (or4_0);

--The above five “for” statements are to bind the inv,

-- and3, and2, or3, and or4 with the architecture

-- beh_vhdl. See Chapter 4, “Structural Descriptions.”

signal Xbar, Ybar, cinbar, s0, s1, s2,

s3, s4, s5, s6 : std_logic;

begin

Iv1 : inv port map (X, Xbar);

Iv2 : inv port map (Y, Ybar);

Iv3 : inv port map (cin, cinbar);

A1 : and3 port map (X, Y, cin, s0);

A2 : and3 port map (Xbar, Y, cinbar, s1);

A3 : and3 port map (Xbar, Ybar, cin, s2);

A4 : and3 port map (X, Ybar, cinbar, s3);

A5 : and2 port map (X, cin, s4);

A6 : and2 port map (X, Y, s5);

A7 : and2 port map (Y, cin, s6);

O1 : or4 port map (s0, s1, s2, s3, sum);

O2 : or3 port map (s4, s5, s6, cout);

end beh_vhdl;

--The following is the behavioral description of the

--components instantiated in the entity full_add.

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity bind1 is

port (I1 : in std_logic; O1 : out std_logic);

end bind1;

architecture inv_0 of bind1 is

begin

O1 <= not I1;

end inv_0;

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity bind2 is

port (I1, I2 : in std_logic; O1 : out std_logic);

end bind2;

architecture and2_0 of bind2 is

begin

O1 <= I1 and I2;

end and2_0;

architecture or2_0 of bind2 is

begin

O1 <= I1 or I2;

end or2_0;

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity bind3 is

port (I1, I2, I3 : in std_logic; O1 : out std_logic);

end bind3;

architecture and3_0 of bind3 is

begin

O1 <= I1 and I2 and I3;

end and3_0;

architecture or3_0 of bind3 is

begin

O1 <= I1 or I2 or I3;

end or3_0;

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity bind4 is

Port (I1, I2, I3, I4 : in std_logic;

O1 : out std_logic);

end bind4;

architecture or4_0 of bind4 is

begin

O1 <= I1 or I2 or I3 or I4;

end or4_0;

LISTING 9.6 Mixed-Language Description of a Master-Slave D Flip-Flop

//This is the Verilog module

module D_Master (D_in, clk, Q_out, Qb_out);

input D_in, clk;

output Q_out, Qb_out;

wire Q0, Qb, clkb; /* wire statement here can be omitted.*/

assign clkb = ~ clk;

assign clk2 = ~ clkb;

D_Latch D0 (D_in, clkb, Q0, Qb);

//D_Latch is the name of a VHDL entity describing a D-Latch

D_Latch D1 (Q0, clk2, Q_out, Qb_out);

endmodule

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity D_Latch is

--The entity has the same name as

--the calling Verilog module

port (D, E : in std_logic;

Q, Qbar : buffer std_logic);

end D_Latch;

architecture DL_DtFl of D_Latch is

--This architecture describes a D-latch using

--data-flow description

constant Delay_EorD : Time := 9 ns;

constant Delay_inv : Time := 1 ns;

begin

Qbar <= (D and E) nor (not E and Q) after Delay_EorD;

Q <= not Qbar after Delay_inv;

end DL_DtFl;

LISTING 9.7 Mixed-Language Description of a 4x4 Comparator

module compr_genr (X, Y, xgty, xlty, xeqy);

parameter N = 3;

input [N:0] X, Y;

output xgty, xlty, xeqy;

wire [N:0] sum, Yb;

wire [N+1:0] carry, eq;

assign carry[0] = 1’b0;

assign eq[0] = 1’b1;

assign Yb = ~Y;

FULL_ADDER FA (X[0], Yb[0], carry[0],

sum[0], carry[1]);

-- The module FULL_ADDER has the same name

-- as the VHDL entity FULL_ADDER

FULL_ADDER FA1 (X[1], Yb[1], carry[1],

sum[1], carry[2]);

FULL_ADDER FA2 (X[2], Yb[2], carry[2],

sum[2], carry[3]);

FULL_ADDER FA3 (X[3], Yb[3], carry[3],

sum[3], carry[4]);

generate

genvar i;

for (i = 0; i <= N; i = i + 1)

begin : u

and (eq[i+1], sum[i], eq[i]);

end

endgenerate

assign xgty = carry [N+1];

assign xeqy = eq [N+1];

nor (xlty, xeqy, xgty);

endmodule

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity FULL_ADDER is

Port (A, B, cin : in std_logic;

sum_1, cout : out std_logic);

end FULL_ADDER;

architecture beh_vhdl of FULL_ADDER is

--This architecture is a behavioral

--description of a full adder.

begin

oneBit : process (A, B, cin)

variable y : std_logic_vector (2 downto 0);

begin

Y := (A & B & Cin);

--The above statement is a concatenation of

--three bits A, B, and Cin

case y is

when “000” => sum_1 <= ‘0’; cout <= ‘0’;

when “110” => sum_1 <= ‘0’; cout <= ‘1’;

when “101” => sum_1 <= ‘0’; cout <= ‘1’;

when “011” => sum_1 <= ‘0’; cout <= ‘1’;

when “111” => sum_1 <= ‘1’; cout <= ‘1’;

when others => sum_1 <= ‘1’; cout <= ‘0’;

--Others here refer to 100, 001, 010

end case;

end process;

end beh_vhdl;

LISTING 9.8 Mixed-Language Description of an AND Gate

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

--This is the VHDL module

entity andgate is

port (a, b : in std_logic; c : out std_logic);

end andgate;

architecture andgate of andgate is

component and2

--For correct binding with the Verilog module,

--the name of the component should be identical

--to that of the Verilog module.

port (x, y : in std_logic; z : out std_logic);

--The name of the ports should be identical to the name

--of the inputs/outputs of the Verilog module.

end component;

begin

g1 : and2 port map (a, b, c);

end andgate;

//This is the Verilog module

module and2 (x, y, z);

input x, y;

output z;

and( z, x , y);

endmodule

LISTING 9.9 Mixed-Language Description of a JK Flip-Flop

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity JK_FF is

Port (Jx, Kx, clk, clx : in std_logic;

Qx, Qxbar : out std_logic);

end JK_FF;

architecture JK_FF of JK_FF is

--The JK flip flop is declared as a component

component jk_verilog

port(j, k, ck, clear : in std_logic;

q, qb : out std_logic);

end component;

begin

jk1 : jk_verilog port map (Jx, Kx, clk, clx, Qx, Qxbar);

end JK_FF;

module jk_verilog (j, k, ck, clear, q, qb);

// The module name jk_verilog matches

// the name of the VHDL components

input j, k, ck, clear;

output q, qb;

--The input and output ports match those of the

--VHDL component, jk_verilog

reg q, qb;

reg [1:0] JK;

always @ (posedge ck, clear)

begin

if (clear == 1)

begin

q = 1’b0;

qb = 1’b1;

end

else

begin

JK = {j, k};

case (JK)

2’d0 : q = q;

2’d1 : q = 0;

2’d2 : q = 1;

2’d3 : q = ~q;

endcase

qb = ~q;

end

end

endmodule

LISTING 9.10 Mixed-Language Description of Three-Bit Counter with Clear

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity countr_3 is

port (clk, clrbar : in std_logic;

q, qb : inout std_logic_vector (2 downto 0));

end countr_3;

architecture CNTR3 of countr_3 is

component JK_FF

port (I1, I2, I3 : in std_logic;

O1, O2 : inout std_logic);

end component;

component inv

port (I1 : in std_logic; O1 : out std_logic);

end component;

component and2

port (I1, I2 : in std_logic; O1 : out std_logic);

end component;

component or2

port (I1, I2 : in std_logic; O1 : out std_logic);

end component;

signal J1, K1, J2, K2, clr, clrb1, s1, high : std_logic;

begin

high <= ‘1’;

FF0 : JK_FF port map (clrb1, High, clk, q(0), qb(0));

A1 : and2 port map (clrb1, q(0), J1);

inv1 : inv port map (clr, clrb1);

inv2 : inv port map (clrbar, clr);

r1 : or2 port map (q(0), clr, K1);

FF1 : JK_FF port map (J1, K1, clk, q(1), qb(1));

A2 : and2 port map (q(0), q(1), s1);

A3 : and2 port map (clrb1, s1, J2);

r2 : or2 port map (s1, clr, K2);

FF2 : JK_FF port map (J2, K2, clk, q(2), qb(2));

end CNTR3 ;

module and2 (I1, I2, O1);

//This Verilog module represents an AND function

input I1, I2;

output O1;

assign O1 = I1 & I2;

endmodule

module inv (I1, O1);

//This Verilog module represents an INVERT function

input I1;

output O1;

assign O1 = ~I1;

endmodule

module or2 (I1, I2, O1);

//This Verilog module represents an OR function

input I1, I2;

output O1;

assign O1 = I1 | I2;

endmodule

module JK_FF (I1, I2, I3, O1, O2);

//This Verilog module represents a JK flip-flop.

input I1, I2, I3;

output O1, O2;

reg O1, O2;

reg [1:0] JK;

initial

begin

O1 = 1’b0;

O2 = 1’b1;

end

always @ (posedge I3)

begin

JK = {I1, I2};

case (JK)

2’d0 : O1 = O1;

2’d1 : O1 = 0;

2’d2 : O1 = 1;

2’d3 : O1 = ~O1;

endcase

O2 = ~O1;

end

endmodule

LISTING 9.11 Mixed-Language Description of an N-Bit Asynchronous Counter

--This is a VHDL module

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity asynch_ctrMx is

Generic (N : integer := 3);

port (clk, clear : in std_logic;

C, Cbar : out std_logic_vector (N-1 downto 0);

rco : out std_logic);

end asynch_ctrMx;

architecture CT_strgnt of asynch_ctrMx is

component jkff is

--This is a JK flip-flop with a clear bound to Verilog

-- module jkff

port (j, k, clk, clear : in std_logic;

q, qb : out std_logic);

end component;

component andgate is

--This is a three-input AND gate bound to Verilog module

-- andgate

port (I1, I2, I3 : in std_logic; O1 : out std_logic);

end component;

signal h, l : std_logic;

signal s : std_logic_vector (N downto 0);

signal s1 : std_logic_vector (N downto 0);

signal C_tem : std_logic_vector (N-1 downto 0);

begin

h <= ‘1’;

l <= ‘0’;

s <= (C_tem & clk);

-- s is the concatenation of Q and clk. We need this

-- concatenation to describe the clock of

--each JK flip-flop.

s1(0) <= not clear;

Gnlop : for i in (N - 1) downto 0 generate

G1 : jkff port map (h, h, s(i), clear, C_tem(i), Cbar(i));

end generate GnLop;

C <= C_tem;

rc_gen : for i in (N - 2) downto 0 generate

--This loop to determine the ripple carry-out

rc : andgate port map (C_tem(i), C_tem(i+1),

s1(i), s1(i+1));

end generate rc_gen;

rco <= s1(N-1);

end CT_strgnt;

module jkff (j, k, clk, clear, q, qb);

// This is a behavioral description of a JK flip-flop

input j, k, clk, clear;

output q, qb;

reg q, qb;

reg [1:0] JK;

always @ (posedge clk, clear)

begin

if (clear == 1)

begin

q = 1’b0;

qb = 1’b1;

end

else

begin

JK = {j,k};

case (JK)

2’d0 : q = q;

2’d1 : q = 0;

2’d2 : q = 1;

2’d3 : q = ~q;

endcase

qb = ~q;

end

end

endmodule

module andgate (I1, I2,I3, O1);

//This is a three-input AND gate

input I1, I2, I3;

output O1;

assign O1 = (I1 & I2 & I3);

endmodule

LISTING 9.12 Mixed-Language Description of a 2x1 Multiplexer

--This is the VHDL module.

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity mux2x1_mxd is

Port (a, b, Sel, E : in std_logic; ybar : out std_logic);

end mux2x1_mxd;

architecture mux2x1switch of mux2x1_mxd is

component nmos_verlg

--This component, after linking to a

--Verilog module, behaves as an nmos switch

port (O1 : out std_logic; I1, I2 : in std_logic);

end component;

component pmos_verlg

--This component, after linking to a Verilog module,

-- behaves as a pmos switch.

port (O1 : out std_logic; I1, I2 : in std_logic);

end component;

--constant vdd : std_logic := ‘1’;

--constant gnd : std_logic := ‘0’;

-- In Chapter 5 we wrote Vdd and gnd as constants.

-- Some VHDL/Verilog simulators do not transfer constants

-- between VHDL and Verilog. So we wrote them as signals.

signal vdd, gnd, Selbar, s0, s1, s2, s3 : std_logic;

begin

vdd <= ‘1’;

gnd <= ‘0’;

--Invert signal Sel. If the complement of Sel is available,

--then no need for the following pair of transistors.

v1 : pmos_verlg port map (Selbar, vdd, Sel);

v2 : nmos_verlg port map (Selbar, gnd, Sel);

--Write the pull-down combination

n1 : nmos_verlg port map (s0, gnd, E);

n2 : nmos_verlg port map (s1, s0, Sel);

n3 : nmos_verlg port map (ybar, s1, a);

n4 : nmos_verlg port map (s2, s0, Selbar);

n5 : nmos_verlg port map (ybar, s2, b);

--Write the pull-up combination

p1 : pmos_verlg port map (ybar, vdd, E);

p2 : pmos_verlg port map (ybar, s3, Sel);

p3 : pmos_verlg port map (ybar, s3, a);

p4 : pmos_verlg port map (s3, vdd, Selbar);

p5 : pmos_verlg port map (s3, vdd, b);

end mux2x1switch;

// This is the Verilog Module

module nmos_verlg (O1, I1, I2);

input I1, I2;

output O1;

nmos (O1, I1, I2);

endmodule

module pmos_verlg (O1, I1, I2);

input I1, I2;

output O1;

pmos (O1, I1, I2);

endmodule

LISTING 9.13 Instantiating casex in a VHDL Module

library IEEE;

use IEEE.STD_LOGIC_1164.all;

entity P_encodr is

Port (X : in std_logic_vector (3 downto 0);

Y : out std_logic_vector (3 downto 0));

end P_encodr;

architecture P_encodr of P_encodr is

component cas_x

--The name of the component is identical to the name of the

--Verilog module

port (a : in std_logic_vector (3 downto 0);

b : out std_logic_vector (3 downto 0));

end component;

begin

ax : cas_x port map (X, Y);

end P_encodr;

module cas_x (a, b);

input [3:0] a;

output [3:0] b;

reg [3:0] b;

always @ (a)

begin

casex (a)

4’bxxx1 : b = 4’d1;

4’bxx10 : b = 4’d2;

4’bx100 : b = 4’d4;

4’b1000 : b = 4’d8;

default : b = 4’d0;

endcase

end

endmodule

LISTING 9.14 Mixed-Language Description of a Simple RC Filter

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use std.textio.all;

use ieee.numeric_std.all;

entity Filter_draw is

Port (w, w_ctoff : in std_logic_vector (3 downto 0);

Hw_vhd : out std_logic_vector (7 downto 0));

end Filter_draw;

architecture Filter_draw of Filter_draw is

Function TO_Intgr (a : in std_logic_vector) return

integer is

--This Function converts std_logic_vector type to integer

variable result : integer;

begin

result := 0;

lop1 : for i in a’ range loop

if a(i) = ‘1’ then

result := result + 2**i;

end if;

end loop;

return result;

end TO_Intgr;

component flter_RC

--The name of the component “flter_RC” is the same name as

-- the Verilog module.

port (I1, I2 : in std_logic_vector (3 downto 0);

O1 : out std_logic_vector (7 downto 0));

end component;

signal Hw_tmp : std_logic_vector (7 downto 0);

begin

dw : flter_RC port map (w, w_ctoff, Hw_tmp);

//output the data on a file

fl : process (w, w_ctoff, Hw_tmp)

file outfile : text;

variable fstatus : file_open_status;

variable temp : line;

variable Hw_int, w_int, w_ctoffintg : integer;

begin

--Files can take integer, real, or character;

--they cannot take std-logic-vector; so convert to integer.

Hw_int := TO_Intgr (Hw_tmp);

w_int := TO_Intgr (w);

w_ctoffintg := TO_Intgr (w_ctoff);

file_open (fstatus, outfile, “Wfile_int.txt”, write_mode);

--The file name is Wfile_int.txt

--Write headings. Be sure your simulator supports

-- formatted output. otherwise take out all formatted

-- output statements

write (temp, “ This is a Simple R-C Low Pass Filter”);

--The above statement when entered in the VHDL module

-- should be entered in one line without carriage return.

writeline (outfile, temp);

write (temp, “ “);

writeline (outfile, temp);

write (temp, “ FREQUENCY

CUTOFF Amplitude Square”);

--The above statement when entered in the VHDL module

--should be entered in one line without carriage return.

writeline (outfile, temp);

write (temp, “ “);

--write the values of the filter parameters

write (temp, w_int);

write (temp, “ “);

write (temp, w_ctoffintg);

write (temp, “ “);

write (temp, Hw_int);

writeline (outfile, temp);

file_close (outfile);

Hw_vhd <= Hw_tmp;

end process fl;

end Filter_draw;

// Next we write the Verilog module;

// the module performs a real division

module flter_RC (I1, I2, O1);

/*The module performs the real division

O1 = 1/[(I1/I2)**2 + 1]*/

input [3:0] I1, I2;

output [7:0] O1;

reg [7:0] O1;

real S, s1;

always @ (I1, I2)

begin

s1 = ((1.0*I1)/(1.0 * I2))**2 ;

/*we multiply by 1.0 so the division is done in real

format.*/

S = 1.0 / (1.0 + s1);

O1 = 100.00 * S;

end

endmodule

LISTING 9.15 Mixed-Language Description of a 2x1 Multiplexor with Active-

Low Enable Using Verilog User-Defined Primitive

--This is the VHDL code

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity muxVHDL is

port(G1,SL1,A1,B1: in std_logic; Y1: out std_logic);

end muxVHDL;

architecture Behavioral of muxVHDL is

component Mux2x1Prmtvvlog

port(Gbar, SEL,A,B: in std_logic; Y: out std_logic);

end component;

begin

pl1: Mux2x1Prmtvvlog port map(G1,SL1,A1,B1,Y1);

end Behavioral;

//This is the Verilog code that should be

//attached in the same project as the VHDL code

module Mux2x1Prmtvvlog(Gbar, SEL,A,B,Y);

input Gbar, SEL,A,B;

output Y;

multiplexer MUX1 (Y, Gbar, SEL,A,B) ;

endmodule

primitive multiplexer (mux, enable, control, dataA, dataB) ;

output mux;

input enable, control, dataA, dataB;

table

// enable control dataA dataB mux

1 ? ? ? : 0;

0 0 1 ? : 1;

0 0 0 ? : 0;

0 1 ? 1 : 1;

0 1 ? 0 : 0;

0 x 0 0 : 0;

0 x 1 1 : 1;

endtable

endprimitive

Chapter 10

LISTING 10.1 VHDL Code for System1 Entity

entity system1 is

port (a, b : in bit; d : out bit);

end system1;

LISTING 10.2 VHDL Code for System2 Entity

entity system2 is

port (a, b : in std_logic; d : out std_logic);

end system2;

Listing 10.3 VHDL Code for System3 Entity

entity system3 is

port (a, b : in std_logic_vector (3 downto 0);

d : out std_logic_vector (7 downto 0));

end entity system3;

LISTING 10.4 VHDL Code for System4 Entity

entity system4 is

port (a, b : in signed (3 downto 0);

d : out std_logic_vector (7 downto 0));

end entity system4;

LISTING 10.5 VHDL Code for System5 Entity

entity system5 is

port (a, b : in unsigned (3 downto 0);

d : out std_logic_vector (7 downto 0));

end entity system5;

LISTING 10.6 VHDL Code for System6 Entity

entity system6 is

port (a, b : in unsigned (3 downto 0);

d : out integer range -10 to 10);

end entity system6;

LISTING 10.7 VHDL Code for System7 Entity

entity system7 is

generic (N : integer := 4; M : integer := 3);

Port (a, b : in std_logic_vector (N downto 0);

d : out std_logic_vector (M downto 0));

end system7;

LISTING 10.8 VHDL Code for ALUS2 Entity

package codes is

type op is (add, mul, divide, none);

end;

use work. codes;

entity ALUS2 is

port (a, b : in std_logic_vector (3 downto 0);

cin : in std_logic; opc : in op;

z : out std_logic_vector (7 downto 0);

cout : out std_logic; err : out Boolean);

end ALUS2;

LISTING 10.9 VHDL Code for Array1 Entity

package array_pkg is

constant N : integer := 4;

constant M : integer := 3;

subtype wordN is std_logic_vector (M downto 0);

type strng is array (N downto 0) of wordN;

end array_pkg;

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

use work.array_pkg.all;

entity array1 is

generic (N : integer := 4; M : integer := 3);

Port (a : in strng; z : out std_logic_vector (M downto 0));

end array1;

LISTING 10.10 VHDL Code for Weather_frcst Entity

package weather_fcst is

Type cast is (rain, sunny, snow, cloudy);

Type weekdays is (Monday, Tuesday, Wednesday,

Thursday, Friday, Saturday, Sunday);

end package weather_fcst;

library ieee;

use ieee.std_logic_1164.all;

use std.textio.all;

use work.weather_fcst.all;

entity WEATHER_FRCST is

port (Day_in : in weekdays; out_temperature : out integer

range -100 to 100; out_day : out weekdays;

out_cond : out cast);

end WEATHER_FRCST;

LISTING 10.11 VHDL Code for Entity Procs_Mchn

library ieee;

use ieee.std_logic_1164.all;

package state_machine is

Type machine is (state0, state1, state2, state3);

Type st_machine is

record

state : machine;

weight : natural range 3 to 16;

Measr : std_logic_vector (5 downto 0);

end record;

end package state_machine;

library ieee;

use ieee.std_logic_1164.all;

use std.textio.all;

use work.state_machine.all ;

entity Procs_Mchn is

port (S : in machine; Y : in st_machine;

Z : out integer range -5 to 5);

end Procs_Mchn;

LISTING 10.12 Verilog Code for Module System1v

module system1v (a, b, d);

input a, b;

output d;

endmodule

LISTING 10.13 Verilog Code for Module System2v

module system2v (X, Y, Z);

input [3:0] X, Y;

output [7:0] Z;

reg [7:0] Z

........

endmodule

LISTING 10.14 Verilog Code for Module System3v

module system3v (a, b, c);

parameter N = 4;

parameter M = 3;

input [N:0] a;

output [M:0] c;

input b;

.........

endmodule

LISTING 10.15 Verilog Code for Module Array1v

module array1v (start, grtst);

parameter N = 4;

parameter M = 3;

input start;

output [3:0] grtst;

reg[M:0] a[0:N];

..............

endmodule

LISTING 10.16a Mapping VHDL Code for Signal-Assignment Statement Y <= X

library ieee;

use ieee.std_logic_1164.all;

entity SIGNA_ASSN is

port (X : in bit; Y : out bit);

end SIGNA_ASSN;

architecture BEHAVIOR of SIGNA_ASSN is

begin

P1 : process (X)

begin

Y <= X;

end process P1;

end BEHAVIOR;

LISTING 10.16b Mapping Verilog Code for Signal-Assignment Statement Y = X

module SIGNA_ASSN (X, Y);

input X;

output Y;

reg y;

always @ (X)

begin

Y = X;

end

endmodule

LISTING 10.17 VHDL Code for Signal-Assignment Statement Y = 2 * X + 3:

VHDL and Verilog

VHDL Signal-Assignment Statement Y = 2 * X + 3

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

entity sign_assn2 is

port (X : in unsigned (1 downto 0);

Y : out unsigned (3 downto 0));

end ASSN2;

architecture BEHAVIOR of sign_assn2 is

begin

P1 : process (X)

begin

Y <= 2 * X + 3;

end process P1;

end BEHAVIOR;

Verilog Signal-Assignment Statement Y = 2 * X + 3

module sign_assn2 (X, Y);

input [1:0] X;

output [3:0] Y;

reg [3:0] Y;

always @ (X)

begin

Y = 2 * X + 3;

end

endmodule

LISTING 10.18 Structural Verilog Code for the Logic Diagram in Figure 10.15b.

module sign_struc(X, Y);

input [1:0] X;

output [3:0] Y;

reg [3:0] Y;

always @ (X)

begin

Y[0] = 1'b1;

Y[1] = ~ X[0];

Y[2] = X[0] ^ X[1];

Y[3] = X[1] & X[0];

end

endmodule

Listing 10.19 VHDL Variable-Assignment Statement

library ieee;

use ieee.std_logic_1164.all;

entity parity_even is

port (x : in std_logic_vector (3 downto 0);

C : out std_logic);

end parity_even;

architecture behav_prti of parity_even is

begin

P1 : process (x)

variable c1 : std_logic;

begin

c1 := (x(0) xor x(1)) xor (x(2) xor x(3));

C <= c1;

end process P1;

end behav_prti;

LISTING 10.20 Mapping Logical Operators: VHDL and Verilog

VHDL: Mapping Logical Operators

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity decod_var is

port (a : in std_logic_vector (1 downto 0);

D : out std_logic_vector (3 downto 0));

end decod_var;

architecture Behavioral of decod_var is

begin

dec : process (a)

variable a0bar, a1bar : std_logic;

begin

a0bar := not a(0);

a1bar := not a(1);

D(0) <= not (a0bar and a1bar);

D(1) <= not (a0bar and a(1));

D(2) <= not (a(0) and a1bar);

D(3) <= not (a(0) and a(1));

end process dec;

end Behavioral;

Verilog: Mapping Logical Operators

module decod_var (a, D);

input [1:0] a;

output [3:0] D;

reg a0bar, a1bar;

reg [3:0] D;

always @ (a)

begin

a0bar = ~ a[0];

a1bar = ~ a[1];

D[0] = ~ (a0bar & a1bar);

D[1] = ~ (a0bar & a[1]);

D[2] = ~ (a[0] & a1bar);

D[3] = ~ (a[0] & a[1]);

end

endmodule

LISTING 10.21 Example of IF-else Statement: VHDL and Verilog

VHDL IF-else Description

process (a, x)

begin

if (a = '1') then

Y <= X;

else

Y <= '0';

end if;

end process;

Verilog IF-else Description

always @ (a, X)

begin

if (a == 1'b1)

Y = X;

else

Y = 1'b0;

end

LISTING 10.22 Example of Multiplexer IF-else Statement: VHDL and

Verilog

VHDL Multiplexer IF-else Description

process (a, X, X1)

begin

if (a = '1') then

Y <= X;

else

Y <= X1;

end if;

end process;

Verilog Multiplexer IF-else Description

always @ (a, X, X1)

begin

if (a == 1'b1)

Y = X;

else

Y = X1;

end

LISTING 10.23 Example of Comparison Using IF-else Statement: VHDL

and Verilog

VHDL IF-else Statement

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity IF_st is

port (a : in std_logic_vector (2 downto 0);

Y : out Boolean);

end IF_st;

architecture IF_st of IF_st is

begin

IfB : process (a)

variable tem : Boolean;

begin

if (a < "101") then

tem := true;

else

tem := false;

end if;

Y <= tem;

end process;

end IF_st;

Verilog IF-else Statement

module IF_st (a, Y);

input [2:0] a;

output Y;

reg Y;

always @ (a)

begin

if (a < 3’b101)

Y = 1'b1;

else

Y = 1'b0;

end

endmodule

LISTING 10.24 Example of elseif and Else-If: VHDL and Verilog

VHDL elseif Description

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

entity elseif is

port (BP : in natural range 0 to 7;

ADH : out natural range 0 to 15);

end;

architecture elseif of elseif is

begin

ADHP : process(BP)

variable resADH : natural := 0;

begin

if BP <= 2 then resADH := 15;

elsif BP >= 5 then resADH := 0;

else

resADH := BP * (-5) + 25;

end if;

ADH <= resADH;

end process ADHP;

end elseif;

Verilog Else-If Description

module elseif (BP, ADH);

input [2:0] BP;

output [3:0] ADH;

reg [3:0] ADH;

always @ (BP)

begin

if (BP <= 2) ADH = 15;

else if (BP >= 5) ADH = 0;

else

ADH = BP * (-5) + 25;

end

endmodule

LISTING 10.25 Example of IF Statement with Storage: VHDL and Verilog

VHDL IF Statement with Storage

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity If_store is

port (a, X : in std_logic; Y : out std_logic);

end If_store;

architecture If_store of If_store is

begin

process (a, X)

begin

if (a = ‘1’) then

Y <= X;

end if;

end process;

end If_store;

Verilog IF Statement with Storage

module If_store (a, X, Y);

input a, X;

output Y;

reg Y;

always @ (a, X)

begin

if (a == 1’b1)

Y = X;

end

endmodule

Listing 10.26 Else-If Statement with Gate-Level Logic

package weather_fcst is

Type unit is (cent, half, offset);

end package weather_fcst;

library ieee;

use ieee.std_logic_1164.all;

use work.weather_fcst.all;

entity weather is

port (a : in unit; tempr : in integer range 0 to 15;

z : out integer range 0 to 15);

end weather;

architecture weather of weather is

begin

T : process (a, tempr)

variable z_tem : integer range 0 to 15;

begin

if ((tempr <= 7) and (a = cent)) then

z_tem := tempr;

elsif ((tempr <= 7) and (a = offset)) then

z_tem := tempr + 4;

elsif ((tempr <= 7) and (a = half)) then

z_tem := tempr /2;

else

z_tem := 15;

end if;

z <= z_tem;

end process T;

end weather;

LISTING 10.27 Example of case Mapping

module case_nostr (a, b, ct, d);

input [3:0] a, b;

input ct;

output [4:0] d;

reg [4:0] d;

always @ (a, b, ct)

begin

case (ct)

1’b0 : d = a + b;

1’b1 : d = a - b;

endcase

end

endmodule

LISTING 10.28 case Statement with Storage

module case_str (a, b, ct, d);

input [3:0] a, b;

input ct;

output [4:0] d;

reg [4:0] d;

always @ (a, b, ct)

begin

case (ct)

1’b0: d = a + b;

1’b1: ; /* This is a blank statement with

no operation (null in VHDL)*/

endcase

end

endmodule

LISTING 10.29 Verilog casex

module Encoder_4 (IR, RA);

input [3:0] IR;

output [3:0] RA;

reg [3:0] RA;

always @ (IR)

begin

casex (IR)

4’bxxx1 : RA = 4’d1;

4’bxx10 : RA = 4’d2;

4’bx100 : RA = 4’d4;

4’b1000 : RA = 4’d8;

default : RA = 4’d0;

endcase

end

endmodule

LISTING 10.30 Example of case with Storage

library IEEE;

use IEEE.STD_LOGIC_1164.all;

package types is

type states is (state0, state1, state2, state3);

end;

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use work.types.all;

entity state_machine is

port (A, clk : in std_logic; pres_st : buffer states;

Z : out std_logic);

end state_machine;

architecture st_behavioral of state_machine is

begin

FM : process (clk, pres_st, A)

variable present : states := state0;

begin

if (clk = ‘1’ and clk’event) then

--clock’event is an attribute to the signal clk;

--the above if Boolean expression means the positive

-- edge of clk

--

case pres_st is

when state0 =>

if A ='1' then

present := state1;

Z <= '0';

else

present := state0;

Z <= '1';

end if;

when state1 => if A ='1’ then

present := state2;

Z <= '0';

else

present := state3;

Z <= '0';

end if;

when state2 => if A ='1' then

present := state3;

Z <= '1';

else

present := state0;

Z <= '0';

end if;

when state3 => if A ='1' then

present := state0;

Z <= '1';

else

present := state2;

Z <= '1';

end if;

end case;

pres_st <= present;

end if;

end process FM;

end st_behavioral;

LISTING 10.31 A For-Loop Statement: VHDL and Verilog

VHDL For-Loop Statement

for i in 0 to 63 loop

temp(i) := temp(i) + b(i);

end loop;

Verilog For-Loop Statement

for i = 0; i <= 63; i = i + 1

begin

temp[i] = temp[i] + b[i];

end

LISTING 10.32 VHDL Code Includes For-Loop

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity Listing10_32 is

port (a : in std_logic_vector (3 downto 0);

c : in integer range 0 to 15;

b : out std_logic_vector (3 downto 0));

end Listing10_32;

architecture Listing10_32 of Listing10_32 is

begin

shfl : process (a, c)

variable result, j : integer;

variable temp : std_logic_vector (3 downto 0);

begin

result := 0;

lop1 : for i in 0 to 3 loop

if a(i) = ‘1’ then

result := result + 2**i;

end if;

end loop;

if result > c then

lop2 : for i in 0 to 3 loop

j := (i + 2) mod 4;

temp (j) := a(i);

end loop;

else

lop3 : for i in 0 to 3 loop

j := (i + 1) mod 4;

temp (j) := a(i);

end loop;

end if;

b <= temp;

end process shfl;

end Listing10_32;

Listing 10.33 A Verilog Example of a Task

module example_task (a1, b1, d1);

input a1, b1;

output d1;

reg d1;

always @ (a1, b1)

begin

xor_synth (d1, a1, b1);

end

task xor_synth;

output d;

input a, b;

begin

d = a ^ b;

end

endtask

endmodule

LISTING 10.34 An Example of a Procedure

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity Int_Bin is

generic (N : integer := 3);

port (X_bin : out std_logic_vector (N downto 0);

Y_int : in integer;

flag_even : out std_logic);

end Int_Bin;

architecture convert of Int_Bin is

procedure itb (bin : out std_logic_vector;

signal flag : out std_logic;

N : in integer; int : inout integer) is

begin

if (int MOD 2 = 0) then

flag <= ‘1’;

else

flag <= ‘0’;

end if;

for i in 0 to N loop

if (int MOD 2 = 1) then

bin (i) := ‘1’;

else

bin (i) := ‘0’;

end if;

int := int / 2;

end loop;

end itb;

begin

process (Y_int)

variable tem : std_logic_vector (N downto 0);

variable tem_int : integer;

begin

tem_int := Y_int;

itb (tem, flag_even, N, tem_int);

X_bin <= tem;

end process;

end convert;

LISTING 10.35 Verilog Example of a Function

module Func_synth (a1, b1, d1);

input a1, b1;

output d1;

reg d1;

always @ (a1, b1)

begin

d1 = andopr (a1, b1);

end

function andopr;

input a, b;

begin

andopr = a ^ b;

end

endfunction

endmodule

LISTING 10.36 Example of Function Synthesis

module Function_Synth2 (x, y);

input [2:0] x;

output [3:0] y;

reg [3:0] y;

always @ (x)

begin

y = fn (x);

end

function [3:0] fn;

input [2:0] a;

begin

if (a <= 4)

fn = 2 * a + 5;

end

endfunction

endmodule

LISTING 10.37 (same as Listing 8.15) Verilog Code 2x1 Multiplexer with

Active-Low Enable Using Combinational User-Defined Primitive

module Mux2x1Prmtv(A, B, SEL, Gbar,Y);

input A,B,SEL,Gbar;

output Y;

multiplexer MUX1 (Y, Gbar, SEL,A,B) ;

endmodule

primitive multiplexer (mux, enable, control, dataA, dataB) ;

output mux;

input enable, control, dataA, dataB;

table

// enable control dataA dataB mux

1 ? ? ? : 0;

0 0 1 ? : 1;

0 0 0 ? : 0;

0 1 ? 1 : 1;

0 1 ? 0 : 0;

0 x 0 0 : 0;

0 x 1 1 : 1;

endtable

endprimitive